2. Basic Electricity

- How many numbers of electrons in a copper atom?

  - 29

  - 39

  - 49

  - 59

- How many number of electrons will move in one second for one ampere current through a cross section of the conductor?

- 6.24 x 1012

- 6.24 x 1016

- 5.24 x 1014

- 6.24 x1018

- One joule per second is …………..

- one volt

- one ampere

- one watt

- one ohm

- Practical SI unit of heat is expressed in ……….

- Calorie

- Centrigrade

- Joule

- Celsius

- What is the unit of power?

- kilo volt

- kilo hour

- kilo watt

- kilo watt hour

- What is the value of resistivity of copper conductor?

- 1.72 micro ohm metre

- 1.73 ohm/cm3

- 1.72 micro ohm/m3

- 1.72 micro ohm/cm3

- What is the unit of specific resistance?

- ohm/cm2

- ohm/metre

- micro ohm/cm3

- milli ohm/m2

- Which electrical device is the example for medium resistance?

- Shunt resistance

- Heater resistance

- Contact resistance

- Armature - winding resistance

- What is the commercial unit of electrical energy?

- mega watt hour

- kilowatt hour

- Watt hour

- Watt seconds

- The increase in resistance per ohm per degree centigrade rise in temperature is…….

- positive temperature coefficient

- difference in temperature

- linear Coefficient

- negative temperature coefficient

- Which material is having negative temperature coefficient?

- Tungsten

- Eureka

- Carbon

- Nichrome

- Which is the example for negative temperature coefficient substance?

- Copper

- Nichrome

- Aluminium

- Mica

- The change in resistance per ohm per degree centrigrade is called as .......

- specific resistance

- laws of resistance

- mechanical equivalent of heat

- temperature - coefficient of resistance

- As per Ohm’s law statement inthe circuit, current is directly proportional-to ......,

- voltage

- resistance

- temperature

- specific resistance

- Which law states that the relation between the current, voltage, and resistance in a closed circuit is at a constant temperature?

- Kirchhoff’s current law

- Ohm’s law

- Kirchhoff’s Voltage law

- Laws of resistance

- Which law states that, in each closed circuit, the sum of all voltage drops are

equal to lam?

- Ohms law

- Kirchhoff’s first law

- Kirchhoff’s second law

- Coulomb’s law

- In any electric network, the sum of the incoming currents at each junction is

equal to the sum of the outgoing currents.

What is the name of the law?

- Ohm’s law

- Laws of resistance

- Kirchhoff’s second law

- Kirchhoff’s first law

- Which law states that the sum of the currents meeting at junction is equal to zero?

- Ohm’s law,

- Kirchhoff’s 1st law

- Kirchhoff’s 2nd law

- Coulomb’s law

- When the current: is passed through the conductor, it gets heated. That effect is called as......

- magnetic effect

- thermal effect

- chemical effect.

- electrostatic effect

- As per IE rule, caution notice shall be displayed on electrical equipments/lines depending on the level of voltage. What type of caution notice shall be displayed on equipment/ line operating between 650 V to 33W?

- CAUTlON -  Medium, voltage

- CAUTlON - High voltage

- CAUTlON - Extra voltage

- CAUTlON - Very high voltage

- The conversion bf energy, which takes place in a thermal power station is from……..

- heat energy to electrical energy.

- light energy into electrical energy.

- hydro energy into electrical energy

- solar energy into electrical energy

- What is the formula to be used to find the power, if the current and voltage values are known?

- V × I

- V2 I

- V2 × I

- V × I2

- The work or energy in an electrical circuit is calculated by .......

W = V.C joules

W = I. R. C joules

W = V.I E ioules

W = I2. R. T joules

- An appliance has a resistance of 100 ohms, takes 2 amperes. What is the power consumed by the appliance? ‘

- 400 watts

- 300 watts

- 450 watts

- 350 watts

- A 1000 watts heater is connected to a 240 V, AC supply. Then the current drawn by the heater is .......

- 4.006 amperes

- 4.106 amperes

- 4.160 amperes

- 4.166 amperes

- The given Wheatstone bridge is balanced when P=100Ὡ, Q=1kὩ and S=120Ὡ. Calculate the value of the unknown resistance.

- kὩ

- kὩ

- kὩ

- kὩ

- What is the power input to an electric heater, that draws 4 amperes at 250 volts supply source?

- 360 watts

- 480 watts

- 720 watts

- 960 watts

- A 10 volts battery is Connected to a 5 ohms resistor, then the current flows through resistor is.......

- 50 amperes

- 25 amperes

- 4 amperes

- 2 amperes

- From the given figure of dual supply, determine “the current through the resistance.

- 0.5 A

- 1.0 A

- 2.0 A

- 5.0 A

- The voltage ‘V’ is determined through ‘R’ and ‘l’ with reference from the circuit shown is .......

- V = I x R

- V = I/R

- V = I2R

- V = R/I

- A conductor of length 1000 cm having a diameter 0.7 cm is connected across 20

volts supply in a circuit and 2 amperes

current flowing in the circuit. Calculate the

specific resistance of the conductor.

- 0.3849 ohm/cm3

- 0.03849 ohm/cm3

- 0 003849 ohm/cm3

- 00003849 ohm/cm3

- From the given figure calculate the resistance measured between x and y.

- 2Ὡ

- 3Ὡ

- 8Ὡ

- 38Ὡ

- Calculate the hot resistance value of the lamp 2.00 watts/250 Volts?

- 6250 ohms

- 625 ohms

- 3125 ohms

- 312.5 ohms

- Two lamps rated 200 W/240 V are connected in series and combined set is connected across 240 V, AC supply mains. The total power consumed in the circuited is ........

- 400 W

- 300 W

- 200 W

- 100 W

- What is the current in ‘R1’=, resistor shown in the circuit?

- 100 mA

- 200 mA

- 250 mA

- 300 mA

- What will happen to the indication of the voltmeter when the switch is closed?

- Increases

- Decreases

- Zero

- No change

- What is the value of medium resistance based on its ohmic value?

- One ohm to 1000 ohms

- Above one ohm up to 10 kilo ohms

- Above one ohm up to 100 kilo ohms

- Above 100 kilo ohms

- When no current flows through the galvanometer in Wheatstone bridge in the figure, the bridge circuit is said to be.......

- Misaligned

- Balanced

- Unbalanced

- Opened

- Two incandescent lamps of 125 V, 40 W and 125 V, 100 W are connected in series to a supply voltage of 230' volts. What happens due to this connection?

- The drop across the 100 W lamp will be higher  than its rated voltage, therefore it will fuse.

- The lamps will be fused immediately because they are rated for 125 V.

- The drop across the 40 W lamp will be higher than its rated voltage, therefore it will fuse.

- The 100 W lamp draws higher current, therefore the 40 W lamp will be fused

- In which materials, the temperature is Increased then the resistance will decrease?

- Tungsten

- Carbon

- Nichrome

- Manganese

- A 12 ohm resistor connected across a 60 V, DC source. Calculate the current flows through the resistor.

- 4 amperes

- 5 amperes

- 10 amperes

- 12 amperes

- Calculate the equivalent, resistance of the circuit shown in-figure?

- 0.8 Ὡ

- 1.0 Ὡ

- 1.25 Ὡ

- 1.50 Ὡ

- What is the commercial unit of electrical energy?

- watt hour

- kilo volt ampere

- kilo watt hour

- volt ampere

- The resistance offered between the two opposite faces of a metre cube" or centimetre cube of the material is called as........

- Internal resistance

- Specific resistance

- Laws of resistance

- Equivalent resistance

- The specific resistance of a substance depends on........

- its length

- its cross sectional area

- its kind of material

- all the above stated factors

- The amount of current flowing through a resistance depends on

- applied voltage only

- ohmic value of the resistor only

- applied voltage and ohmic value of the resistor

- none of the above

- For a Constant applied voltage the current (I) ﬂowing through a resistor is –

- directly proportional to R

- inversely proportional to R

- equal to R

- constant irrespective of change in R

- According “to Ohm’s Law, the relation between resistance (R), current (I) and p.d.(V) is stated as –

- I  = V/R

- R = V.I

- R = I/V

- V = I/R

- What will be the voltage drop across a resistor of 60 ohms if the current through it is 0 1 ampere

- 60 V

- 6 V

- 1 V

- 600 V

- What will be the total resistance of the three resistances R1, R2 and R3 which are connected as shown in the given figure

- 20 kὩ

- 30 kὩ

- 60 kὩ

- 180 kὩ

- if three resistances of 100 Ὡ , 200 Ὡ and     300 Ὡ are connected in parallel and the applied voltage is When

- the current flowing through the resistors is the same, whereas, voltages developed across them will be different.

- the current ﬂowing through the resistors and the voltages developed across them will be the same

- the voltages across the resistors will be the same, whereas, currents flowing through them will be different

- the current flowing " through the resistors and the voltages developed across them will be different

- What will be the amount of current (I) flowing through the given circuit –

- 6 A

- 3 A

- 1 A

- 0 A

- if in the given circuit the value of R is doubled then for a given voltage V ......

- l3 would be halved.

- I2 and I3 would be halved.

- I2 would be unaffected.

- All currents would be affected.

- The cross - sectional area of a conductor is 20 sq.m and its resistance is 4 ohms. If its cross sectional area is doubled then its new resistance will be :

  - 8 ohms

  - 5 ohms

  - 4 ohms

  - 2 ohms

- it was experimentally found by James Precott Joule that the: heat produced in a current carrying conductor is directly proportional to

- the square of current

- the square of resistance

- the current

- the square of the applied voltage.

- If a 200 watts lamp is connected across a 250 volt source then what will be the circuit current?

- 4 A

- 2 A

- 0.8 A

- 1 A

- The monthly power consumption of 10 lamps of 100 watts each day will be: used 8 hours a day will be

- 240 units

- 245 units

- 225 units

- 250 units

- The power consumption of 100 watts lamp used for 10 hours daily will be :

- 2 units

- 1 units

- 0.5 units

- c5 units

- In a closed loop of a d.c circuit, the algebraic sum of the e.m.f. (∑E) is equal to…….

- Zero

- algebraic sum of the products of resistances and current flowing through them

- sum of the voltage drops across the resistors

- none of the above

- What is the value of current I5 in the circuit shown below?

- 1A

- 7A

- 4A

- 3A

- What is the magnitude of current in branch OX of the given figure?

- 15 A

- 6.5 A

- 10.5 A

- 11.5 A

- What will be the value of the current I4?

- 2 A

- 4 A

- 5 A

- 6 A

- How many electrons are in the outer most orbit of a semi conductor substance?

- 5 only

- 4 only

- 3 only

- 1 only

Resistors

- The fundamental unit of measurements of the resistance is -

- Megaohm

- millohm

- ohm

- kiloohm

- A resistor is used in an electric circuit for -

- blocking the flow of current

- increasing the flow of current

- reducing the flow of current

- none of the above

- For measuring an unknown resistance, choose range initially

- the lowest

- the highest

- any

- R x 10

- For measuring insulation resistance, the best choice would be -

- digital multimeter

- megger

- multimeter

- Wheat Stone Bridge

- Potentiometer is used to provide -

- a fixed - resistance value

- a variable resistance value

- any two resistance values

- a resistance value in steps

- Potentiometers of wire wound type are made in

- rotary type only

- sliding contact type only

- both rotary and sliding contact types

- preset type

- Potentiometers of carbon type are available in

- linear type only

- logarithmic type only 14

- stepping type only

- both linear And logarithmic types

- A preset is a sort-of -

- variable-resistor

- adjustable resistor

- fixed value resistor

- stepped value resistor

- Carbon resistors are made In –

- composition type only

- film type only

- both composition and film types

- none-of the above stated types

- The resistance value of a carbon resistor having Orange, Red, Brown and Golden colour bands is –

- 32 kὩ ± 5%

- 3.2 kὩ ± 5%

- 320 Ὡ ± 5%

- 32Ὡ ± 5%

- The colour bands for 1 5 ohms resistor will be

- Brown, Green, Brown

- Brown, Green, Golden

- Brown, Golden, Green

- Brown, Golden, Golden

- What is the value of the resistor which is having the colour bands in the order of red, green, orange and gold respectively?

- 24000 ohm ± 10%

- 25000 ohm ± 5%

- 26000 ohm ± 10% ‘

- 27000 ohm ±  5%

- What is the value of resistance given in the figure by colour coding?

- 3.9 Ὡ

- 39 Ὡ

- 390 Ὡ

- 3900 Ὡ

- What will be the tolerance of the resistor shown in the figure, it the fourth band of the resistor is coated with brown colour?

- ±20%

- ±10%

- ±5%

- ±1%

- As per letter and digit code values, how the value of 1500000 ohm (Ὡ) resistance is to be represented?

- 1500 k Ὡ

- 150 x 104 Ὡ

- 1 M5

- 1.5 MὩ

- The resistance value of 1800 ohms resistor in designated code as …………

- 1.8 K

- 1 K 8

- 1 M 8

- 18 K

- What is the actual value of resistance of 100 Ὡ ± 10%, 1 W resistor?

- Between 110 Ὡ to 120 Ὡ

- Between 100 Ὡ to 110 Ὡ

- Between 90 Ὡ to 110 Ὡ

- Between 80 Ὡ to 100 Ὡ

- In an open circuit ………

- resistance and current both are zero

- resistance and current both are infinity

- resistance is zero and current is infinity

- resistance is infinity and current is zero

- The effect of open circuit, in series circuit is:

- no current flows in the circuit

- no device in the circuit functions

- total supply voltage/source voltage appears across the open

- all of the above

- To compensate the lower value of cold resistance of the filament which one of the following is connected in series with the serial set?

- NTC thermistor

- PTC thermistor

- Voltage dependent resistor

- Flasher

- Which type of resistor is used for are quenching purpose?

- PTC resistors

- LDR (Light Dependent Resistor)

- VDR (Voltage Dependent Resistor)

- NTC Resistors

- Which type of resistor, is used for measuring the intensity of, light?

- PTC resistor

- NTC resistor

- VD resistor

- LD resistor

- Which types of resistors are called as thermistors?

- PTC resistors

- Varistors (VDR)

- Light dependent resistor (LDR)

- NTC resistors

- Which type of resistor is depending upon the voltage?

- Sensistors

- Varistors

- LDR

- Thermistor

5. Conductor, Insulator and Insulating Materials

- A solid substance or insulated conductor subjected tensile stress with or without screen is called as........

- cable

- core

- conductor

- wire

- A conductor should have.........

- low specific resistance

- medium specific resistance

- high specific resistance

- any-specific resistance

- Which one of the following conductors is used in bulbs?

- Tungsten

- Carbon

- Copper

- Nichrome

- Which one of the following Conductors is used in heating elements?

- Tungsten

- Carbon

- Copper

- Nichrome

- Which is the metal most commonly used as conductor material?

- Steel wire

- Copper wire

- Aluminium wire

- G. l. wire

- When compared to copper, the aluminium

has ....... of conductivity.

- 60%

- 60.2%

- 60.4%

- 60.6%

- Copper used in electric work should have a very high degree of purity, say .........

- 90.9%

- 95.9%

- 96.9%

- 99.9%

- Which one is NOT an electrical conducting material?

- Copper

- Aluminium

- Silver

- Glass

- Which one of the following is NOT a conductor?

- Silver

- Copper

- Aluminium

- PVC

- The material that contains free electron is ……

- Silver

- PVC

- Mica

- Rubber

- Which among the following materials is used for making split rings?

- Wrought iron

- Copper

- Cast iron

- Aluminium

- Field coils are wound by using.......

- Wrought iron wire

- Steel wire

- Bare copper wire

- Enameiled copper wire

- Which material is NOT a good Conductor?

- Aluminium

- Copper

- Porcelain

- Steel

- Which of the following is a good conductor?

- China clay

- Porcelain

- Graphite

- Glass

- The standard wire gauge (SWG) can measure sizes from .....

- 0 to 32

- 0 to 34

- 0 to 35

- 0 to  36

- The SWG No. 36 is equal to …….

- 0.0084 inch or 0.21 mm dia

- 0.0092 inch or 0.23 mm dia

- 0.0100 inch or 0.25 mm dia

- 0.0076 inch or 0.19 mm dia

- While inserting a bare conductor in the notch of the standard wire gauge for measuring the size of the conductor, it ………….

- should be tight in the notch

- should be loose in the notch

- should be neither loose or tight

- may be either loose or tight

- Four illustrations are shown in the figure for a crimped termination of a cable to an eyelet lug. Which of the representation is the correct termination?

- What is the voltage grade of heavy duty PVC sheathed power cable?

- 440 V / 11000 V

- 650  V / 1100 V

- 1100V / 11000V

- 11000 V / 132000 V

- What is the expansion of PlLC single core cable?

- Paper insulated lead covered

- Paper insulated lead core

- Polythene insulated  lead core

- Polyvinyl insulated lead core

- What is the advantage of stranded conductors over solid conductors?

- more flexible

- more area of cross section

- more resistance

- more weight

- What is the use of Eureka wire?

  - Rheostat

  - Heating element

  - Earth wire

  - Winding wire

- The good conductor of heat and electricity -

  - stainless steel wire

  - iron wire

  - copper wire

  - brass wire

- Size of the wires can be measured normally by standard wire gauge (SWG). Which instrument must be used for accurate measurements of wire size?

- Vernier caliper

- Depth gauge

- Fuel gauge

- Micrometer

- Normally metals are good conductor of electricity. Which is the good conductor, in terms of conductivity?

- Copper

- Aluminium

- Silver

- Brass

- We find, for a required current carrying capacity, aluminium conductor is larger in cross sectional area than copper The reason for this is ……….

  - aluminium gets heated up more than copper

  - resistivity of aluminium is greater than copper

  - aluminium is mechanically weaker than copper

  - Aluminium is cheaper than copper

- 7/1.40 is the specification of the one stranded conductor. What does the denominator indicate in the above specification?

- Diameter of the conductor in mm

- Radius of the conductor in mm

- Area of the conductor in mm2

- Diameter of the conductor in inch

- The material that have only few electrons and are incapable of flowing current is

- Porcelain

- Silver

- Gold

- Copper

- Which one the following material is the insulating material?

  - Water

  - Carbon

  - Mica

  - Copper

- Which of the following material is NOT a good insulator?

- Porcelain

- Plastic

- Rubber

- Copper

- Which of the following is NOT an insulator?

- Zinc

- Glass

- Rubber

- Plastic

- Which is not a property or an insulator?

- High heat conductivity

- High permittivity

- High dielectric Strength

- High mechanical strength

- What is the insulating material used for heater base?

- Asbestos

- Mica

- Porcelain

- Bakelite

- Insulating materials are classified according to their withstanding temperature. Which class of material can withstand temperature up to 225°C?

- “Y” class

- “A” class

- “B” class

- “C” class

- Which insulating material is having high dielectric strength?

- Porcelain

- Fibre

- Mica

- Glass

- Which material is NOT an insulating material?

- Wood

- Carbon

- Asbestos

- Bakelite

- What is the fundamental property of insulating material?

- Low breakdown voltage

- High dielectric strength

- Less flexibility

- Hygroscopic

- The property of an insulating material is the measure of how much potential difference, the insulation layer can withstand without breakdown is called as

- mechanical strength

- dielectric strength

- insulation resistance

- flexibility

- The" insulating ‘material used in connecting leads of the electric heater is

- Bakelite material

- Porcelain material

- Rubber material

- Asbestos material

- The potential difference that causes the breakdown of insulation is called the ......

- breakdown voltage

- dielectric strength

- insulation resistance

- conductivity

- Which is the high temperature withstanding insulating material?

- Bakelite

- Wood

- Asbestos

- Mica

- Which type of insulation tape is used for  insulating joints?

- Friction tape

- Plastic tape

- Rubber tape

- varnished cambric tape

- Which type of insulating tape is having high dielectric strength?

- Rubber tape

- Friction tape

- PVC tape

- Varnished 'cambric tape

- Where joints to be moved within stipulated distance for low current circuits, the most suitable joint is.......

- aerial joint

- knotted joint

- duplex cross tap joint

- double cross tap joint

- Why soldering is done on joints?

- To improve tensile strength

- To improve resistivity

- To improve ductility

- To improve conductivity

- What purpose is served by flax during soldering?

- It cleans the surface to be soldered

- It lowers the melting point of the solder

- It makes the flow of solder In the joint easy

- It does all the above

- Name the chemical which is used as soldering flux - for the electricians solder?

- Zinc chloride

- Hydrochloric acid

- Sal ammonia rosin

- Rosin

- A substance used to dissolve oxides on the surface of conductor using the soldering process is called as .......

- solder

- soldering bit

- bonding agent

- soldering flux

- The solder wire used in electronic soldering melts at …….

- 200°C

- 400°C

- 600°C

- 800°C

- Which method of Soldering iron is used for soldering mininature components on printed  circuit boards (PCB)?

- Soldering with the soldering gun

- Soldering with torch

- Temperature controlled Soldering iron

- Dip soldering

- Zinc is added by 2% along with lead and tin in solders, which are used for ......

- copper joints

- silver joints

- gold joints

- aluminium joints

- Which one of the following is preferred to use in soldering aluminium cables with a ferrule joint?

- A lead ferrule

- An aluminium ferrule

- A copper ferrule

- A tin ferrule

- For soldering aluminium conductors, Alca – P solder is used. What are the material compositions of Alca - P solder?

- Lead-50%, Tin 30%, Zinc 10% and Cadmium 10%

- Lead 30%, Tin 50% and Zinc 20%

- Lead 51%, Tin 31%, Zinc 9% and Cadmium 9%

- Lead 63%, Tin 35% and Zinc 2%

- The solder used for soldering consists of an alloy of tin-and lead. What is the composition of tin and lead of solder used for electrician solder?

- Tin 50%, and lead 50%

- Tin 60% and lead 40% ‘

- Tin 35% and lead 65%

- Tin 90% and lead 10%

- Which is the application of the tinman solder?

- Heavy duty solder

- Tinning

- Fine soldering

- Electrical joint soldering

- Which type of spider is suitable for sal ammonium resin flux?

- Coarse solder

- Electrician’s solder

- Fine solder

- Tin man’s solder

- Which Soldering method is suitable for soldering printed circuit board?

- Soldering with soldering gun

- Soldering with flame

- Dip soldering

- Machine soldering

- The solders used for soft soldering is .....

  - Tinman’s solder

  - Electrician’s solder

  - Fine solder

  - Plumber’s solder

- A Britannia joint is used

  - H. lines in

  - underground cables

  - conduit wiring

  - power wiring

- The bit of a soldering iron is made of:

- Iron

- brass

- copper

- stainless steel

- The type of soldering iron used to solder the steel plate is of

  - light duty

  - medium duty

  - heavy duty

  - any one

- The correct way of soldering the resistor leads etc is

- heat up the solder and apply the molten solder to the joint

- heat up the joint and apply the solder

- apply heat and spider simultaneously

- apply solder before heating

- A good soldered joint should be:

- Glossy bright and shiny

- Sufficiently large in Size

- Grainy

- Flat

- in an installation four numbers of similar cable joints are soldered with the help of the same pot ladle and solder by the same cable-jointer. It is found that the quality of fourth joint is not good. (The surface is dull and flow is improper). The reason may be

- solder has lost tin heating due to repeated

- solder has lost lead due to repeated heating

- the flux has lost its characteristic due to long exposure with atmosphere

- the joints over heated in the process

Electric Accessories

- The fuse current in amperes is related to fuse Wire’s diameter is:

- 1 ∞ 1D

- 1 ∞ D

- 1 ∞ D1/2

- 1 ∞ D2

- A fuse wire should have:

  - low specific resistance and high melting point .

  - low specific resistance and low melting point

  - high specific resistance and high melting point

  - high-specific resistance and low melting point

- Over-load current protection for a motor is provided by:

  - cartridge fuse

  - kit - kat fuse '

  - over - load current relay

  - all the above

- In a meter circuit, a fuse provides:

- over-load current protection

- short-circuit protection

- open - circuit protection

- none of the above

- A fuse operates due to the .......... effect of the electric current:

- Magnetic

- electrostatic

- heating

- chemical

- The fuse rating is expressed in terms of:

- voltage

- current

- kVA

- VAr

- A Delayed - action cartridge fuse is used for

- motor circuits

- lighting circuits

- ﬂuorescent lamp circuits

- heating circuits

- A fuse should be connected in:

- series of neutral conductor

- parallel of, neutral Conductor

- series of live conductor

- parallel of live conductor

- For safety point of view the fuses should be placed in .........

- neutral cable

- phase cable

- earth wire

- any wire/cable

- A fuse in a circuit provides protection against

- open circuit

- over load

- short circuit and overload

- open circuit and overload.

- Fusing factor is .....

  - Minimum fusing currentRated current

  - Maximum fusing current Rated current

  - Rated currentMinimum fusing current

  - Rated currentMaximum fusing current

- Which one of the following is the primary function of the fuse?

- To protect the operator

- To protect the line

- To prevent excessive current

- To  open the circuit

- A fuse in an electrical circuit “blows” as a result of a high …………

- wattage caused by a short Circuit

- current flow caused by a short circuit

- vintage caused by an Open circuit

- current flow caused by an Open circuit

- 35 S.W.G. copper wire has a fusing rate of ,,,,,

- 6 amperes

- 5.5 amperes

- 5 amperes

- 4 amperes

- As compared to rewirable fuses H.R.S. fuses have ………

- high rupturing capacity

- high Speed of operation

- no ageing effect

- low rupturing effect

- the quick response type catridge fuse is  used for ,,,,,,

- fighting circuits

- heating circuits

- motor circuits

- Wiring circuits

- The protective device which will not open the circuit even after. 4 hours, when the current is 1.5 times the excess of the rated current will be

- H.R.C. fuses

- Catridge fuses

- Circuit breakers

- rewirable fuses

- Which type device, switches are classified?

- Controlling device

- Holding device

- Safety device

- Outlet device

- Different type of switches are being used  in lighting circuits. Which switch is used for stair case wiring?

- One way switch

- Two way switch

- Bed switch

- Push switch

- How many two way switches are required for the stair case wiring?

- 2

- 3

- 4

- 5

- The specification surface-type tumbler switch, one way is ......

- 6A, 250 V

- 6 A, 240 V

- 6 A, 230 V

- 5 A, 250 V

- In which type of accessories, intermediate switch belongs?

- Holding accessories

- Safety accessories

- Controlling accessories

- Outlet accessories

- Though the l.C.D.P. switch of an electrical installation is in “OFF" position, load connected to the switch is functioning normally. This is due to......

- Earth fault between L & E in the switch

- Loose handle fixed on baffle carrier rod of switch

- Short between L & N in the switch

- Battles in the switch are in open condition due to damaged operating rod

- The main switch used in single phase supply is to be provided with .........

- fuse in both line and neutral

- a fuse in the line and a link in the neutral

- a link in the line and a fuse in the neutral

- thicker fuse wire in the phase and thinner fuse wire in the neutral.

- How many number of SPT and two way switches are required for the installation of go down wiring with 6 No. of lamps?

- One SPT and 6 Two way

- Two SPT and 5 Two way

- One SPT and 5 Two way

- Two SPT and 6 Two way

- Switch and accessory boxes need not be earthed if they are made up of......

- metal box

- sheet metal box

- cost iron box

- PVC box

- The electrical accessories used in wiring installation are classified according to their uses. In what classification of accessories, the ceiling-rose will belong?

- Holding Accessories

- General Accessories

- Safety Accessories

- Controlling Accessories

- In what Classification of accessories, the distribution board will belong?

- Safety

- Controlling

- General

- Holding

- As per BIS 1293 - 1988, the voltage rating is …

- 220 V

- 230 V

- 235 V

- 240 V

- The safe maximum current that can pass continuously without overheated is called as

- fusing current

- current rating

- over load current

- safe current

- The socket is rated as ...... and having only two terminals holes without earth connection.

- 6A, 220 V

- 6A, 230 V

- 6A, 240 V

- 6A, 250 V

- Which type of outlet accessory is suitable for double insulated appliances having P.V.C. or insulated body?

- Three pin Socket

- Two pin socket

- Three pin plug

- Two pin plug

- Two pin sockets should not be used in domestic wiring be connected is unless the appliance to

- double earthed

- double insulated

- controlled by E.L.C.B.

- controlled by M.C.B.

- A length of conduct which is having one or more cores insulated from each other is called as……..

- core

- strand

- wire

- cable

- Which tools used to measure the size of the conductor?

- Bevel gauge

- Vernier caliper

- Standard wire gauge

- Depth gauge

- What is the advantage of stranded conductors over solid conductors?

- More flexible

- More area - of cross section

- Mere resistance

- More weight

- The outer screwed contacts of Edison screw type lamp holder must always be connected to the

- neutral wire of the circuit

- phase wire of the circuit

- switch (half wire) of the circuit

- earth wire of the circuit

- Which-type of lamp holders can be used for more than 300 watt lamps only?

- Edison screw type lamp holders

- Goliath Edison screw type holder

- Swivel lamp holder

- Bayonet cap lamp holder

- What type of accessor is shown in the figure?

- Holding Accessories

- Controlling Accessories

- Safety Accessories

- Outlet Accessories

- What is the type of metal conduit outlet box shown in the figure?

- T’ type outlet box

- Angle type Outlet box

- Through type outlet

- Back type outlet box

- Name the type of“ conduit accessory shown in the figure.

- Solid conduit bend

- Inspection conduit bend

- Solid conduit elbow

- inspection conduit elbow

- You have to join two circuit ends for drawing a very long cable in a straight run with a joint. Which type of junction box you need for this purpose?

- Which type of junction box is used to connect three directional circuits, shown in the figure.

- Name the type of conduit accessory shown in the figure?

- “T” Type outlet box

- Back type outlet box

- Through type outlet box

- Angle type outlet box

- Name the type of conduit accessory shown the figure.

- A solid conduct elbow

- Inspection conduit elbow

- Solid conduit “Tee”

- Inspection conduit “Tee”

- Name the part indicated as ‘X’ in the Distribution board shown in the figure?’

- Earth Terminal

- Neutral link

- Enclosure lock nut

- Fixing screw

- What is the specification of the terminal connector-shown in the figure?

- 6 amperes, 6 way, 250 V terminal connector

- 16 amperes, 6 way, 250 V terminal connector

- 16 amperes, 6 way, 500 V terminal connector

- 32 amperes; 6 way, 500 V terminal connector

- A terminal connector is shown in the figure. What is the speciﬁcation of this

- 6 amperes, 12 ways, 250 V

- 16 amperes, 12 ways, 250 V

- 30 amperes, 12 ways, 500 V

- 32 amperes, 12 ways, 500 V

- Name the electrical-accessory shown in the figure?

- Terminal connector

- Fuse cut out

- Earth Terminal

- Neutral link

- To reduce the length of cable in a wiring installation having same number of points and Wired in loop in terminal system, one of the following wiring accessories is required. Identify the accessory.

- Two way switch

- Intermediate switch

- Two plate ceiling rose

- Three plate ceiling rose

Circuit Breakers

- A circuit breaker normally operates........

- when the power is to be supplied

- when the line is to be tested

- when the switch is to be put on

- whenever fault occurs in the line

- The fluids used in circuit breakers should be of.......

- high dielectric strength and thermal stability

- non inflammability

- are extinguishing ability

- all of the above

- In a circuit breaker, the current that exists at the instant of contact separation is called the ...... current

- restriking

- breaking

- arc

- recovery

- In circuit breakers, oil as quenching  medium has the advantage of ......

- goo Cooling” properties and absorption of arc energy

- high dielectric strength

- acting as an insulator

- all of the above

- Which of the following circuit breakers Is generally used in Railway electrification?

- Air blast circuit breaker

- Minimum oil circuit breaker

- Bulk oil circuit breaker

- SF6 circuit breaker

- Main purpose of oil in OCB is to ......

- provide insulation

- provide cooling of contacts

- quenching arc

- none of the above

- in a minimum oil Circuit breaker, the oils used

- to act as circuit breaking medium only

- for circuit breaking and providing insulation

- for providing insulation only

- for none of the above purposes

- What is the preferred type of “circuit breaker” to be installed in extra high voltage A.C.L System?

- Bulk oil type circuit breaker

- Air- blast circuit breaker

- Vacuum circuit breaker

- Sulphur hexafluoride(SF6) circuit breaker

- Which of the following circuit breakers has high reliability and negligible maintenance?

- Air blast circuit breaker

- SF6 circuit breaker

- Oil circuit breaker

- Vacuum circuit breaker

- Which of the following circuit breakers take minimum time in installation?

- Air blast

- Minimum oil

- Bulk oil

- Sulphur hexafluoride (SF6)

- breaker is of filling oil in a circuit

- to produce insulation effect

- to stop the ionization of the surrounding air

- to ensure the operation of the device at a low temperature

- to suppress the sparking produced across the connector at the time of breaking a circuit.

- In circuit breakers the device used for tripping the line in the event of short-circuit is of ...... type.

- magnetic

- thermal

- magnetic and thermal

- low voltage

- The circuit breaker which does not contain any serviceable part is known as:

- oil circuit breaker (OCB)

- miniature circuit breaker (MCB)

- air circuit breaker (ACB)

- air blast circuit breaker (ABCB)

- circuit breaker by ...... provided In the

- Magnetic and thermal release

- Thermal release

- Magnetic release

- Low voltage release

- ln an installation, leakage current of smaller magnitudes causes……..

- frequent tripping of ELCB

- electric shock

- fire hazard

- excessive billing of energy

- Miniature circuit breaker (MCB) are used in domestic and commercial installations for control and protection. What is the range-of current rating of single pole MCBs, commercially available?

- 2.0 to 6 amps.

- 0.5 to 60 amps,

- 1.0 to 62 amps.

- 6.0 to 32 amps

- Miniature circuit breakers (M.C.B.) are used in domestic and commercial installations for control and protection. The current rating of double pole and triple pole MCB is………

- 2.0 to 6 amperes

- 0.5 to 60 amperes

- 5.0 to 60 amperes

- 6.0 to 100 amperes

- Current rating of M08 selected for an air  conditioner of 1 ton is

- 10/15 amperes

- 15/20 amperes

- 10/20 amperes

- 15/30 amperes

- M.C.Bs are normally rated...... ambient temperature.

- 25°C

- 35°C

- 45°C

- 40°C

- Which one of the following is not applicable to MCB?

- Tamper proof as they are sealed

- Used for close excess current protection

- Used for control as well as to protect the circuit

- Used for coarse excess current protection.

- Which device can be Used as switch and as well as protective device?

- Triple pole iron clad switch

- Intermediate switch

- Miniature circuit breaker

- Earth leakage circuit breaker

- ELCB stands for......

- Electrical Leakage Circuit Breaker

- Earth Leakage Current Breaker

- Earth Leakage Circuit Breaker

- Electrical Line Circuit Breaker

- The ELCB expansion form is......

- Earth Leakage Circuit Breaker

- Earth Leakage Control Breaker

- Earth Leakage Current Breaker

- Earth Leakage Component Breaker

- Earth Leakage Circuit Breaker (ELCB) works on the principle of.......

- over load current

- short circuit current

- residual Current

- neutral current

- An ELCB must always be connected in the circuit......

- before energy meter

- after energy meter

- at the entry of Supply to load after energy meter

- tail end of the installation

- ELCBs are specially used to disconnect the supply under .........

- short circuit condition

- ground fault condition

- open circuit condition

- over load condition

- The expansion form of RCCB is.........

- Residual Current Circuit Breaker

- Residual Circuit Current Breaker

- Residual Current Control Breaker

- Residual Control Circuit Breaker

- MCCB’s expansion is.........

- Moulded Case Circuit, Breaker

- Main Case Circuit Breaker

- Main Control Circuit Breaker

- Main Current Circuit Breaker

- The figure shown is the internal circuit details of a ......

- ELCB voltage operated

- Over current relay

- ELCB current operated

- Under voltage trip device

- The arrangement of residual current device is shown in the figure. Name the part as ‘X’ is……

- Secondary winding

- Primary winding

- Torroid magnetic coil

- Relay coil

- Name the part indicates as ‘X’ in the ELCB circuit shown in figure?

- Operating coil

- Secondary coil

- Primary coil

- Test winding

Earthing

- The size of the earth or ground wire is based on the:

- maximum fault current through the ground wire

- rated current carrying capacity of the service line

- depends on the soil resistance

- both (a) and (b)

- Earth wire or ground wire is made of:

- Copper

- aluminium

- iron

- galvanized steel

- Average resistance of human; body is:

- 500 ohms

- 1000 ohms

- 1500 ohms

- 2000 ohms

- Generally earthing is provided for:

- only for the safety of the equipment carrying

- only for the safety of the personnel operating

- both (a) and (b)

- none of the above

- Ground resistance should be designed Stich that:

  - Grounding resistance sh0uld be as low as possible

  - Grounding resistance should be as high as possible

  - Grounding always Zero

  - None of the above

- For an EHV equipment for maintenance first it should be, isolated and connected to ground because:

- to provide low impedance

- to discharge the capacitance to ground

- protection for operating personnel charging

- both (b) and (c)

- When earth fault occurs:

- voltage potential at the earth mat increases due to earthing

- voltage potential at the earth mat decreases tide to earth

- voltage potential at the earth mat remains zero irrespective of fault

- none at the above

- The objective of earthing or grounding Is:

  - to provide as low resistance possible to the ground

  - to provide as high resistance to the ground possible

  - to provide flow of positive, negative and zero sequence currents

  - none of the above

- Moisture content in the soil ..........A the earth soil resistance:

- Increase

- decrease

- does not affect

- none of the above

- Factors on which soil resistance depends:

- depth of the electrode '

- moisture

- NaCl

- all the above

- What is the main purpose for providing earthing to the electrical equipments?

- To protect the equipment from excess  current,

- To maintain the line current constant

- To maintain the supply voltage constant

- To increase the life of the equipments

- According to IE. rules, the supplier shall provide and maintain on the consumer’s premises, a suitable ......

- earth terminal.

- phase terminal

- neutral terminal

- live terminal

- According to I.E. rules, the supplier shall provide and maintain on the consumer’s premises, a suitable.

- phase terminal

- neutral terminal

- earth terminal

- live terminal

- Why does the earth conductor provide a path to ground?

- For over current

- For high voltage

- For leakage current

- For low voltage

- Which colour wire is recommended for earth connection as per lSl code?

- Red

- Green

- Black

- Blue

- Which colour wire is recommended for earth connection as per lSI code?

- Red

- Black

- Blue

- Green

- An earthing connection to a GI pipe is necessary.

- To prevent a damage to the insulation of wires

- To prevent a damage to the wires

- For sate current flow

- To pass the earth leakage current to the

- The resistance of the earth is:

- 1MὩ

- 20 MὩ

- Infinite

- about zero

- In an earth-point, charcoal and salt are used:

- To protect the plate against rust

- To absorb the moisture of the surrounding soil

- To increase the moisture of the surrounding soil

- To hold the plate firmly in the soil.

- Earth continuity conductors should run from the machine to the panel boards and to the main no or earth electrodes are .......

- 1

- 2

- 3

- 4

- The equipment earthing protects from the

- leakage current

- break down current

- lightning strokes

- operational surge voltages

- The purpose of providing system earthing is to protect the equipment from

- insulation weakness

- lightning strokes

- voltage fluctuation

- frequency vibration

- Which one of the following is the minimum size of a, copper earth continuity conductor for a domestic installation having 5 amperes capacity?

- 1.5 mm2

- 2.5 mm2

- 5 mm2

- 25 mm2

- To make extension in earth Conductor, the most suitable joint is.......

- Brittnia joint

- Married joint

- Western joint

- Scarfed joint

- As a rule, the minimum distance between two electrodes shall not be less than ......

- One time the length of electrode

- Two times the length of electrode

- Three times the length of electrode

- Four times the, length of electrode

- Earth tester is used for measuring earth electrode resistance in which generated DC is converted into AC by current reverser. The reason for using AC supply for earth resistance measurement is…….

- AC resistance is more than DC resistance

- To reduce error due to electrolytic emf

- AC wave form is pulsating in nature

- AC contains sine wave form

- A metal plate, pipe (or) other conductor (or) an array of conductors electrically connected to the general mates of the earth is called……

- Earth wire

- Earth electrode

- Earth continuity conductor

- Earth terminal

- Which method can be suggested to bring down the earth electrode resistance to an acceptable value?

- By reducing the size of the earth electrodes

- By increasing the size of the earth electrodes

- By pouring water in the earth pit at repeated intervals

- By connecting a number of earth

- The function of current reverser in earth tester is to..…..

- change A.C into D C. supply

- bring the needle to stand Still position without vibration

- change D.C. into A.C. supply

- provide controlling torque

- You have taken four readings from an earth resistance tester, placing the spikes at different position around the electrode under test, keeping the’ distance between the electrodes same as in the first test. If the resistance values are 3.0Ὡ, 2.4Ὡ, 2.3Ὡ, 1.9Ὡ value should you consider?

- Sum of alt values

- Maximum of the four values

- Minimum of the four values

- Average of the four values

- The minimum dimension plate as per ISl code is of copper earth.....

- 60 cm x 30 cm x 6.38 m

- 60 cm x 60 cm x 6.38 mm

- 60 cm x 30 cm x 3.18 mm

- 30 cm x 30 cm x 318 mm

- As per lSl, for pipe earthing, the minimum size of internal diameter of galvanized iron pipe is………..

- 19 mm

- 25 mm

- 32 mm

- 38 mm

- In pipe earthing the minimum internal diameter of galvanized iron or steel pipe required is…….

- 38 mm

- 32 mm

- 25 mm

- 12.5 mm

- As per ISI for pipe earthing, the minimum internal diameter of galvanized iron or steel pipe is……….

- 12.5 mm

- 18 mm

- 20 mm

- 38 mm.

- The minimum size of, the earth continuity conductor of aluminium is not less than……..

- 1.0 mm2

- 1.5 mm2

- 2.5 mm2

- 4.0 mm2

- What size of the copper earth continuity conductor is recommended to the 2.5 mm2 copper conductor in a circuit?

- 1.5 mm2

- 2.5 mm2

- 3 mm2

- 4 mm2

- Rod electrodes of copper used for earthing should be at least 12.5 mm in diameter and those of steel or galvanized iron shall be at least in diameter......

- 18 mm

- 16 mm

- 12.5 mm

- 10 mm

- For plate earthing the thickness of the G.I. or steel plate should not be less than……

- 3.30 mm

- 4.30 mm

- 5.30 mm

- 6.30 mm

- As per I.E.E regulations green and yellow (combination) colour cores of flexible cables are recommended for......

- Phase

- Neutral

- Positive of D. C 3 wire

- Protective earth

- At equal Intervals of time the earth resistance to be measured periodically. Which Instrument is used for this?

- Megger

- Ohm meter

- Wheat stone bridge

- Earth tester

- For pipe earthing the minimum internal diameter of Galvanized Iron (GI) pipe

- 25 mm

- 30 mm

- 32 mm

- 38 mm

- How many times the faulty current will be more than the fuse-rating current?

- At Ieast 2.0 times

- At |east 3.0 times

- At least 3 5 times

- At least 4.0 times

Electrolysis, Cells and Batteries

- Electra-chemical equivalent is:

- ratio of atomic weight of an element to atomic weight of hydrogen

- mass of the element liberated per unit quantity of hydrogen

- ratio of atomic weight to valency

- none of the above

- The mass of material deposited over an electrode is…………

- proportional to voltage

- proportional to time only

- proportional to current only

- proportional to quantity of electricity

- In electroplating the positive electrode is called

  - cathode

  - terminal

  - anode

  - ion trap

- In an electrolyte can cause an internal short circuit condition called

  - depolarization

  - electrolysis

  - local action

  - polarization

- Distilled water is used in electrolytes because it:

- prevents or slows down local action

- speeds up electrochemical action

- Improves specific gravity

- prevents polarization

- The condition of electrolyte in a battery is measured in terms of

- current value

- specific gravity

- acid contents

- voltage output

- In a primary cell, the local action can be rectified by

- charging he cell

- amalgamating the zinc electrode with mercury

- using a cell for few minutes only

- making a dry cell

- The function of a dry cell is to convert:

- chemical energy to mechanical energy

- chemical energy to electrical energy

- electrical energy into mechanical energy

- electrical energy into magnetic energy

- Polarization in a primary ceil can be removed by:

- coating the electrodes of the cell

- chemical action

- discharging the cell

- disposing the cell

- The two main defects of a primary cell are:

- local-action and polarization

- local-action and sulphation

- buckling and polarization

- sulphation and sedimentation

- An average dry cell gives an approximate e.m.f. of:

- 1.2 V

- 1.3 V

- 1.4 V

- 1.5 V

- Lechlanche cell is most suited for:

- continuous use

- large current requirements

- Intermittent use

- large voltage requirements

- Cells are connected, in parallel to:

- Increase the internal resistance

- Increase the output e.m.f.

- Increase the current capacity

- Decrease the current capacity

- Cells are connected in series to :

- decrease the output emf,

- increase the output e.m.f.

- decrease the internal resistance

- increase the current capacity

- The function of depolariser in Lechlanche cell is that:

- it converts the, produced hydrogen into water

- it prevents the fast chemical action on the zinc electrode

- it synthesizes the decomposed electrolyte

- It absorbs the oxygen produced in the cell

- Which one of the following acts as depolariser in a dry cell?

- Manganese dioxide

- Zinc chloride?

- Ammonium chloride

- Carbon powder

- One main advantage of a secondary cell is this that it:

- can be used in portable equipments

- can be recharged

- is compact and hence easy to transport

- cannot be recharged

- Nickel-cadmium dry cell is becoming popular in power supplies for electronic calculators because:

- it has a standard shape

- it is dry

- it is rechargeable

- it is easily manufactured

- Secondary cell can produce large amount of power for a :

- short time and can be recharged

- long time and can be recharged

- short time and cannot be recharged

- longtime and Cannot be recharged

- Gassing occurs in the process of.

- charging an accumulator

- charging a dry cell

- discharging an accumulator

- discharging a dry cell

- The charge condition of a secondary cell can be checked by :

- its terminal voltage

- the colour of its electrolyte

- the level of its electrolyte

- its terminal voltage and specific gravity of its electrolyte

- AL lead-acid accumulator should be recharged when the specific gravity of its electrolyte is about :

- 1.18

- 1.15

- 1.25

- 1.35

- Other types of accumulators besides the lead-acid type are :

- alkaline batteries only

- alkaline and solar batteries

- alkaline and dry batteries

- nickel - cadmium and nickel iron batteries

- Electrolyte of a lead-acid battery is formed-by adding :

- water to hydrochloric acid

- sulphuric acid to water

- hydrochloric acid to Water

- water to sulphuric acid

- The ampere how capacity of a battery depends on:

- the thickness of plates

- the distance between the plates

- the number and area of plates

- the strength of electrolyte

- In order to keep the terminals of a lead acid battery free from corrosion, it is advisable to :

- clean the terminals frequently

- change the battery at frequent intervals

- clean the terminals and to apply petroleum jelly.

- wash the terminals with water frequently

- The internal resistance of a discharged battery:

- is high

- is low

- remains the same

- is negative

- It is not desirable to leave a lead acid battery in discharged state for a long time, because :

  - the electrolyte will attack the container.

  - the electrolyte will become weak

  - the plates will get sulphated strongly

  - the electrolyte will evaporate

- The separators in a lead acid battery are designed to prevent the plates from :

  - touching the electrolyte

  - shorting together

  - shorting with the sediments

  - touching the container

- The advantage of nickel-iron cell over lead acid cell is this that :

  - it has a higher efficiency

  - it needs less maintenance

  - it is much cheaper

  - it gives a higher emf

- While charging the accumulators, one should :

- check the acid level with a lighted match

- shot-circuit the cell terminals to see that they are fully charged

- keep them in a well venilated room

- connect and disconnect them with the d. c. source

- A lead- acid cell is rechargeable because:

- its electrolyte is sulphuric

- it is a wet cell

- its chemical action is reversible

- its electrolyte has a high specific gravity

- A 100 Ah capacity battery should deliver a current of 8 A for approximately:

- 12 h

- 8 h

- 20 h

- 100 h

- When a lead-acid battery is needed to be kept idle for a long time then :

- overcharge the battery

- clean the plates with distilled water

- remove the electrolyte

- remove electrolyte and dry up the battery and store the same in a cool, dry and clean place

- 6 cells of 1.5 V each are connected in series as shown in the figure. Voltmeter connected across the terminals + and - reads :

- 4.5 V

- 6.0 V

- 7.5 V

- 9.0 V

- Which one of the following is not a maintenance free battery?

- Ni – Cd

- Ni – Fe

- Alkaline magnesium

- Lead – acid

- The advantage of alkaline magnesium battery Is:

- high internal resistance

- high nominal voltage. .

- low initial cost

- all the three stated above

- As compared to lead-acid cell, the efficiency of a Ni - Fe cell is less due to :

- lower e.m.f.

- smaller quantity of electrolyte

- higher internal resistance

- compactness of the cell.

- If a battery has not been Used for a long period then the same should be put on:

- float charge

- boost charge

- trickle charge

- freshening charge

- in order to increase the Ah capacity of a lead acid battery –

- Iess number of plates are to be, used

- mere number or plates are to be used

- height of the plates is to be decreased

- thickness of the plates is to be increased

- The state of charge / discharge of a lead acid battery can be known by measuring

- the level of the electrolyte

- the weight of the electrolyte

- the specific gravity of the electrolyte

- any one of the above stated parameter

- The instrument used for measuring the S. G or electrolyte of a lead-acid battery is called

- lactometer

- hydrometer

- hygrometer

- voltmeter

- The level of electrolyte in lead acid cells should be –

- 10 mm below the top of the plates

- 25 mm below. the top of the plates

- exactly at the top of the plates

- 10 to 15 mm above plates the top of the

- The e. m.f produced by a zinc-carbon cell is —

- 1.2 V

- 1.5 V

- 1.8 V

- 2.0 V

- A- dry cells declared unfit for use if its output e.m.f. has remained only _____ of its rated value.

- 40%

- 50%

- 60%

- 75%

- Cells can be classified into two categories namely ______

- cylindrical cell a d rectangular cell

- dry cell and wet cell

- primary cell and-secondary cell

- acid cell and alkali cell

- In order to increase the ampere hour rating of a battery cells are connected in –

- series

- parallel

- series parallel

- star

- A lead-acid battery should not be used, if it Is discharged beyond the minimum value of —

- 1.2 V per cell.

- 1.5 V per cell

- 1.7 V per cell

- 2.0 V per cell

- When a secondary cell is supplying current to the load, it is said to be –

- Loading

- Charging

- Discharging

- Unloading

- A discharged battery should not be tested by using –

- high rate discharge

- voltmeter

- hydrometer

- any of the above stated instruments

- The current rating of a battery is given in -

- amperes

- ampere-hours

- coulombs

- none or the above stated units

- The active material of a lead acid cell is/are –

- spongy lead

- lead peroxide

- dilute sulphuric acid

- all the above stated materials

- In a lead-acid battery, sulphation occurs due to

- Keeping the battery idle for a long time

- Trickle charging

- High rate discharging

- High rate charging

- The active materials of Ni – Fi cell are—-

- Nickel hydroxide

- Powdered iron and ferrous hydroxide

- 21% solution, of caustic potash

- All the above stated ‘materials

- During charging or discharging of a Ni-Fe cell

- Its e.m.f. remain constant

- Water is neither formed nor absorbed

- Corrosion fumes are produced

- Nickel hydroxide remained unsplit

- The battery charging method shown in the given figure is –

- constant voltage charging method

- constant current charging method

- rectifier battery charging method

- trickle Charging method

- in constant voltage battery charging method, the voltage applied across the battery is –

- equal to the battery nominal voltage

- twice the battery nominal voltage

- 2.3 t0 2.5 V per cell

- half the nominal voltage of the battery

- When a fresh battery is connected to DC source for charging, it is called

- inital charge

- freshening charge

- boost charge

- float charge

- boost charge is give to a —

- dead battery

- battery in danger ‘becoming over discharged during a working' shift

- fully charged battery

- completely discharged battery

- In order to charge a 6V battery in constant voltage charging method, the D.C. output of the charger should be………….

- 6 V

- 7 V

- 8 V

- 9 V

- The ratio of normal charging current to the trickie charging current is –

- 1:2

- 1:10

- 1:20

- 1:100

- Which type of battery is generally used in emergency lights

- Primary

- Alkaline

- Lead - acid

- Edison

- Which gas is produced during charging a lead-acid battery

- Hydrogen

- Methane

- Oxygen

- Carbon dioxide

- The nominal e.m.f. of a Ni-Cd cell is –

- 1.25 V

- 1.50 V

- 1.0 V

- 2.00 V

- Which type of cell is shown in the given figure?

- Denial cell

- Voltaic cell

- Lechlanche cell

- Lead – acid cell

- As compared to lead-acid cell, the efficiency of a nickel iron cell (Edison cell) is less due the –

- Lower e.m.f.

- Smaller quantity of electrolyte

- Higher internal resistance

- Compactness of the cell

- The disadvantage of constant current battery charging method is —

- its charge efficiency is low

- it takes comparatively long time to charge a battery fully

- in it, per ‘cell charging voltage should  no exceed 2.3 to 2.5 volts.

- all the three problems stated above

- Which one of the following is not a maintenance free battery –

- Ni-Cd

- Ni-Fe

- Alkaline magnesium

- Lead-acid

- Which of the following statement is true for the circuit shown below:

- The total voltage is 24 V

- The total voltage is zero

- The arrangement can be loaded upto 10 A maximum

- The arrangement can supply a maximum Current of 20 A at 12 V

- The colour of positive plate of a lead acid battery is ......

- brown

- grey

- white

- black

- The colour of negative plate of a lead acid battery is

- brown

- grey

- white

- black

- Lead acid cells are used in ......

- automobiles

- quartz watches

- portable radio receivers

- all the above

- The normal voltage of a fully charged lead acid cell is ......

- 1.1 V

- 1.5 V

- 2.2 V

- 3.0 V

- The negative plate of a lead acid battery has.…

- PbO2

- Pb

- PbSO4

- H2SO4

- The relative density of the electrolyte in a lead acid battery is 1.280. This value indicates that the battery is ......... .

- fully discharged

- half charged

- three quarter charged

- fully charged

- The main purpose of lead acid battery is to store electrical energy in the form of .........

- kinetic energy

- potential energy

- chemical energy

- mechanical energy

- What type of device is a lead acid battery?

- Electrical device

- Electronic device

- Chemical device

- Electro chemical device

- Which metal ‘is used to provide strength to the lead acid battery plates?

- Zinc

- Tin

- Antimony

- Copper

- The electrolyte used in a lead .acid battery is....

- sulphuric acid and diluted lead

- diluted lead and pure water

- pure water and distilled water

- distilled water and sulphuric acid

- The active material used in positive plates of lead acid-cell is

- Pb

- PbO3

- PbO2,

- PbSO4

- The material of the negative plate of the lead acid battery is ...... .

- Lead sulphate (PbSO4)

- Carbon rod

- Nickel chromium

- Pure lead (Pb)

- Which electrolyte is used in lead acid battery?

- Potassium Hydroxide (KOH)

- Ammonium Chloride (NH4CL)

- Dilute Sulphuric acid (H2SO4)

- Alkaline solution

- What is the minimum discharged voltage allowed after the fully discharged cell? condition of a lead acid

- 1.6 V

- 1.8 V

- 2.0 V

- 2.1 V

- What defect will occur in the lead acid cell, when the cell being left in a discharged condition for a long time?

- Partial short

- Buckling

- Hard sulphation

- Depolarization

- Which material is applied to the terminal posts of a lead acid battery to avoid corrosion?

- Ammonium sulphate

- Silica gel

- Petroleum jelly

- Sulphur powder

- What is the reason to keep open the vent plugs of lead acid cell during charging?

- To escape the gas freely

- To speed up the charging rate

- To increase the Specific gravity of the electrolyte

- To reduce the sulphation defect

- What is the name of the part of lead acid cell indicated as ‘X’ in the figure?

- Positive electrode

- Negative electrode

- Cell connector

- Terminal post

- How many lead acid cells are to be grouped for 12 V battery and by which connection?

- 6 cells in parallel

- 6 cells in series

- 12 cells in parallel

- 12 cells in series

- A 120Ah capacity battery can deliver a current of 8 A for a period of ......

- 20 hrs

- 15 hrs

- 12 hrs

- 8 hrs

- A carbon-zinc cell with an internal resistance of 0.5 ohm and an EMF of 1.5 volt is loaded with an external resistor of 19 5 ohm. Calculate the current ff flowing through the circuit in mA?

- 82 mA

- 75 mA

- 72 mA

- 60 mA

- Which instrument is used to check the battery for its charging condition by measuring the voltage?

- Hydrometer

- Volt meter

- High rate discharge tester

- Multimeter

- The purpose of cells are connected in the parallel is to………….

- Increase the voltage output

- Increase the ampere hour capacity of the cell

- Decrease the voltage output

- Increase the current capacity

- What is the unit of the capacity of a secondary cell?

- volt ampere

- kilo watt

- watt hour

- ampere hour

- In electrolysis process the mass of the substances liberated (or) deposited is directly proportional to………

- current and time

- voltage and resistance

- internal resistance

- specific resistance of the plate

- Battery electrolyte consist of approximately (by volume)……………

- 35 percent sulphuric acid and 65 percent water

- 65 percent sulphuric acid and 35 water

- 65 percent nitric acid and 35 percent water

- 35 percent nitric acid and 65 percent water

- What are vet holes provided in wet cells?

- To let gases escape during charging

- To let gases escape during discharging

- To let gases escape during charging and discharging

- To pour electrolyte

- Which type of energy transformation takes place during discharging of battery?

- Chemical energy to electrical energy

- Electrical energy to chemical energy

- Mechanical energy to electrical energy

- Electrical energy to mechanical energy

- A spark must not be-produced when a fully charged battery is .either being disconnected from a bench charger or being fitted to a vehicle because.........

- gas leaving the cells is highly explosive

- gas produced by a sudden battery discharge is corrosive.

- spark and gas react to form dangerous fumes.

- Spark can cause the battery polarity to reverse

- The positive plates in a battery cell is………..

- one more than the negative plates.

- one less than the negative plates,

- two Iess than the negative, plates.

- two more than the negative plates

- If the electrolyte level is low in a lead – acid battery, the battery should be topped up with………..

- diluted acid

- clean distilled water

- sulphuric acid and having a relative density of 1.280.

- an electrolytes similar to that used in the battery

- The ability of battery to supply a current similar to that required by the starter motor is tested by ……….

- A wide scaled hydrometer

- A high rate discharge tester

- A voltmeter scaled 0-200 V

- An ammeter scaled 0-100 V

- The need to top up a battery at frequent interval indicates that the………

  - battery is being over charged

  - charging rate is too low.

  - electrical system is shorting to earth.

  - battery separators are not sealing the cell.

- A maintenance free battery

- has lead antimony plate grid.

- has lead calcium plate grid

- does not contain acid

- does not contain water

- With the increase of battery temperature, the specific gravity of the electrolyte………..

- increases

- decreases

- stays the same

- none of the above

- The liquid in the battery, a mixture of water and sulphuric acid is called as……..

- electricity

- electro Chemical

- electrolyte

- electro liquid

- Figure shows an Instrument being used to check the specific gravity of the electrolyte of the battery. This instrument is called………..

- Hydrometer

- Hygrometer

- Hydroscope

- None of these

- During discharge of a battery, the active material in changed to both the plates is changed to…….

- spongy lead

- lead oxide

- lead sulphate

- lead carbide

- The capacity of a battery is determined by the number of plates and .........

- number of cells

- shape of plates

- size of plates

- number of separators

- To increase output voltage, battery cells are connected in……….

- series

- parallel ,

- series parallel

- none of the above

- Each solar cell has a open circuit voltage having equal value to……….

  - 0.55 V

  - 0.45 V

  - 0.35 V

  - 0.25 V

- How many electrodes does a cell consist of?

- One

- Two

- Three

- Four

- While fitting the battery, observe correct.........

  - Mounting

  - polarity

  - distilled water

  - nothing

- the amount of voltage that can be obtained from a single cell of a storage battery is above,,,,,,,,,,,,

- 2 V

- 4 V

- 6V

- 8V

- A12 V lead-acid battery consists of………….

- three cells in series

- by six cells in series

- three cells in parallel

- six cells in parallel

- According to the 20-hour rate, a battery that can deliver 5 A for 20 hours with the cell  voltage remaining above 1.75 V would be rated at.....

- 6 Ah

- 8.75 Ah

- 35 Ah

- 100 Ah

- What is output voltage of the battery shown in figure?

- 6 V

- 4.5 V

- 3 V

- 1.5 V

- What is the formula to be applied to calculate the mass M deposited in electrolysis process?

- M = I x t

- M = z x i x t

- M = zt/i

- M = i x t/z

- What detect will occur when the V battery is over charged or discharged at a high rate?

- Sulphation

- Local action

- Buckling

- Partial short

- What is the effect on internal resistance of the cell, it the cell, it the cell is kept idle for long time?

  - Remain the same

  - Increased

  - Decreased

  - Reduced to zero

- What is the name of the defect occurred in simple voltaic cell due to the presence of impurities on the zinc plate?

- Polarisation

- Local action

- Sulphation

- Buckling

- When the current is passed through the electrolyte, what type of effect of current will produce?

- Magnetic effect

- Heating effect

- Chemical effect

- Lighting effect

- The mass of a substance liberated from an electrolyte by one coulomb of electricity is termed as......

- quantity of electricity

- electro chemical equivalent

- electric charge

- electro chemical expression

- What is the method of charging the battery is called, when the battery is charged at a very low rate?

- Rectifier method

- Constant Current method

- Trickle charge method

- Constant voltage method

- Four number of cells are connected in parallel and the rating of each cell is 1.5 V and 16 Ah. What will be the “Ah rating of this battery?

- 16 Ah

- 32 Ah

- 48 Ah

- 64 Ah

- The ampere hour capacity of the battery in an emergency tube light having two 20 cm tubes will be......

- 4.5 Ah

- 6.5 Ah

- 10.5 Ah

- 12.5 Ah

- At the time of charging a lead acid battery which one of the harmful gas evolute?

- Oxygen

- Nitrogen

- Hydrogen

- Carbon dioxide

10. Capacitor

- A capacitor consists of two

- Insulation separated by a dielectric

- conductors separated by an insulator

- ceramic plates and one mica disc

- silver coated insulators

- The capacitance of a capacitor is NOT influenced by

- plate thickness

- plate area

- plate separation

- nature of the dielectric

- A capacitor that stores a charge of 0.5 coulomb at 10 volts has a capacitance

of ......... farads

- 5

- 20

- 10

- 0.05

- A capacitor of 60 micro farads capacity is connected across the supply voltage of 500 volts. Calculate the charge of the capacitor

- 0.03 coulomb

- 0.003 coulomb

- 0.8 coulomb

- 0.0003 coulomb

- Three capacitors 3uF, 4uF and 60uF are connected in series. Calculate the total capacitance.

- 13.33 uF

- 1.333 uF

- 133.3 uF

- 1333 uF

- Find the total effective capacitance of three condensers 5uF, 7uF and 12uF connected in parallel.

- 2.4 uF

- 0.24 uF

- 240 uF

- 24 uF

- A capacitor of capacitance 50 micro farads is charged to a potential difference of 200 volts. Find out energy stored in the condenser.

- 1 joule

- 100 milli jouies

- 10 joules

- 100 joules

- Given two identical mica capacitors of 0.1 MP each. One of these is charged to a potential difference of 125 volts and disconnected from the charging circuit. The charged capacitor is then connected in parallel with the uncharged capacitor. What Voltage will appear across the combination?

- 625 volts

- 62.5 volts

- 6.25 volts

- None of these

- A capacitor of 2 micro farads is charged to a potential difference of 150 volts and is connected in parallel with an uncharged capacitor of 4 micro farads. What is the voltage existing on the combination?

- 500 Volts

- 300 Volts

- 50 Volts

- 250 Volts

- By increasing the charge of a capacitor

- its capacity increases

- its capacity decreases

- its capacity remains unchanged

- its potential difference remains unchanged

- On connecting two charged capacitors having different P.Ds. and different capacities :

- the charges become equal and energy loss is caused

- the P.Ds. become equal and the charge loss is caused

- both the energy loss and charge loss are caused

- the P.Ds. become equal and the energy loss is caused

- By bringing a charged conductor closed to an earthed conductor: ’

- the voltage of the conductor will increase

- the capacity of the Conductor will Increase

- the capacity of the conductor will decrease

- both the voltage and capacity of the conductor will decrease

- On charging the capacitors of different capacities connected in parallel, each capacitor will have:

- equal charge and equal capacity

- equal P.D. and unequal charge

- unequal charge and unequal P.D.

- equal charge and unequal PD.

- For decreasing the capacity of a parallel plate capacitor:

- dielectric is filled between the two plates

- distance between the two plates is decreased

- distance between the Mo plates is increased

- area of the plates is increased.

- The capacity of a parallel plate capacitor is 4uF and the distance; between the two plates is 4 cm. if the distance is reduced to 2 cm then the capacity will become:

- 8 uF

- 12 uF

- 16 uF

- 2 uF

- The relation between the capacity (C), PD. (V) and charge (0) for a capacitor is given by:

- C = QV

- C = VQ

- C = Q2V

- C = Q.V2

- The capacity of a parallel plate air capacitor is 5uF it the air is replaced by glass, the capacity gets increased to 40uF. Hence, the permittivity of the glass is:

- 5

- 8

- 85

- 200

- The total capacity of two capacitors of 2uF each connected in series will be equal to:

- 2uF

- 4uF

- 1uF

- 12 uF

- The static electrical energy stored in a capacitor of 1.2 mF at 3000 volts P.D. will be equal to :

- 3.6 joules

- 5.4 joules

- 1.8 joules

- 7.2 joules.

- The unit of capacitance is :

- Coulomb

- Volt

- Farad

- Ampere

- if two capacitors of 2uF and 3uF are connected in series and then this group is connected across a capacitor of 4uF,  the totally capacity will be :

- 5.2 uF

- 2.5 uF

- 4.0 uF

- 52 uF

- The total capacitance of a group of capacitors is increased when they are connected in:

- Series

- Parallel

- Series - parallel

- Parallel -  series

- Opening the switch of an inductive circuit sets an arc across the switch contacts due to large voltage induced and stored energy in magnetic field. The arc is suppressed in such circuits by connecting......

- a capacitor across the supply

- a capacitor across the switch contacts

- arcing horns across the switch contacts

- another small inductor across the switch contacts in reverse direction.

- The capacitance to the capacitor is directly proportional to the…….

- material of the plate

- area of the plate

- voltage across the plate

- polarity of the plates

- The amount of current at a point in an electric circuit will be equal to........ if 20 coulombs charge is passed in2 seconds through that point.

- 0.1 ampere

- 10 amperes

- 22 amperes

- 40 amperes

- When testing a capacitor with a multimeter, the needle is showing zero position right from the beginning. What does it indicate?

- The capacitor is open circuited

- The capacitor is short circuited

- The capacitor is charged in capacity

- None of the above

- The SI. unit of the quantity of electricity is .......

- Ampere

- Volt

- watt hour

- coulomb

- One ampere current is passed for one second is called as one.......

- Ampere

- unit

- coulomb

- ohm

- The value of charge (Q) is .......

when: (C) capacitance, (V) voltage, (A) current.

- Q = (c)V

- Q = CA

- Q = CV

- Q = VC

- What is the formula to find out the quantity of electricity ‘Q’?

- Q = Current Time

- Q = Current2 x Resistance

- Q = Current x Time

- Q = Current ×Voltage Time

- Find the value of the capacitance of a capacitor that requires 0.5 coulomb to charge it to 25 volts?

- 0.2 farad

- 0.002 farad

- 0.0002‘farad

- 0.00002 farad

- Two capacitors of 3uF each are connected in parallel. Their total effective capacitance is.........

- 1.5uF

- 3uF

- 6uF

- 12uF

- The type of capacitor ‘used in capacitor start motor is .......

- electrolytic capacitor

- ceramic capacitor

- paper capacitor

- mica capacitor

- A 6 amps 250 V switch which controls an equipment is found to be sparking on operation and gets damaged often. The spark that occurs in the switch contacts. can be reduced by connecting one of the following device across the contacts. The device is a........

- capacitor

- inductor

- high resistance

- low resistance

- You are testing a capacitor by voltmeter method as shown in the figure.

What the switch is closed, the indication of voltmeter for good condition of

capacitor is........

- zero and does not deflect

- supply voltage continuously

- less than supply voltage

- supply voltage initially and slowly moves down to zero

- What is the effect on the capacitance value if the dielectric constant of the material of the capacitor is increased?

- Capacitance Value increases

- Capacitance value decreases

- Capacitance value remains same

- Capacitance value becomes zero

- What will happen to the capacitance value, if the area of the plates of the capacitor is reduced?

- Capacitance Value increases

- Capacitance value reduces

- Capacitance value remains same

- Capacitance value becomes Zero

- The capacitance value of a capacitor depends on factors,

- 2

- 3

- 4

- 5

13. Magnetism and Electromagnetism

- A natural magnet is called :

  - steel

  - load stone

  - magnetism

  - soft iron

- Externally, the magnetic lines of force travel: .

  - north to south

  - south to north

  - negative to positive

  - in both the directions

- The magnetism that remains in a magnetic material after the withdrawal of magnetising force is called:

  - permeability

  - induction

  - residual magnetism

  - saturation

- lron becomes magnetised by induction when it is :

- heated

- brought near to one end of a magnet

- suspended in north-south direction

- treated with some chemical

- The force acting between two magnetic poles is:

- directly proportional to the distance

- inversely proportional to the distance

- directly proportional to the square of the distance

- inversely proportional to the square of the distance

- Permeance is analogdus to :

  - conductance

  - resistance

  - reluctance

  - none of these

- The permeability of a material means:

  - strength of a magnet

  - strength of an electromagnet

  - magnetisation remained in an iron bar on withdrawal of the magnetising force

  - the conductivity of a magnetic material for the magnetic lines o force

- Which of the following material is used for making permanent magnets?

- Carbon steel

- Platinum cobalt

- ALNICO-V

- All of these

- The relative permeability of a magnetic material depends on the:

- flux

- type of material

- type of magnetising force

- nothing

- In order to induce more magnetic flux a magnetic circuit should have minimum:

  - Reluctance

  - Resistance

  - Reactance

  - Conductance

- The simplest way of varying the flux produced by an electromagnet smoothly in an electrical device is by:

- increasing and decreasing the number of turns of the coil

- using an exciting coil with number of tapping’s

- varying the current through the exciting coil.

- varying the core position of the exciting coil

- The figure given here explains:

- Helix rule

- End rule

- Cork-screw rule

- None of these

- Which rule is used to determine the direction of the magnetic field?

- Helix rule

- End rule

- Right hand grip rule

- Fleming’s right hand rule

- How many laws are there in Faraday’s laws of electro-magnetic induction?

- 1

- 2

- 3

- 4

- Which among the following is the direction of flow of line of force around a bar magnet?

- Out of the east pole into the west pole

- Out of the west pole into the east pole .

- Out of the north pole into the south pole

- Out of the south pole into the north pole

- Which one of the following is NOT a property of a magnet?

- When suspended freely moves to a fixed position relative to North and of earth South pole

- Produces magnetism in a nearby magnetic substance by induction,

- Unlike poles repel each/other, and like I poles attract each other

- Attracts magnetic substances like iron and steel

- Which law/rule states that the magnitude of the induced e.m.f is directly proportional to the rate of change of flux linkage?

- Lenz’s law

- Faraday’s first law of- electromagnetic induction

- Faraday’ 8 second law of electromagnetic induction

- Fleming right hand rule

- What will happen if a bar magnet is broken/cut into two pieces?

  - Each piece will become a separate magnet having north and south pole

  - Magnetic property will be destroyed in both the pieces completely

  - Longer piece only will have magnetic property with north and south pole on either end

  - One piece will have only north pole and the other piece will have only south pole

- What will be the direction of the magnetic field shown in the figure, when the two wires carrying current in, opposite direction?

- Will be in the same direction

- Will form loop around the wires

- Will be in opposite direction

- Will aid one to another

- When the distance between the turns of wire in a magnetic coil is increased then the inductance of the coil will........

- increase

- decrease

- become zero

- remain same

- Which of the following materials have higher retentivity of magnetism?

- Alnico

- Manganese

- Copper

- Bismuth

- A substance that has low retentiveness can be used for manufacturing of.........

  - electromagnets

  - permanent magnets

  - bar magnets

  - paramagnets

- What type of electrical current effect will be produced when the current is passed through the spirally wound coil?

- Heating effect

- Magnetic effect

- Chemical effect

- Thermal effect

- The unit of reluctance is.....

- ampere/cm2

- weber/m2

- ampere turns/weber

- ampere/weber

- The resistance in electrical circuit is analogous to magnetic circuit is ........

- retentivity

- reluctance

- reactance

- permeability

- Which magnetic term is the analogous (similarity) to the electrical term of current?

- Flux density

- Permeability

- Flux

- Reluctance

- What is the, unit ct permeability?

- ampere turns

- weber/m2

- ampere turns/weber

- no unit

- What is the unit of magneto motive force (mmf)?

- tesla

- ampere turns

- ampere/turns

- weber/metre2

- The flux per unit area of a material is known as

- flux density

- magnetic lines of force

- reluctance

- magneto motive force (mmf)

- The SI unit of flux density of magnet is

- weber/metre

- tesla

- lines/cm2

- gauss

- Taste is the unit of.........

- magneto motive force (mmf)

- flux

- flux density

- permeability

- Total flux density of magnetic core is ........

- B = ϕA

- B = Aϕ

- B = Lϕ

- B = ϕL

- Tesla is the S.l. unit of‘.......

- flux density

- magneto motive force

- reluctance

- magnetic field strength

- Which one is NOT an application of electromagnet?

- Calling bell

- Electric iron

- Protection relays

- Transformers

- Which substance belongs to paramagnetic materials?

- Glass

- Paper

- Wood

- Air

- A material that can be magnetized only very slightly is called……..

- magnetic

- paramagnetic

- diamagnetic

- ferromagnetic

- Which of the following materials is a paramagnetic substance?

- Iron

- Glass

- Aluminium

- Nickel

- Which of the, following materials is a diamagnetic substance?

- Platinum

- Glass

- Copper

- Aluminium

- Which substance is the example of diamagnetic material?

- Manganese

- Nickle

- Glass

- Cobalt

- In which class of magnetic substance, water does belong to?

- Diamagnetic substances

- Paramagnetic substances

- Non- magnetic substances

- Ferromagnetic substance

- Which of the following materials is a ferromagnetic substance?

- Platinum

- Aluminium

- Steel

- Copper

14.  Alternating Current

- Power factor of an A.C. circuit is equal to:

- cosine of angle between voltage and current

- tangent of angle between voltage and current

- impedance/resistance

- apparent power/real power.

- The resistance of an open circuit is equal to:

- zero

- lesser, than one ohm

- infinite

- lesser than before.

- The unit of inductance is :

- Farad

- Ohm

- Mho

- Henry

- The power is dissipation of a pure inductor

- very low

- zero

- very high

- lesser than unity.

- On passing A.C. through an inductor:

- voltage leads the current

- voltage and current remain in phase

- voltage lags behind the current

- voltage drop is equal to / x Z

- The magnitude of inductance ............ with an increase in the frequency.

- decreases

- remains unchanged

- increases

- becomes zero

- The power factor of a pure resistive circuit is equal to:

- zero

- 8.1

- 0.5

- 0.8

- The difference between peak positive and peak negative values of 'an ac. Voltage is called :

- Maximum value

- average value

- effective value

- peak to peak value

- The r.m.s. value of AC is equal to :

- 0.637 of max. value

- 0.707 of max. value

- 1.414 of max. value

- the maximum value

- The value of form factor for a pure sine-wave is:

- 1.414

- 0.707

- 0.637

- 1.11

- if the voltage and current have a phase difference of 90° in an ac circuit the power will be

- maximum

- minimum

- 1.1 x V x l

- Zero

- Which of the following frequencies has the longest time period?

- 1 Hz

- 10 Hz.

- 1 kHz.

- 100 kHz.

- The total voltage in a series R- L circuit leads the current by an angle:

- of 90° angle

- between 0° and 90°

- between 90° and 180°

- between 0° and 360°

- The total voltage in a series R - C circuit lags the current by an angle :

- of 45°

- of 0°

- of 90°

- between 0° and 90°

- The net reactance in a R - L - C circuit is:

- XL

- XC

- 0

- XL - XC

- The frequency at which XL : XC is called:

- natural frequency

- resonance frequency

- neutral frequency

- super frequency.

- The formula for determining resonance frequency is :

- fr = L.C.

- fr = 1L.C.

- fr = 12πL.C.

- fr = 2πL.C.

- In a series R - L - C circuit the impedance is equal to______ and current equal to ______ at resonance.

- minimum, maximum

- zero, zero

- infinite, zero

- zero, infinite.

- The power factor is defined as the ratio of:

- volts amperes to watts

- watts to volt amperes

- volt amperes reactive to watts

- watts to volt amperes reactive.

- The power factor of an inductive circuit can be improved by connecting a capacitor to it in :

- series.

- either series or parallel.

- parallel.

- depends on the value of the capacitor

- The power factor of an ac. circuit can be determined by the formula

- Cos ϕ = RZ

- Cos ϕ = actual powerapparent power

- =R√.R2 + (XL – XC)2

- All of above

- What is the power factor of a pure resistive circuit?

- 1.0,

- 0.95

- 0.9

- 0.85

- The power factor in a AC single phase:

- Cos Ɵ = RZ

- Cos Ɵ = 1RZ

- Cos Ɵ = ZR

- Cos Ɵ = R2 + Z2

- The power factor is defined as the ratio………

- Cos Ɵ = True PowerApparent Power

- Cos Ɵ = Apparent PowerTrue Power

- Cos Ɵ = True PowerReactive Power

- Cos Ɵ = Apparent PowerReactive Power

- Which supply source changes the polarity consultancy?

- DC supply

- AC supply

- Dynamo

- Battery

- The average value of alternating current for a pure sine wave is equal to.......

- 0.536 x maximum value of current

- 0.636 x maximum value of current

- 707 x maximum value of current

- 0.737 x maximum value of current

- The RMS value of AC voltage is......

- Vrms = 0.770 Vmax

- Vrms = 0.707 Vmax

- Vrms  = 0.077 Vmax

- Vrms  = 0.007 Vmax

- Inductive reactance of an inductor ......

- XL = 2πf L

- XL = 12πf L

- XL = 2πfL

- XL = fL2π

- The reactance of a 2 henry choke coil at 100Hz is........

- 100 ohm

- 216 ohm

- 718 ohm

- 1258 ohm

- The formula to calculate the capacitive reactance, XC of the AC circuit is

- XC = 2.πf Cohm

- XC = 2πf C ohm

- XC = 1 2πf C ohm

- XC = 1√2πf C ohm

- The single phase domestic AC supply voltage in india is ......

- 110 V

- 230 V

- 440 V

- 550 V

- What is the value of resistance of coil, having impedance of 50 Ὡ connected across 240 V at 0.8 p.f.?

- 50 Ὡ

- 60 Ὡ

- 6 Ὡ

- 4.8 Ὡ

- What is the opposite term, of conductance?

- Resistivity

- Resistor

- Resistance

- Reluctance

- What is the time period for the frequency of 50 Hz?

- 200 milli seconds

- 20 milli seconds

- 2 milli seconds

- 0.2 milli second

- What is the form factor of pure sine wave?

- 1.11

- 1.01

- 1.00

- 0.11

- The phasor diagram of auxiliary winding circuit of a ceiling fan is given below :

What is the nature of power factor of the circuit?

- Unity

- Lagging

- Leading

- Zero

- The maximum voltage is 99 V. What is the effective value of this sine-wave?

- 70 V

- 44.5 V

- 49.5 V

- 35 V

- If the supply voltage V = 10 volts and the voltage across the cell VL = 8 volts, the amount of voltage read by the voltmeter VR in the given figure is:

- 2 volts

- 6 volts

- 10 volts

- 18 volts

- A contactor coil is designed for 220 V. d. c. What happens if it is connected to 220 V a c.?

- Too large a current is drawn by the coil and it may destroy

- it draws the same current

- it works better

- Too small a current is drawn by the coil and its operation is not certain.

- The capacitive reactance (XC) of a capacitor varies:

- inversely With the frequency

- directly with the frequency

- directly with the applied voltage

- inversely with the applied voltage.

- Which one of the following relationships is true between true power (PT), reactive power (PR) and apparent power (PA)?

- PA =  PT + PR

- PA =  PT  -  PR

- PA = √P2T  +  P2R

- PA = P2T  +  P2R

- What is the power consumed in the circuit given below at a power factor of 0.6?

- 32 W

- 48 W

- 60 W

- 112 W

- The current IT in the circuit shown here under is equal to:

- 14 A

- 2 A

- 10 A

- 8 A

- In a 3-phase generator 220 volts a. c. is induced in each coil. Which of the following values is indicated by the voltmeters?

- 220 V x √3 = 330 V

- 220 V

- 220 V ÷ √3 =127 V

- 220 V x 5= 310 V

- The positive and negative halves of a complex wave are symmetrical when

- it contains even harmonics

- phase difference between even harmonics and fundamental is 0 to p

- it contains odd harmonics

- phase difference between even harmonics and fundamental is either

π2 or 3π2

- Non-sinusoidal Wave forms are made up of

- different sinusoidal wave forms

- fundamental and even harmonics

- fundamental and odd harmonics

- even and odd harmonies only

- In a 3--phase system ------------ the harmonic has negative phase sequence of RBY

- 9

- 13

- 5

- 15

- A complex voltage wave is applied across a pure capacitor. As compared to the fundamental voltage, the reactance offered by the capacitor to the third harmonic voltage would be

- nine times

- three times

- one third

- one ninth

- When a pure inductive coil is fed by a complex voltage wave, its current wave

- has larger harmonic content

- is more distorted .

- is identical with voltage wave

- shows less distortion

- Which of the following harmonic voltage components in a 3phasesystem would be in phase with each other?

- 3rd, 9th, 15th etc.

- 7th, 13th, 1c etc.

- 5th, 11th, 17th etc.

- 2nd, 4th, 6th etc.

- Alternating currents and voltages are represented by straight line with an arrow head to represent the magnitude and direction.

What does this representation called?

- Scalar quantity

- Vector quantity

- Alternating quantity

- Phase quantity

- While finding solution to AC circuit, vector quantities are added or subtracted one of the method for solving vector quantities is........

- scalar method

- algebraic method

- numerical method

- parallelogram method

- Which formula is used to calculate the impedance (Z) involving resistance and reactance?

- Z = √R2 + X2

- Z = R2 + X2

- Z = R2 -  X2

- Z = R + X

- circuit is (Y) in AC parallel

- ohm

- farad

- henry

- mho

- In the figure given below you see the wave forms of voltage and current in a AC circuit.

Which electrical components does this AC

circuit contain?

- R and L in series

- R and C in series

- C only

- L only

- What will be the voltage across the resistance. “R2” When the switch “S1” in the circuit diagram below is opened?

- 400 V

- 380 V

- 220 V

- 190 V

- What is the reading of Ammeter in the coil shown below?

- 9 A

- 5 A

- 4 A

- 3 A

- When several impedances say Z1, Z2 and Z3 are connected in parallel their combined impedance Z is.......

- Which of the following is NOT a characteristic of a three phase AC supply?

- Same phase angle difference

- Same peaky values

- Same RMS values

- Same instantaneous value

- In a 3 phase system three phased voltages are displaced by an angle of .......

- 90°

- 120°

- 180°

- 270°

- A circuit has an impedance of 22 ohms and drawing a current of 10 amperes at 0.8 power factor lagging. Which one of the following will be the apparent power?

- 2.2 kVA

- 22 kVA

- 220 kVA

- 275 kVA

- Which one is the correct unit of the A reactive power?

- VA

- kW

- kVAR

- kVA

- The line voltage E in a star connected system is equal to.......

- EL = √3 Ep

- EL = Ep

- EL = 3.Ep

- EL = 13.Ep

- You have to find true power, apparent  power, reactive power and power factor of a phase circuit. From which one or the, following values can be determined?

- By 2 element type 3 phase wattmeter

- By 2 wattmeter method;

- By 3 element 4 wire 3 phase, wattmeter

- By 3 element 3 wire 3‘ phase wattmeter

- The wattmeter, ‘W2’ indicates the reading in reverse direction in the figure given below:

How will you get the forward reading?

- Interchange the connections to the meters W1 and W2 (i.e.) L1 and L3

- Reverse the potential coil of the connection of the meter W1

- Reverse the potential coil of the connection of the meter W2

- Reverse the Connections of both the current and, pressure coil or the W2

- Which figure shows the correct connection of two watt meter method of measuring 3 phase power

- In a 3 phase circuit the reading of two wattmeter at unity 'power faster is………….

- P = 2 x W1

- P = W1 – W2

- P = W2 – W1

- P = 2W1

15.16. Basic Electronics

- Which one of the following materials is used for doping’ to prepare P type semi conductor –

- Arsenic

- Antimony

- Indium

- Phosphorus

- Which one of the following materials is used for doping’ to prepare N - type semiconductor –

  - Indium

  - Gallium

  - Arsenic

  - Silver

- The function of a diode can be Compared with that of a --

  - fuse

  - relay

  - coil

  - switch

- The potential difference across a diode is termed as –

- reverse potential

- PN potential

- barrier - potential

- forward potential

- What will be the voltage across the silicon diode shown in the given figure

- 0.3 V

- 0.7 V

- 1.0 V

- 12.0 V

- A semiconductor element in its pure form is called a/an semiconductor and in its doped form is called a/an semiconductor –

- crystal, compound

- doped, undoped

- intrinsic, extrinsic

- normal, abnormal

- What will be the voltage across the diode shown in the given figure –

- 0.3 V

- 0.7 V

- V

- equal to the applied voltage

- it a diode shows very high resistance in one direction and a low resistance in the opposite direction then its condition is —

- good

- short-circuited

- open-circuited

- defective

- Silicon diodes are preferred over Germanium diodes because –

- their resistivity is lesser

- their barrier potential is higher

- their barrier potential is lower

- these are lesser temperature sensitive

- Below the break down region the leakage current of a silicon diode is of the order of —

  - 1 µA

  - 100 µA

  - 1 mA

  - 100 mA

- The maximum reverse voltage that a diode can with stand is called ---

  - threshold voltage

  - peak inverse voltage

  - cut in voltage

  - avalanche voltage

- Diodes are categorised as Bipolar Device because —

- they can work both on positive and negative supplies

- they consist of both P-type and N-type semiconductor

- the conduction of current through them is caused by both holes and free electrons

- of ail the three above stated reasons

- In BY 127, B stands for ………. and Y stands for ……….

- Germanium, rectifier

- Silicon, rectifier

- Germanium, signal detector

- Silicon, "signal detector

- Two important specifications of a diode are –

- leakage current (Il) and current (IS)   saturation

- IS and forward current (IF)

- PIV and Is

- PIV and IF

- What will be the voltage across the diode in non-conducting state –

- Zero

- 230 V

- 24 V

- 0.7 V

- PIV across each diode of a bridge rectifier is equal to —

- 2 x VS (rms)

- 2 x VS (peak)

- VS  (rms)

- VS (peak)

- The circuit shown in the given figure is a simple

- inverter circuit

- H.W. rectifier circuit

- voltage regulator circuit

- zener controlled filter circuit

- The phenomenon which causes reverse break down in a zener diode is called

- Avalanche effect

- Hall effect

- breakdown effect

- Seeback effect

- The power dissipation of a zener diode…..

- Is minimum when load current is zero

- is maximum zero when load current is

- is always zero

- remains constant irrespective of load current variation

- Which one is the symbol of zener diode out of the symbols shown in the given figure?

- A band-pass filter circuit is that which -

- passes low frequency signals

- passes high frequency signals

- passes signals between two frequency limits

- rejects signals between two frequency limits

- A filter circuit that allows signals above a particular frequency to pass-through it is called a ………..

- low-pass filter

- high-pass filter

- band-pass filter

- band stop filter

- What-type of filter circuits are used in graphic equalizers?

- Low-pass filter

- High-pass filter

- Band-pass filter

- Band-stop filter

- The filter circuit which attenuates or rejects a specific band of frequencies is called a –

- band pass filter

- band-stop filter

- high-pass filter

- low--pass filter

- Which type of filter is used to block the humming sound of 50 Hz produced by as main?

- Low - pass

- High - pass

- Band – pass

- Band – stop

18. Oscillators

- Feedback used in a RC oscillator is of…….

- negative type

- positive type

- any type, positive or negative

- none of the stated above

- In RC oscillator the phase of the feedback voltage is shifted to….. by employing a RC phase shifting network –

- 0°

- 60°

- 90°

- 180°

- An amplifier can work as an oscillator when its loop gain becomes —

- Infinity

- 0

- -1

- +1

- Weinbridge Oscillator uses —

- positive feedback

- both positive and negative feedback

- negative feedback

- none of the stated above

- An oscillator which generates sine wave oscillations is called —

- damped wave oscillator

- relaxation oscillator

- feedback oscillator

- harmonic oscillator

- An oscillator which generates non sinusoidal waves is called —

- harmonic oscillator

- feedback oscillator

- relaxation oscillator

- none of the above stated

- We frequency generated by an oscillator can be measured by means of –

- a voltmeter

- a frequency meter

- either CRO or an electronic frequency counter

- an AVO Meter

- Barkhausen criterion for sustaining oscillation is expressed as

- A.ß = 0

- A.ß = 1

- A.ß >1

- A.ß <1

- The amplitude of damped waves ~-~

- decreases with time

- increases with time

- is zero

- is constant

- A parallel fed Hartley oscillator is preferred over series fed circuit because of……….

- lower cost

- higher frequency stability

- lower power consumption

- circuit simplicity

- A Colpitt’s Oscillator employs –

- inductive feedback

- capacitive feedback

- negative feedback

- stray capacitance feedback

- The circuit shown in the given figure is of a…..

- Hartley series-fed oscillator

- Clapp oscillator

- Crystal oscillator

- Colpitt‘s oscillator

- Crystal oscillators are used in transmitters because of their ----

- frequency stability property

- low cost

- rigidness

- simplicity

- A Colpitt’s oscillator generates –

- square waves

- sine waves

- saw tooth waves

- damped waves

- An alternating emf, is developed across two opposite phases of a crystal when –

- a small amt. is applied to it

- a magnetic field is applied to it

- a static field is applied to it

- compression and: expansion takes place

- The circuit shown in the given figure is of a and it is preferred for generating frequencies above1 MHz –

- Hartley parallel-fed oscillator

- Colpitt’s oscillator

- Clapp oscillator

- Crystal oscillator

- RC oscillator is suitable for generating

- high frequencies

- very high frequencies

- very low frequencies

- audio frequencies

- The frequency generated by a RC oscillator is given by the expression

- f = 12π.R.C.

- f = 12πR.C.

- f = 1R.C.

- f = 2π.R.C.

- An as table multivibrator has –

- no stable state

- one stable state

- two stable states

- either one or two stable states

- IC 555 is operated at –

- 3 to 10 volts d.c.

- 1 to 5 volts d.c.

- -5 to +5 volts d.c.

- +5 to +15 volts d.c.

- A blocking oscillator is a ——

- sine wave oscillator

- square wave oscillator

- saw-tooth wave oscillator

- none of the stated above

- An oscillator circuit which is meant for converting sine wave signal into square wave signal is called a —-

- Schmitt trigger

- blocking oscillator

- multivibrator

- Weinbridge oscillator

- In a BC phase shift oscillator, the frequency generated depends on —

- VCC applied

- transistor used in the circuit

- time constant of RC network

- all the components mentioned above

- Square wave signals play important role in –

- amplifier

- rectifier

- oscillators

- digital equipments

- A sinusoidal oscillator is an amplifier with............. feedback.

- no

- positive

- negative

- zero

- To initiate oscillation, the loop gain of the oscillator must be................. unity.

- less than

- equal to

- greater than

- equal to or less than

- 1 mega hertz oscillator produces a wave whose length is

- 3 metres

- 3 cm

- 300 metres

- 300 cm

- A Colpitts oscillator uses

- a tapped inductor

- a tapped capacitor

- no tapped inductor or capacitor

- both tapped inductor and capacitor

- Crystal oscillator is used because

- it gives higher output voltage

- it has high efficiency

- the frequency of oscillations remains substantially constant.

- it requires very low D.C. supply voltage

- An audio oscillator uses

- positive feedback

- negative feedback

- both positive and negative feedback

- none of these

- Weinbridge oscillator is basically a

- pulse generator

- sine wave generator

- square wave generator

- triangular wave generator

- In a transistor Hartley oscillator,

- inductive feedback is used.

- untapped coil is used.

- entire coil is in the output circuit.

- none of the above

- To generate a one mega hertz signal, the most suitable oscillator is

- Colpitt’s oscillator

- phase shift oscillator

- Weinbrdige oscillator

- None of these

- in an RC phase shift oscillator, the minimum number of RC. networks to be connected in cascade will be

- one

- two

- three

- four

- The most stable sine wave oscillator is

- Colpitts

- Armstrong

- Phase shift

- Crystal

- lf RF signal is 600 kHz and local oscillator frequency is 1000 kHz, then the output of the mixer will be

- 400 kHz

- 600 kHz

- 1000 kHz

- 1600 kHz

- in an as table multivibrator, frequency of oscillation is given by

- 0.69 Rb .Cb

- 1.38 Rb .Cb

- 11.38 Rb .Cb

- 11.69 Rb .Cb

- As table multivibrator has

- a stable state

- two quasi stable states.

- an oscillator

- none of these

- The output time period of a transistorized monostable multivibrator using base resistor Rb and coupling capacitor Cb for the output transistor is given by

- Rb.Cb

- 0.69 Rb.Cb

- 2 Rb.Cb

- 1.38 Rb.Cb

- A monostable multivibrator is widely used

- in counting circuits.

- in memory and timing circuits.

- for regeneration of distorted waves.

- for generating square waveforms.

- A monostable multivibrator

- has no stable state.

- returns to its stable state automatically.

- has no energy storage component.

- is frequently used in counting circuits.

- A bistable multivibrator

- has no stable state.

- has two stable states.

- has one energy storage element.

- switches between its two states automatically.

- A bistable multivibrator is a

- triggered oscillator.

- crystal oscillator.

- free running oscillator.

- Saw-tooth wave generator.

- Schmitt trigger is basically

- an astable multivibrator

- a bistable multivibrator.

- a monostable multivibrator

- an oscillator.

- Trigger pulses are essential for the operation of a bistable multivibrator because

- there are no RC timing circuits

- the circuit cannot switch between its two states automatically.

- transistor operation is controlled by the triggering pulses.

- all of these

- The frequency of oscillation in case of an astable multivibrator depends mainly on the

- value of transistor

- RC values of the circuit

- value of V∞ power supply.

- collector resistors.

- In a multivibrator

- feedback between two stages is 100%

- feedback employed is positive.

- when one transistor is on, the other is off.

- all of the above.

- An astable multivibrator produces

- sine wave

- triangular wave

- saw-tooth wave

- square wave

- The width of the output pulse of a monostable multivibrator is given by

- RC

- 5 RC

- 0.69 RC

- none of the above

- A crystal oscillator provides very stable frequency because of

- high stability of the crystal

- the rigid crystal structure

- XLR ratio of the crystal is low

- high Q of the crystal

- For generating a 1 kHz signal, the most suitable circuit is

- Hartley oscillator

- Colpitts oscillator

- un tuned collector oscillator

- Wein bridge oscillator

- Which one of the following oscillators is used for generation of high frequencies?

- R.C. phase shift

- Wein bridge

- L.C. oscillator

- Blocking oscillator

- A relaxation oscillator is one which

- produces sinusoidal output

- produces non-sinusoidal output

- relaxes indefinitely

- oscillates continuously

- in a piezo electric crystal oscillator, the oscillation or turning frequency is linearly proportional to the

- mass of the crystal

- square root of the mass of the crystal

- square of the mass of the crystal

- inverse of the square root of the mass of the crystal

- Which one of the following statements is true of phase shift type and Wein bridge type RC. oscillators?

- Both use positive feedback

- The former uses positive feedback only, whereas the tater uses both positive and negative feedback.

- The former uses both positive and negative feedback only whereas later uses positive feedback

- Both use negative feedback

- The essential requirements of a saw tooth wave generator, in addition to a DC. power source are

- a switching device

- a resistor

- a capacitor

- all of the above

- The type of multivibrator used for generation of clock pulses is

- monostable multivibrator

- astable multivibrator

- bistable multivibrator

- none of the above

- Circuit used for production of delay is

- an astable multivibrator

- a bistable multivibrator

- a monostable multivibrator

- none of the above

- .........is used for accurate and stable time base circuit in a digital frequency meter.

- Quartz

- Rochellel salt

- Aluminium

- Carbon

- A 555 timer can be used as

- an astable multivibrator only

- a monostabie multevibrator only

- a frequency divider only

- an astable multivibrator or a monostable multivibrator or a frequency divider

- Square wave generator is a special type of

- variable frequency oscillator

- sweep frequency generator.

- function generator and pulse generator.

- none of the above.

19. Domestic and industrial Wiring

- Lines are drawn on the middle portion of capping so that :

- the same looks beautiful.

- the screws can drill the wires.

- the screws can be fitted at proper places in the wiring.

- the same remains safe

- The effect of smoke, acid and alkali is negligible on:

- T.R.S. wire

- V.l.R. wire

- Flexible wire

- Lead sheathed wire.

- Tumbler switches are made of :

- Iron

- rubber

- plastic

- bakelite

- The minimum cross sectional area of a copper conductor used in a ‘light and fan’ sub-circuit should be :

- 1 mm2

- 2 mm2

- 2.5 mm2

- 3 mm2

- The minimum cross-sectional area of an aiuminium conductor used in a ‘light and fan sub circuit should be:

- 1 mm2

- 1.5 mm2

- 2.5 mm2

- 4 mm2

- The minimum cross-sectional area of a copper conductor used in a ‘power’ sub-circuit should be:

- 1.5 mm2

- 2.0 mm2

- 2.5 mm2

- 4.0 mm2

- The minimum cross-sectional area of an aluminium conductor used in a ‘power’

sub-circuit should be :

- 1.5 mm2

- 2.0 mm2

- 2.5 mm2

- 4.0 mm2

- if mechanical and fire safety are important factors, the wiring should be :

- lead sheathed type

- conduit type

- C.T.S. type

- Casing-capping type.

- A stair case point is operated with :

- two one-way switches

- two two-way switches

- two intermediate switches

- none of these.

- Outlets above 15 A rating should be connected through a :

- Two-way switch

- D.P, switch

- T.P. Switch

- D.P.S.T. switch,

- What is the maximum permissible wattage of all the appliances on a 16A fuse branch circuit?

- 3.0 kW

- 3.9 kW

- 4.0 kW

- 4.6 kW

- In batten wiring, for the batten Width of 19 mm what length of overlapping portion do you use for straight joint?

- 12 mm

- 25 mm

- 30 mm

- 40 mm.

- The rating factor which the normal current carrying capacity should be multiplied to get the permissible current in the cable for the circuits protected by close-excess current protection will be

- 0.81

- 0.91

- 1.01

- 1.23

- The bed lamp in the circuit shown below is not lighting up.

You found that there is voltage across one terminal of lamp and the earth point of the system by operating switches 2 and 3 in both positions. Which one of the following tests do you carry out next after disconnecting the supply?

- Continuity test between switches 1and 2

- Continuity test between switch 2 and ceiling rose

- Continuity test of the neutral conductors of the lamp.

- Continuity test at the switch connections at switches ‘2 and 3 and half wire.

Which switch or switches can control the dressing table lamp?

- Switch nos. 1 and 2

- Switch nos. 2 and 3

- Switch nos. 2

- Switch no 1

- At what maximum distance do you fix saddles from a switch box, in a conduit wiring?

- 30 cm

- 50 cm

- 60 cm

- 70 cm.

- Which one of the following material is used to fix the screw on concrete wall and ceiling?

- Link clips

- Bushings

- Saddles

- Row plugs.

- You have to install cables for a switch board using clips on batten. The distance between the accessories is 3.6 m. How many clips do you need for horizontal run of the cable?

- 20

- 27

- 37

- 70

- in the metal conduit system of wiring where there is zig-zag wall protection the cables can be run using :

- standard solid bends

- standard inspection bends

- metal flexible conduit

- offset bends.

- For tapping Supply from mains to different branches three types of layout for wiring system are used. As per lE regulation,

one ring main circuit has to be there for

every specified floor area or part there

of. How much is this area?

- 100 sq. mts

- 120 sq. mts.

- 150 sq. mts.

- 160 sq. mts.

- The only wiring which has to be planned well before starting of a house construction work and executed along with the construction is :

- cleat wiring

- batten wiring

- conduit surface wiring

- conduit concealed wiring.

- Corridor lighting for three lamps can be operated by:

- all lamps in ‘QN’ and ‘OFF’ position by two-way switches and two intermediate switches.

- Independent light in ‘ON’ and ‘OFF’ position by two intermediate switches.

- all lights in ‘ON’ and ‘OFF’ position by two intermediate switches.

- both A and B.

- According to NE code, the colour code for the neutral is:

- Red

- blue

- black

- green

- To estimate the materials and cables, the diagram which is very useful will be :

- laying diagram and wiring diagram.

- installation plan and circuit diagram.

- circuit diagram and installation plan.

- wiring diagram and circuit diagram.

- This diagram is not possible to be drawn for the entire Wiring installation. But it is drawn to get the approval of the customer to have overview.

- Layout diagram

- Installation plan

- Circuit diagram

- Wiring diagram

- Which type of, looping method uses less cables in wiring?

- looping out from switch and ceiling rose

- looping out from switch

- looping out from 3 plate ceiling rose

- looping out with junction box

- While estimating in wiring for any contingencies generally an additional amount of total cost is added..........

- 3 percent

- 5 percent

- 7 percent

- 10 percent

- The current capacities of the cables are specified for certain ambient temperature in BIS regulations. The specified ambient temperature is..........

- 20°C

- 30°C

- 40°C

- 50°C

- As compared to rewirable fuses, HRS fuses have.........

- high rupturing capacity

- high speed of operation

- no ageing effect

- low rupturing effect

- The outer screwed contacts of Edison screw type lamp holder must always be connected to the..........

- neutral wire of the circuit

- phase wire of the circuit .

- switch (half) wire of the circuit

- earth wire of the circuit

- As per standard practice the circuit of different phases should be kept at........

- 3.5 metres

- 3.05 metres

- 2.55 metres

- 2.05 metres

- Sharpness of knife used for skinning cables is determined from the light reflected off the blade edge. Four conditions are shown in the figure, which determines the correct sharpness?

- To reduce the length of cable in a wiring installation having same number of points and wired in loop in terminal system, one of the following wiring accessories is required.

Identify the accessory

- Two way switch

- intermediate switch

- Two plate ceiling rose

- Three plate ceiling rose

- While harnessing cables, it is  recommended to use separate bundles for power and control wiring cables. The reason to separate the bundles is..........

- to avoid transfer of heat from power cables to control cables

- to avoid the mixing at both cables

- to make good, appearance of wiring

- to avoid the wrong connection of both cables

- The under writer’s knot is to be made on twin-twisted or twin-core flexible cable inside the accessories like pendent holders.

What is the main reason?

- To avoid loose connection of the terminals

- To increase strength more mechanical

- To relieve the strain from the terminals of the accessory

- To prevent the excess wire outside the cap cover

- To determine the size of cable the following formula is given below as: Cross sectional area of the wire

= 2 ×  K ×L × IVoltage drop

What does the letter ‘K’ represent in the above formula?

- K = Specific resistance of the conductor in ohm per cubic centimetre/m

- K = insulation resistance of the cable in Mega  ohm

- K: Resistance per sq mm per metre length of the conductor

- K = Cross sectional area of the conductor in sq mm

- To determine the size of cable, the following formula given below as: Cross sectional area of the wire

= 2 ×  K ×L × IVoltage drop

What does the letter 1’ represent in the

above formula?

- L = Length of the cable in metres

- L = Distance between the load centre from the supply ends in metres

- L = Distance between the load and earthing point in metres

- L = Length of the cable per each load point in metres

- As per NEC, the alpha-numeric notation for AC. 3~phase supply (input) system is represented as..........

- R,Y,B,N

- U,V,W,N

- L1, L2, L3, N

- A, B, C, N

- As per NEC, the alpha-numeric notation for DC. system (input) is represented as

- P1, N

- L+, L-

- +, -

- +ve, -ve

- Polarity test ensures safety to the user and also to the electrician. Verification of polarity in an installation is to ensure that........

- the fuses are connected in the phase wire and the single pole switches are connected in the neutral wire

- the fuses are connected in the neutral, wire and the single pole switches are connected to the, phase wire

- the fuses and the single pole switches are connected in the phase wire

- the fuses and the single pole switches  are connected in the neutral wire

- Which statement describes a characteristics features of routine preventive maintenance?

- Maintenance schedule. needs to be decided, based on the maintenance reports entered in the maintenance card

- Maintenance could be done either during the Working of the machine or during short interval down periods.

- Maintenance is performed at irregular frequencies.

- Maintenance is only performed, if a machine has a fault or defect.

- On wiring installation, as per BlS standard the minimum value of Insulation Resistance (RI) expressed as..........

- Ri = 5000No.of point + Outlets

- Ri = 500No.of point + Outlets

- Ri = 250No.of point

- Ri = 50No.of point + Outlets

- When testing the insulation resistance of new domestic wiring installation, the megger reading should be 1 MὩ, greater than..........

- 50No.of outlets ohms

- 25No.of outlets ohms

- 50No.of outlets mega ohms

- 100No.of outlets  mega ohms

- During continuity test in an installation, the megger indicated reading is..........

- megohm

- megohm

- 500 megohm

- infinity megohm

- How many light points can be allowed in one circuit?

- 5 points

- 10 points

- 15 points

- 20 points

- How many power points can be allowed in one circuit?

- Four

- Three

- Two

- One

- The load on each sub-circuit shall be restricted to.........

- 800 W

- 750 W

- 700 W

- 650 W

- The rating factor of the circuit having coarse excess current protection is.........

- 1.63

- 1.23

- 0.81

- 0.61

- The rating factor of the circuit having close excess current protection is........

- 1.81

- 1.23

- 0.81

- 0.61

- Which device comes under the close excess current protection?

- Rewirable fuse

- Cut out fuse

- Miniature Circuit Breaker (MCB)

- ICDP main switch

- Normal current capacity of a certain cable is 16 amperes. if the circuit is protected by the close excess current protection, then the current capacity of the cable will be .......... _

- 12 amperes

- 13 amperes

- 16 amperes

- 20 amperes

- As per |.E. Rules, the leakage current in wiring installation should not exceed more than........

- 150th part of full load current

- 1500th part of full load current

- 15000th part of full load current

- 150000th part of full load current

- As per ISl code regulation, the load in each branch lighting circuit should not be more than

- 600 watts

- 800 watts

- 1000 watts

- 3000 watts

- The minimum thickness of non hinged boards should be........

- 20 mm

- 25 mm

- 30 mm

- 40 mm

- The distance between clips in horizontal runs shall not exceed.........

- 20 cm

- 15 cm

- 10 cm

- 5 cm

- What is the minimum distance to be kept between link clips in the case of vertical runs?

- 20 cm

- 18 cm

- 15 cm

- 10 cm

- How many number of cables required for connecting the two way switches shown in the layout diagram?

- 2

- 3

- 4

- 5

- Which type of wiring is suitable for multi-storey building?

- Tree-system

- Ring 'main system

- Distribution board system

- Ring main and distribution board system

- Tree system of wiring is suitable for.........

- single storied building

- multi storied building

- industrial wiring installation

- printing press

- Name the type of cables used for cleat wiring?

- Weather proof cables

- Vulcanized Indian Rubber (VlR) cables

- Lead sheathed cables

- Tough Rubber Sheathed (TRS) cables

- Cleats shall be fixed at distance not greater than ......... apart at regular intervals

- 65 cm

- 60 cm

- 55 cm

- 50 cm

- Which type of system is suitable for petroleum product factory?

- Batten wiring

- PVC conduit wiring

- PVC casing wiring

- Metal conduit wiring

- Cables are drawn through the pipe and terminated. The system of wiring is called

- PVC casing capping wiring

- conduit wiring

- T.W. batten wiring

- T.W. casing capping wiring

- The radius of bends used in conduit wiring should not be less than.........

- 3 cm

- 5 cm

- 6 cm

- 7.5 cm

- The surface conduit wiring saddles should be fixed at intervals of not more than........

- 0.5 m

- 0.75 m

- 10 m

- 15 m

- The thread on conduit pipes in all cases should have a length of range..........

- 10 to 20 mm

- 11 to 27 mm

- 12 to 25 mm

- 15 to 35 mm

- Which one of the following is the suitable pitch of hacksaw blade for cutting a heavy gauge conduit pipe?

- 1.8 mm

- 1.4 mm

- 1.0 mm

- 0.8 mm

- What is the wall thickness of 25 mm heavy gauge metal conduit pipe?

- 2.24 mm

- 2.00 mm

- 1.80 mm

- 1.60 mm

- Which one of the following is the most suitable fish wire to be used in conduit runs having more than a couple of bends?

- Copper wire

- Mild steel wire

- Stainless Steel wire

- Steel curtain springs

- The best suitable fish wire to be used in conduit runs is..........

- GI wire

- PVC wire

- VIR wire

- Aluminium wire

- What is the main disadvantage of the PVC casing and capping wiring system?

- Cost is more than other wiring system

- it is inflammable and risk or fire

- it is difficult to locate the fault

- it requires frequent maintenance

- The length of the PVC channel for wiring is available.......

- 2 metre

- 2.5 metre

- 3.0 metre

- 4.0 metre

- The size of PVC pipe for cold bending should not exceed..........

- 10 mm

- 15 mm

- 20 mm

- 25 mm

- The only Wiring which can be taken through shortest route is the roof and required less right angle bends is........

- conduit surface wiring

- conduit concealed wiring

- casing and capping wiring

- batten wiring

- Laying of concealed wiring in root starts, soon after completion of.........

- pouring concrete

- centering

- shuttering

- load bearing walls

- Concealed PVC conduit wiring requires

- lnspection elbows, bends and Tee’s

- Solid elbows, bends and Tee’s

- Conduit saddles

- Metal coupling with check nut

- Which one of the following is the normal wall thickness of PVC conduit pipes?

- 0.8 mm

- 1.0 mm

- 1.2 mm

- 1.6 mm

- In which one of the following wirings, three two way switches only are required to operate three lamps?

- Staircase wiring

- Corridor wiring

- Tunnel wiring

- Godown wiring

- In which type of wiring, two lamps are switched ON at a time?

- Tunnel wiring

- Hostel wiring

- Godown wiring

- Corridor wiring

- A staircase point can be operated…….

- with one one-way and one two-way switches

- with two one-way switches

- with two two-way switches

- with two intermediate switches

- Batten wiring is recommended for…….

- temporary house wiring

- permanent house wiring

- industrial wiring

- power wiring

- While fixing batten, the distance between the fixing screws should not exceed……..

- 75 cm

- 7.5 cm

- 75 mm

- 7.5 mm

- T.R.S. cables on batten is suitable for……..

- high voltage

- extra high voltage

- low voltage

- medium voltage

- Identity the BlS symbol which represents  the wiring in conduit concealed?

- Identify the type of diagram/plan from the given diagram.

- Installation plan

- Circuit diagram

- Wiring diagram

- Layout diagram

- What is the name of the diagram, shown in the figure?

- Layout diagram

- Circuit diagram

- Installation diagram

- Wiring diagram

- Name the type of wiring circuit diagram as shown in the figure.

- Corridor wiring

- Bank wiring

- Tunnel wiring

- Godown wiring

- 88. Name the wiring circuit diagram shown in the figure.

- Corridor wiring

- Bank wiring

- Tunnel wiring

- Godown wiring

20. Measuring Instruments

- In a moving coil instrument, the damping torque is developed by :

- air-friction

- eddy-current

- gravity-friction

- fluid-friction

- The force acting in a MC instrument is equal to

- B./.v.

- B.I.v.  sin q

- B.l.i.

- B.l.i. sin

- MC. type instruments are used on :

- A.C.

- D.C.

- A.C. or D.C.

- none of these.

- M.l. type instruments are used on

- A.C. only

- D.C. only

- AC. or DC

- none of these.

- The force which produces movement of the pointer of an instrument is called :

- deflecting force

- controlling force

- damping force

- distracting force.

- What is the purpose of providing a mirror behind the pointer in a measuring instrument?

- The scale is illuminated through the mirror

- To observe that the pointer is bend or not

- Being semi-transparent, a mirror helps in the study of construction of the instrument

- To make the reading free from an error by  removing parallax) between the pointer and its image inner

- When using any instrument for measurement or testing purpose, first attention should be given to the :

- theoretical knowledge

- personal safety

- surroundings

- dress

- Which one of the following statements is true

about the instrument in the given figure?

- it is a high frequency measuring instrument

- it is designed to work in horizontal position only

- It is designed to work in vertical position only

- its current coil can be loaded up to 5 A

- Which one of the following statements is true about the internal resistance of an ammeter and a voltmeter?

- The internal resistance of an ammeter should be very low and that of a voltmeter should be very high

- The internal resistance of an ammeter should be very high and that of a voltmeter should be very low

- The internal resistance of an ammeter and a voltmeter should be very low

- The internal resistance of an ' ammeter and a voltmeter should be very high

- The reliability of measuring instrument means :

- the life of the instrument

- the extent to which its characteristics remain linear

- the degree to which repeatability continues to remain within specific limits

- all of these

- The sensitivity of a measuring instrument means

- the maximum value of a physical quantity that can be measured

- the highest reading of the scale

- the minimum reading of the scale

- the smallest amount of a physical quantity that can be detected by the instrument

- in order to increase the range of a voltmeter :

- a low resistance is Connected in series

- a low resistance is connected in parallel

- a high resistance is connected in series

- a high resistance is connected in parallel

- To reduce the effect of a voltmeter on the circuit under test, we should:

- decrease the multiplier resistance

- use an instrument with a high internal resistance

- use a shunt resistor

- use an ac. meter with a rectifier.

- If the connections of a M.l. instrument used in a circuit are interchanged then:

- it will not give a reading

- its reading will not change

- it will give a deflection in an opposite direction

- it will burn out

- A M.l. type instrument has 2

- a linear scale

- a non-linear scale

- an exponential scale

- a logarithmic scale.

- A MC. type instrument has :

- a linear scale

- a non-linear scale

- an exponential scale

- a logarithmic scale

- it two ammeters X and Y require 40 mA and 50 mA current respectively to give a full-Scale deflection, then:

- X is more sensitive

- Y is more sensitive

- both are sensitive

- sensitivity cannot be judged with the given information.

- The value of a shunt resistance in an ammeter is usually:

- equal to the meter resistance

- greater than the meter resistance

- less or than the meter resistance

- of any value.

- If the approximate value of the current to be measured is not known, measurement should be started on :

- the lowest range

- the middle range

- the highest range

- any range.

- The sensitivity of a voltmeter is expressed as :

- volts/ohm

- ohm. volt

- 1/ohm. volt

- ohms/volt

- The disadvantage associated with an electrodynamics instrument is this that :

- it consumes more electric power

- it has low torque to weight ratio

- its frequency range is low

- all of these.

- An electrodynamics meter can be used to measure: ‘

- ac. voltage

- dc. voltage .

- do  as well as ac. voltage

- dc voltage but for ac. Voltage rectification is necessary.

- Megger is an instrument meant for measuring :

- very low resistance

- insulation resistance

- inductance of a coil

- ‘Q’ of a coil.

- The operation of a Megger is based on :

- moving-iron meter

- electrodynamics meter

- electrostatic meter

- moving coil meter.

- A multimeter consists of :

- voltmeter and ohmmeter

- voltmeter and ammeter

- voltmeter, ammeter and ohmmeter

- none of these.

- A VTVM is more reliable than a multimeter for measuring voltage across a low. impedance because :

- its sensitivity is very high

- it offers a high impedance

- it does not alter the voltage measured

- all of these.

- induction type single-phase energy meter is  :

- an ampere-hour meter

- a true watt-hour meter

- 3 watt meter.

- none of these.

- A wattmeter can measure

- ac. power only

- dc power only

- ac. as well as d.c. power

- dc power and ac. power after rectification.

- A multimeter cannot measure :

- current

- potential difference

- capacitance

- resistance.

- Which of the following electric quantities can be measured by a moving coil instrument?

- DC current and DC. voltage

- DC. and AC, current

- AC. voltage

- A.C. current

- The scale of a power factor meter used in ac. circuits is shown; A reading can be noted from the position of pointer on the scale. Which one calibration of the scale is correct?

- When the electric supply is ON then which of the following instruments should not be used?

- Wattmeter and power factor meter

- Frequency meter and tong tester

- Phase sequence meter and energy meter

- Ohm meter and megger.

- Which of the electrical instruments have to be connected in the same manner for measurement?

- Frequency meter and P.F. meter

- Voltmeter and frequency meter

- Voltmeter and power factor meter

- Wattmeter and voltmeter

- Voltmeters of high internal resistance should be used for voltage measurements, in circuits of high resistance.

The reason for this is to :

- avoid overloading the resistor when measuring its voltage.

- avoid inaccuracy ' when the temperature changes

- keep the measurement error small due to the internal consumption (voltage drop) of the meter.

- prevent the meter from overloading.

- Tong tester is used to measure the current in ac. circuits without interrupting the circuit. For measurement, the split core is clamped on to the insulated current carrying conductor. Looping of conductor passing through the split core is done sometimes.

What will be the reading for a selected range if the conductor is looped (double pass)?

- Double the current which passes through the conductor

- 4 times (i.e., 22) the current which passes through the conductor

- Half the current which passes through the conductor

- Same as the conductor current.

- Power drawn by a welding shop which has phase 415 V, 50 Hz supply has to be measured.

The following loads are connected:

- 2 welding sets each of 5 WA across L1, L2 and L3.

- 1 set  of 5 kVA across L2 and L3. What is the type of wattmeter you would choose for this measurement?

- 3 element, 3 wire type, 3-phae wattmeter

- One single phase wattmeter connected between any two phases and multiply reading by 3

- 3 element, 4 wire type 3-phase wattmeter

- 2 element type 3-phase wattmeter.

- One screw is provided in M.C. instrument which is attached to the pointer. What for it is provided?

- To adjust the sensitivity of the meter

- To correct the zero position of the pointer

- To tight the moving part of the meter

- To change the measuring range of the meter

- A voltmeter with a sensitivity of 1 kilo ohm per volt will have a resistance of ........... ohms at 50 V range.

- 1000

- 5000

- 50,000

- 50

- Electrical instruments may be classified based on ............ categories.

- 2

- 3

- 4

- 5

- Which instrument is used for measuring voltage?

- Ammeter

- Voltmeter

- Ohm meter

- Multimeter

- Which instrument is used for measuring the insulation resistance?

- Ohm meter

- Voltmeter

- Ammeter

- Megger

- The force required for moving the pointer in an indicating instruments is.................

- deflecting force

- controlling force.

- eddy current damping force

- air friction damping force.

- Which measuring instrument is an indicating instrument?

- Read type frequency meter

- Energy meter

- Ammeter

- Temperature recording instrument

- Which type of control force is an indicating instrument, which can be kept in any position?

- Gravity control

- Spring control

- Fluid friction control

- Hydraulic friction control

- The meter installed at your house to measure electrical energy is an example of.................

- indicating type instrument

- recording type instrument

- integrating type instrument

- indicating and recording type instrument

- Which is the absolute instrument?

- Ammeter

- Wattmeter

- Energy meter

- Tangent galvanometer

- Watt hour meter is classified as a

- deflecting instrument

- indicating instrument

- recording instrument

- integrating instrument

- Phosphor bronze alloy is used for springs in indicating instruments because the alloy has…………

- high temperature co-efficient of expansion

- low specific resistance

- low resistance to fatigue

- good magnetic property

- When an instrument is using gravity control, it can read correctly...............

- if it is used in horizontal position only

- if it is used in vertical position only

- if it is used in an inclined position only

- if it is used in an inclined and horizontal position

- Which of the following material is preferred for permanent magnet used in the instrument?

- Wrought iron

- Silicon steel

- Y-alloy

- Alnico

- The precision of measurement depends on the range of the instrument selected. What is the general rule for selection of range of instrument? Measured value should be.....

- above the mid-scale

- below the mid-scale

- on the mid-scale

- exactly on full scale

- As a general rule before connecting the meters to the circuit, we should see whether pointer is at-zero or not. This rule is not applicable incase of..............

- multimeter in voltage range

- wattmeter

- milli ammeter

- megger

- A megger is exclusively designed for measuring:

- very high resistance

- very low resistance

- ground faults in power lines

- over leads on DC motors.

- Which instrument is used for measuring the medium resistances directly?

- Ammeter

- Megger

- Ohm meter

- Voltmeter

- A current of approximately 5 A and 10 A has to be measured in a circuit. Which one of the following ammeters will you use to have a more accurate reading for both the values?

- 0-10 A

- 0-15 A

- 0-20 A

- 0-25 A

- Ammeters used for current measurements in circuits with short overload time, differs from other application. Ammeters used in short overload time circuit have.......

- non-linear scale

- extended scale

- linear scale

- fine scale

- Moving coil instrument works on the effect of………

- chemical effect

- electromagnetic effect

- heating effect

- electrostatic effect

- Which of the following electrical quantities can you measure with a moving coil instrument?

- DC current and D.C. voltage

- D.C. and A.C. Current

- A.C. voltage

- A.C. current

- Which part of the PMMC instrument produces eddy current damping?

- Moving coil

- Aluminium former

- Permanent magnet

- Soft iron cylindrical core.

- Which material is used for shunt and multiplier to extend the range of moving coil meters?

- Phosphor bronze

- Silver

- Manganin

- Nichrome

- Moving coil instruments has “+ve and - ve terminals. What is the color of “+ve terminal in moving coil instrument?

- Black

- Blue

- Red

- Yellow

- Shunts and Multipliers used in moving coil instruments are made up of manganin. The reason for using the above metal is that it has................

- Positive temperature coefficient of resistance

- Negative temperature coefficient of resistance

- Perfectly constant temperature coefficient of resistance

- Very low (practically constant) temperature Coefficient of resistance

- Normally the scale of moving iron instruments will be........

- initially crowded, gradually widen scale

- initially widen, gradually crowded scale

- have a uniform graduated scale

- have a un-uniform graded scale

- Moving iron instruments are used to measure both A.C. and D.C. quantities. What will happen if the terminals of a moving iron instrument are interchanged in D.C. circuit?

- It will give no deflection

- it will indicate same reading

- It will give full scale deflection

- It will deflect in the reverse direction,

- Instruments which must not be used in the circuit, when the power is ON are………..

- wattmeter and power factor meter

- frequency meter and tong tester

- phase sequence meter and energy meter

- ohm meter and megger

- A house has 4 kW connected loads and is fed by single phase supply. What range energy meter is recommended for the house?

- 20 amperes

- 15 amperes

- 30 amperes

- 10 amperes

- In an induction type energy meter, the 1 speed of rotation of the aluminium disc is proportional to the...............

- current flowing through the load

- square of the current in the current coil

- the voltage applied to the pressure coil

- the current in the current coil and voltage in the pressure coil.

- A 3 phase 3 element energy meter of 415 V, 50Hz has. its pressure coil rated for

- 230 volts

- 240 volts

- 415 volts

- 440 volts

- You are testing an energy meter of 240 volts, 50Hz 5 amperes, 750 revolutions per kWh for its performance with some lead. When the disc rotates 100 revolutions in 10 minutes, what is the energy supplied?

- 13.33 kWh

- 1.33 kWh

- 0138 kWh

- 0.0133 kWh

- The percentage error of an energy meter is...........

- Energy resistered-True energy True energy  X 100

- True energy - Energy resistered Energy resistered X 100

- Energy resistered-True energy Energy resistered  X 100

- True energy - Energy resistered True resistered X 100

- Ammeters used for current measurements in circuits with short overload time, differs from other application, Ammeters used in short overload time circuit have............

- non linear scale

- extended scale

- linear scale

- fine scale

- You have to connect an ammeter to monitor the current drawn by a 3-phase, 5 H.P, 415 V, 50 Hz squirrel cage induction motor, connected through a DOL starter. The range of ammeter, suitable for connecting in motor circuit is………

- 0 to 30 amperes(extended scale)

- 0 to,15 amperes (linear scale)

- 0 to 10 amperes (non linear scale)

- 0 to 5 amperes (extended scale)

- Which type of wattmeter can be used on AC. only?

- Dynamometer type wattmeter

- Electrostatic type wattmeter

- Induction type wattmeter

- Commutator type wattmeter

- In a dynamometer type wattmeter, the deflecting torque is proportional to

- the square of the current in fixed coil

- the current in the moving coil

- the current in the fixed coil and in the moving coil

- the current in the fixed coil

- What type of quantity is measured in Farranti type mercury meter?

- DC. watt hour

- D.C. ampere hour

- AC. and DC watt hour

- A.C. watt hour

- The reading (in a 3‘ digit milli voltmeter started flashing continuously when connected across a resistor to measure the MV drop across it. The reason for flashing ct display on milli voltmeter is that……….

- the battery in the; instrument is-weak

- there is no battery in the instrument

- the measured value is over range

- the display is damaged

- Which one of the following measurement test cannot be done with a multimeter?

- current drawn by ‘an electronic circuit

- supply voltage to DC. motor

- continuity test of motor winding

- insulation resistance of motor

- in battery powered digital multimeter, LCD display is most commonly used. The reason for this is.............

- mains power is not available everywhere

- it requires only a small amount of power

- LCD can be read even in dark

- LCD has a bright display.

- The scale of a multimeter is set at the range of 60 V. The percentage of the indicated error is highest in the range of……….

- 0 V to 10 V

- 20 V to 40 V

- 40 V to 50 V

- 50 V to 60 V

- What are the measurements that can be  done from a portable digital multimeter without battery?

- Both voltage and current measurements

- Except resistance, all other measurements

- Only current measurement

- No measurement is possible.

- What are the measurements for which multimeter is suitable-without battery?

- Voltage, current and resistance

- Voltage and current

- Only for resistance

- Resistance and voltage

- On the scale of a power factor meter the pointer indicates 0.8 leading power factor. This value means that the load is a…………

- pure capacitance

- high inductive and low capacitive resistance

- high resistance and low inductive reactance

- low inductive reactance and high) capacitive reactance

- How the PF meter is connected in the motor circuit?

- fixed coil in parallel with the supply and moving the coil in series with single phase motor

- fixed coil across the motor and moving coil across the supply

- Fixed coil in series with motor and moving coil across the supply

- Both fixed coil and moving coil in series with the motor

- The circuit for measuring resistance with a multimeter is shown in the figure, To get the correct indication en the dial an adjustment of the potentiometer Rp must be made before each range of measurement.

The correct method of adjustment is, with the measuring terminals A and B.........

- short circuited, the pointer should be set at as on the scale

- open, the pointer should be set at ∞ on the scale

- short circuited, the pointer should be: set at the middle of the scale

- Short circuited, the pointer should be set a 0 on the scale.

- What precautions you have to take when disconnecting voltmeter from potential transformers?

- Potential transformers secondary terminals which are open should be insulated

- Ground the secondary terminals on pen circuit

- The terminals of the potential transformer secondary have to be short circuited.

- After disconnecting, connect a low resistance shunt between the terminal of the secondary

- What you have to do when disconnecting the ammeter from the terminals of the CT?

- Earthing the secondary of CT.

- Short-circuiting the terminals of the CT before disconnecting the ammeter.

- Switching a high resistance shunt between terminals of CT after disconnecting the ammeter.

- Disconnecting primary from supply.

- With the help of multimeter, we can measure.........

- voltage, current and resistance

- power, energy and power factor

- insulation resistance and AG. current only

- ampere hour capacity and DC. current only,

- Before using an ohm meter (or a multimeter) for measuring the resistance —

- adjust its pointer to infinity mechanically

- choose appropriate resistance-range

- set the pointer to zero by short circuiting its probes by means of a potentiometer

- do all the above

21. D.C. Generators

- If the axis of a moving coil is kept parallel to the flux then the e.m.f induced in the coil will be :

- minimum

- maximum

- nil

- one volt

- By rotating a conductor in a magnetic field, the induced e. m. f. will be:

- static

- dynamic

- induced e.m.f.

- none of the above

- For inducing an e.m.f. of1 volt in 1 second time, a conductor will have to cut :

- 10 maxwell lines

- 108 Iines

- 1 weber magnetic flux

- none of the above

- On doubling the supply frequency, the eddy current will:

- become half of their former value

- become four times of their former value

- remain unaffected

- become eight time of their former value

- in order to reduce hysteresis losses, the cores of electrical machines are generally made of:

- tungsten steel

- cast iron

- high carbon, steel

- silicon steel

- Which of the following statements is incorrect:

Whenever the flux linking through a coil or a circuit changes, e. m..f is produced

- The direction of dynamically induced e.m.f. can be determined by applying Fleming’s

- right-hand rule

- Coefficient of coupling for tightly coupled coils is zero

- The coefficient of self inductance is proportional to the square of the number bf turns on it

- Whenever a conductor cuts magnetic flux, an e.m.f. is induced in the same. The statement refers to :

- Joule’s law

- Coulomb’s law

- Faraday’s ‘law

- Lenz’s law

- The given figure shows a coil with a closed iron core. What do the dotted lines represent?

- magnetic potential

- magneto motive force

- magnetic field strength

- magnetic flux

- The magnetic flux shown in above Fig. can be increased by:

- increasing the magneto motive force

- decreasing the magneto motive force

- reversing the direction of current

- making an air-gap in the core

- The law which states that the induced e.m.f. and current always opposes the cause producing it was presented by :

- Lenz

- Faraday

- Maxwell

- Ohm

- Degaussing is the process of:

- removing gases from a material

- removing impurities from magnetic materials

- demagnetising a metallic part

- demagnetising a magnet

- An open-circuit curve is drawn between :

- E and IF

- N and IL

- E and IA

- E and IF

- In a level compound generator, full load voltage is found to be _______ the terminal voltage.

- lesser than

- equal to than

- higher than

- much lesser

- In which type of do machine-poles is residual magnetism necessary?

- Shunt generator

- Motor

- Separately excited generator

- Permanent magnet type generator

- Armature cores are commonly made of :

- iron

- silicon steel

- high carbon steel

- stainless steel

- Lap winding has

- two

- four

- equal to the no. of poles

- one

- The output voltage of a single loop generator is a……….

- steady a.c.

- steady as.

- pulsating a.c.

- pulsating d.c.

- The voltage generated by a generator is determined by the equation :

- E = V – Ia . Ra

- E = V + Ia . Ra

- E = -L didt

- E = -N dϕdt

- What effect on the current flow takes place on reversing the direction of movement of a conductor in the magnetic field of a generator?

- no effect takes place

- current flow is stopped

- the direction of current flow is reversed

- the magnitude of current flow is decreased

- In most generators the output voltage is induced by :

- rotating magnetic field past stationary coils

- relative motion between the field and the armature coils

- converting electrical energy into mechanical energy

- rotating the coils around a magnetic field

- The shunt field of a compound generator is connected across both the series field and the armature. This connection is known as :

- long shunt

- short shunt

- differential compound

- cumulative compound

- For all symbols having their usual meaning, the amt generated in a lap. wound generator is given by :

- E = ϕ.Z.N60(P2)

- E = ϕ.Z.N.A2P

- E = ϕ.Z.N60(PA)

- E = ϕ.Z.N60(AP)

- The brushes are always placed along :

- MNA (magnetic neutral axis)

- GNA (geometric neutral axis)

- perpendicular to MNA

- perpendicular to GNA

- The function of compensating winding is to neutralise :

- reactance voltage generated as a consequence of commutation

- cross-magnetising the effect of armature reaction

- demagnetising effect of armature reaction

- all of above

- In any dc generator, the e.m.f. generated in the armature is maximum when :

- rate of change of flux is minimum

- rate of change of flux is maximum

- flux linked with conductors is maximum

- flux linked with conductors is minimum

- The method which can be used to improve commutation process is :

- to increase the resistance of brushes

- to reduce the no. of turns in the armature coil so as to reduce the no. of commutator segments

- to neutralise the reactance voltage by producing a reversing e.m.f. in the short-circuited coil under commutation

- all the above

- The function of inter poles is :

- to neutralise the reactance voltage to help commutation process

- to neutralise the cross-magnetising effect of armature reaction

- to reduce sparking at the commutator

- all the above

- Generators are run in parallel for achieving the advantage ‘of :

- high reliability

- high efficiency ,

- meeting higher load demands

- all the above

- Shunt generators are used where the main requirement is a constant :

- voltage and current

- current

- voltage over a wide load range

- voltage over a narrow load range

- Inter poles are normally connected in :

- parallel with the field windings

- series with the field windings

- parallel with the load

- series with the load

- In d.c. generator, which loss varies with the load ?

- copper loss

- eddy current less

- hysteresis loss

- windage loss

- The armature of dc. generator is laminated to :

- reduce the overall mass

- insulate core

- reduce-the eddy current loss

- all the above

- in dc generators, the cause of rapid, brush wear may be :

- imperfect contact

- severe sparking

- rough commutator Surface

- any one of the above

- In dc. generators, lap winding is used for:

- low voltage, low current

- high voltage, high current

- low voltage, high current

- high voltage, low current

- D. C. generator preferred for charging automobiles batteries is

- shunt generator

- series generator

- long shunt compound, generator

- short shunt compound generator

- The overall efficiency of a dc generator lies in the range of :

- 60-80%

- 75-85%

- 80-90%

- 85-95%

- The function of an equalizer ring is :

- to avoid unequal distribution of current at the brushes

- to help in reducing sparking

- both the above

- none of the above

- The carbon brushes of D.C machines become shorter by Wearing out during the running time of the machine. At which length do you have to change the brushes?

- 12 of the original length

- 13 of the original length

- 34 of the original length

- 23 of the original length

- A conductor is rotating within a magnetic field as shown in the given figure. At which of the positions shown does the peak voltage occur?

a) At position 1 and 1ϕ

b) At position 2 and 2ϕ

0) At position 3 and 3ϕ

d) At position 4 and 4ϕ

- A 2 kW, 200 V, D.C. generator will supply a load current of :

- 2 amps

- 4 amps

- 10 amps

- 20 amps

- The essential condition for stable parallel operating of two 0.0 generators having similar characteristics is that, they should have

- same kilowatt output ratings

- drooping voltage characteristics

- same percentage regulation

- same no load and full load speed

- D.C. generators are connected to or disconnected from the busbar only under the floating condition. This is done to avoid

- burning of switch contact

- sudden loading of the prime mover

- mechanical jerk to the shaft

- all of the above.

- For parallel operation the DC. generators normally preferred are

- Shunt and over compound

- Series

- Shunt and under compound

- Over compound

- Equalizer bar is necessary for parallel operation of DC.

- series and rover compound generator

- under compound generator

- shunt and series generator

- over compound generator

- D.C. shunt generators are most suited for stable parallel operation because of their voltage characteristics –

- identical

- dropping

- linear

- rising

- Two parallel shunt generators will divide the total load equally in proportion to their kilo Watt output ratings only when they have the same................

- rated voltage

- voltage regulation

- internal Ia, Ra drops

- both A and B

- The simplest way to 3shift toad from one D.C. shunt generator running in parallel with another is to…………

- adjust their field rheostats

- insert resistance in circuits their armature

- adjust speeds of their prime movers

- use equalizer connections

- In case of parallel operation of D.C. compound wound generators, for proper division of load, from no load to full load, it is essential that they should have the same

- kW output rating

- percentage regulation

- series field resistance

- speed of operation

- In parallel operation of two D.C. compound generators provided with interpolest the equalizing bar must be connected to

- inter pole fields only‘

- series fields only

- junction of series and inter pole fields

- none of the above

- The main function of an equalizer bar is to make the parallel operation of two over compounded D.C. generators.............

- stable

- possible

- regular

- smooth

- The main factor that leads to unstable parallel operation, of flat or over compounded D.C generator is........

- their rising voltage characteristics

- unequal series field resistances

- unequal Speed regulation of their prime movers

- their drooping-voltage characteristics

- In the Fleming’s left hand rule the thumb represents…………

- the direction of motion of the conductor

- the direction of flux

- the direction of current flow in the conductor

- the voltage in the conductor

- By which law, the direction of the induced e.m.f. will be identified?

- Faraday’s first law of electromagnetic induction

- Faraday’s second law of electromagnetic induction

- Lenz’s law

- Fleming’s left hand rule

- Fleming’s Flight Hand Rule is to identify the.............

- direction of flux

- direction of rotation of a generator

- direction of current in a motor

- direction of induced emf

- A DC generator works on the principle of……...

- Faraday’s law of electrolysis

- Fleming’s left hand rule

- Lenz’ 5 law

- Faradays’ law of electromagnetic induction

- The formula to find EMF induced (E) in DC generator is.............

- E = ϕZN60×PA  volts

- E = ϕZN60×AP  volts

- E = ϕZP60 volts

- E = ϕZN120  volts

- A conductor is rotating within a magnetic field as shown in figure. At which of the position shown does the peak voltage occur?

- At position B and F

- At position C and G

- At position D and H

- At position A and E

- A 6 pole DC generator has wave wound armature. If the armature is rewound for lap, the induced EMF will be...........

- increased by 6 times

- decreased by 6 times

- increased by 3 times

- decreased by 3 times

- The insulating materials used between the commutator segments of a DC generator will be...............

- paper

- wood

- fibre

- mica

- Inter poles are provided in DC generator, for.............

- increasing the emf of the generator

- increasing the counter emf

- reducing the strength of the main field flux

- reducing the sparking at the commutator segment

- in a DC generator, sparking at the brush position may be :

- due to over Commutation

- due to under commutation

- Both (a) and (b)

- Neither (a) nor (b)

- In a DC Generator, if the field circuit resistance is more than the field critical resistance, then the..............

- generator will build up maximum voltage

- generator will generate maximum voltage

- generator will not build up the voltage

- field Coils will burn

- Brush tension in a DC generator plays very important role in its performance. Excessive brush tension may……..

  - slow down the speed of the machine

  - decrease the output voltage

  - increase the output voltage

  - overheat the commutator and wear out the surface

- The purpose of lamination between core is used in armature is to..........

- increase the induced e.m.f.

- reduce the eddy current loss

- reduce the copper loss

- reduce the armature reaction effect

- The load characteristic of separated excited generator is between..............

- armature current and emf

- armature current and terminal voltage

- load current and terminal voltage

- field current and terminal voltage

- The purpose of the flexible copper pig tail mounted on the brush of the D C. generator is.........

- to collect current. commulator from the

- to facilitate collection of current from the armature conductors

- to convey, the current from the brushes to the holder

- to prevent the brush from flying out under the centrifugal force.

- By providing compensating winding in series with armature of a DC. generator which defect can be prevented?

- Rough commutation

- Cross magnetizing effect

- Demagnetizing effect

- Armature resistance drop effect

- As per B.l.S., the series field winding terminals must be marked as..........

- B1:B2

- E1:E2

- D1:D2

- F1:F2

- Which among the following is the function of the inter-poles in a DC generator?

- Neutralisation of statically induced voltage.

- To make sparkless commutator

- To change rough to smooth commutation

- All of the above

- Which type of DC generator is used for welding generator set?

- D.C. shunt generator

- D.C. series generator

- D.C. Differential compound generator

- D.C. cumulative generator

- Which type of DC generator is used for welding generator set?

- DC series generator

- DC shunt generator

- DC differential compound generator

- DC commutative compound generator

- How many parallel paths in simplex wave winding armature of the DC. generator, which is having 8 poles?

  - 2

  - 4

  - 6

  - 8

- A pulley of DC generator is found to have completed 2000 revolutions within 1.25 minutes. The speed of the DC generator in r.p.m. will be :

- 1200 r.p.m

- 1400 r.p.m

- 1600 r.p.m

- 2000 r.p.m

- The 10 pole D.C generator has 960 armature conductors and a flux per pole of 20 mwb running at 600 rpm. Calculate the amt generated when the armature is connected in simplex wave winding?

- 960 volts

- 720 volts

- 640 volts

- 360 volts

- For DC machine which are subjected to severe working conditions, insulation test should be carried out at an interval of…………

- one week

- one month

- three months

- six months

- Which one of the following is the main reason for using compensating winding in a DC generator?

- Because it has a normal load

- Because it has a fluctuating load

- Because it has a higher value of load

- Because it has a lower value of load

- The generators are designed to deliver full load current (I) and the fall of no-load voltage will be about..........

- 5 to 8%

- 4 to 7%

- 3 to 6%,

- 2 to 5%

- Which type of D.C. generator is called as constant voltage generator?

- D.C. series generator

- D. C shunt generator

- D.C Long shunt compound generator

- D.C. short shunt compound generator

- The DC shunt generator is not building up voltage. The reason is.......

- field winding open

- residual magnetism available in the field poles

- generator running at the rated speed

- generator runs in the correct direction

- Which among the following statements is not correct?

- The best method of stopping a DC shunt generator is to open the field circuit switch

- The existence of residual magnetism is must in a self-excited DC generator

- As the shunt field current increases, the induced voltage also increases initially

- As-the speed of the DC generator increases, the induced-voltage also increases up to a certain extent

- The differential compound D. C generator could be changed to an over compounded generator by changing the connection of the..................

- series and shunt field

- series and armature fields

- armature

- series field

- Which type of DC compound generator is suitable for electroplating purpose?

- cumulative over compounded generator

- cumulative flat compounded generator

- cumulative under compounded generator

- differential compounded generator

- Which type of generator gives a constant voltage output at all loads?

- Level compound generator

- Commutative compound generator

- Differential compounded generator

- Series generator

22. D.C. Motors

- A force will act on a conductor placed in a magnetic field when:

- a current is flowing in the conductor

- the conductor is rotating

- the conductor is placed in the magnetic field and it is not carrying a current

- it is placed at right angle to the field

- The back e.m.f opposes:

- current

- applied e.m.f.

- torque

- terminal voltage

- Back e. m..f is developed in the armature ' due to………

- current

- supply Voltage

- intersection of the magnetic flux by the armature conductors

- interaction of the two magnetic fields

- The magnitude of Eb in a d.c. motor in the start is equal to:

- zero

- applied e.m.f

- more than applied e. m.f.

- maximum

- if in a Shunt motor the applied e. m.f is kept constant then on increasing the load, the shunt field strength will become______ its normal value.

- less than

- more than

- equal to

- zero

- in a series motor, the magnetic field-flux stands:

- proportional to armature current

- equal to armature current

- inversely proportional to armature current

- constant

- The speed of a series motor is very high at no-load because in such state.

- back e. mi. is larger

- armature voltage is lesser

- flux is very low

- armature current is very low

- A series motor is wound with :

- fine wire winding

- nichrome wire Winding

- thick wire winding

- double wire winding

- in a shunt motor a starter is used for :

- speed control

- back e.m.f. control

- applied e.m.f. control

- current control in the start

- Voltage equation for a dc. motor is :

- V = Eb + Ia . Ra

- V = Eb -  Ia . Ra

- Eb = V + Ia . Ra

- Eb = V -  Ia . Ra

- The rotating part of a dc. motor is known as :

- pole

- stator

- rotor

- armature

- In a d.c. machine, a dummy coil is used

- eliminate reactance voltage

- eliminate armature reaction

- create a mechanical balance in the armature

- eliminate vibrations of the machine

- The rotational speed of a dc shunt motor can be increased above its full speed by :

- increasing the armature current

- decreasing the armature current

- increasing the field current

- decreasing the field current

- On increasing the load of a dc. shunt motor, its speed will :

- remain constant

- increase proportionately

- increase slightly

- reduce slightly

- The armature torque of a do motor is given by :

- T = 0.159 ϕ. Z. Ia ( PA )N-m

- T = 0.159( EbN ). Ia N-m

- T = 0.162  Z. Ia ( PA )N-m

- none of these

- The armature current (la) drawn by a d.c. motor is proportional to :

- the applied e.m.f.

- the torque required

- the flux required

- the speed of the motor

- The power written on the name plate of a dc. motor is always :

- the power drawn in kVa

- the power drawn in kW

- the output power available at the shaft

- the gross power

- The efficiency of an electrical machine will be high if :

- its losses are minimum

- its current drainage is minimum

- the applied e.m.f. is minimum

- the load is minimum

- The direction of rotation of a dc. motor can be reversed by :

- reversing the supply connections

- interchanging the armature and field connections

- adding a resistance in the field circuit

- reversing the armature connections or the field connections

- The speed of a series motor at no-load is  :

- zero

- 3800 rpm.

- 3600 rpm.

- infinity

- If the back e.m.f. is absent in a dc. motor then the :

- motor will run at a very high speed

- motor will run at a very low speed

- motor will burn

- motor will not run at all

- By adding a variable resistance across the series field (field diverter) of a dc. series motor, speed above the normal speed can be achieved because :

- armature current is reduced

- flux is reduced

- line current is reduced

- none of these

- It the field of a dc. shunt motor gets opened

- it will continue to run at the rated speed

- its speed will become very high

- the motor will stop

- its speed will reduce

- When an electric train moves down a hill the dc. series, motor then acts as :

- dc. series meter

- dc. shunt motor

- dc. series generator

- d.c. shunt generator

- Which of the following motors is suitable for high starting torque?

- Shunt motor

- Cumulative compound motor

- Series motor

- Compound motor

- Which of the following methods is most economical for finding the no-load losses of a long shunt dc motor?

- Hopkinson’s test

- Swinburne‘s test

- Retardation test

- None of these

- The rated speed of a given shunt motor is 1050 r:p.m. To run this motor at 1200 rpm, which of the following speed control scheme will be suitable?

- Armature current resistance control

- Field resistance control

- Ward-Leonard control

- None of these

- Which of the following statement is more appropriate?

- Shunt motor is suitable for electric trains

- Series motor is suitable for machine tools

- Series motor is suitable for belt drive machines

- Compound motor is suitable for fly-wheel drive

- Which of the following motors is used to drive the constant speed line-shafting lathes, blowers and fans ?

- D.C. shunt motor

- D.C. series motor

- Cumulative compound motor

- Differential Compound motor

- Which type of test is being carried out in the given circuit of a dc. motor?

- Open-circuit test in d.c. shunt motor

- Test for armature short-circuit in d.c. series motor

- Open-circuit test in d.c. series motor

- Test for earthed field winding in compound motor.

- You notice that a dc. motor starts with jerk when starting the motor. What may be the fault for this effect?

- Line voltage too high

- Short-circuit in the field winding

- Pitted starter contact terminals

- Wrong grade of carbon brushes

- The speed of a do. shunt motor of 3 kW is 2000 r.p.m. when connected to a constant voltage supply. The speed is to be varied up to 2500 r.p.m. What speed control method is to be used?

- increasing the back e.m.f. with a resistor

- Limiting the armature current with a shunt resistor

- Changing the voltage with a starter

- Changing the field current by a field rheostat

- Which type of fault is tested in the given figure of a d.c. motor?

- Testing for open-circuit

- Testing the resistance value of each  coil

- Testing for open-circuit in field coil

- Testing for a grounded field coil

- Starting current of a 220 V, 10 HP, shunt motor having an armature resistance of 0.2 ohm without starter is :

- 110 Amp

- 1100 Amp

- 11 Amp

- 11000 Amp

- if the current in the armature of a dc. series motor is reduced by 50%, the  torque of the motor will become :

- 50%

- 25%

- 150%

- 200%

- If the load and flux of a DC motor are held constant, and the voltage applied across its armature is increased by 5 percent, the speed of the motor will……………

- increase by 5 percent

- decrease by 5 percent

- not change ‘

- infinity

- You have to change the defective motor of an electric, locomotive. Which type of motor would you choose?

- DC differential compound motor-

- DC shunt motor

- DC series motor

- Separately excited motor

- You have to change the defective motor of an electric Locomotive. Which type of motor would you choose?

- Differentially compounded DC motor

- D.C. shunt motor

- D.C. Series motor

- Cumulatively compounded DC motor

- Motor used in electric traction is..........

- DC compound

- DC shunt

- DC series

- DC long shunt compound

- For which one of the following applications D.C Series motor is used?

- Drives a machine drives tools and belt

- Rolling mill drives and other drives with toad characteristic

- Drives that can be started and run in no-load operation

- Electrical trams or trains and other vehicle drives independent speed

- What would be the conseqsence of no load operation of D C series motor?

- The motor will come to rest immediately due to low field current

- The armature current will increase abnormally therefore the fuse would blow

- The motor will reach Such a high speed that the centrifugal force may destroy the motor windings.

- There will be a decreased field current with the result motor speed will be low.

- During the normal operation of a DC series motor, if the field winding is suddenly open-circuited, what will be the effect on the motor?

  - it will run at slow speed

  - it will run at high speed

  - it will run at normal speed ‘

  - it will not run.

- Which type of motor would you choose for starting the transport vehicles?

- DC. series motor

- D.C. shunt motor

- cumulatively compounded DC motor

- differentially compounded DC motor

- Which type of motor would you choose for starting the transport vehicle?

- DC series motor

- DC shunt motor

- Separately excited DC motor

- DC differentially compound motor

- The back emf of a DC motor depends on the........

- shape of the conductor

- field flux

- commutator segments

- brush material

- The E.M.F  induced in a DC motor while running, is..........

- assist the applied voltage

- oppose 'the applied voltage

- decrease the current

- increase the current

- Calculate the back e.m.f. (Eb) of a 220 V, DC Shunt motor. Armature resistance is 0.5 ohm, and armature current is 40 A?

- 220 V

- 200 V

- 210 V

- 190 V

- The current taken by a DC. motor armature is given by the formula…………..

- Ia = V-EbRa

- Ia = V+EbRa

- Ia = VRa

- Ia = EbRa

- A 220 V DC. motor having an armature resistance 0.2 ohm and back and back e.m.f 215 V, it draws a current of...........

- 15 amps

- 20 amps

- 25 amps

- 50 amps

- The armature resistance of a DC machine will be in the order of.............

- 750 to 1000 ohms

- 500 to 750 ohms

- 10 to 50 ohms

- 0.1 to 1 ohm

- The carbon brushes of DC machines become shorter by wearing out during the running time of the machine. At what length do you have to change the brushes?

- 1/2 of the original length

- 1/3 of the original length

- 3/4 of the original length

- 2/3 of the original length

- How the HP, torque and speed are related together in a DC motor, if N is in 1/min (rpm) for DC motors?

- HP = 2πNT

- HP = 2πNT60

- HP = 2πNT60 ×735

- HP = 2πNT735

- The supply terminals of a DC. shunt motor are reversed. What will be the effect on the motor?

- it will run as a D.C. generator

- it will run in the reverse direction

- it will run in the same direction

- it will burn out

- What will be the consequences if the field  circuit of a DC. shunt motor is opened when the motor is running at no load condition?

- The motor speed will be reduced drastically

- The torque of the motor will be increased

- The motor speed could become

- Field current will become zero and dangerously high hence motor stops.

- To start a DC shunt motor the type of starter used is .......

- 2 point

- 3 point

- 4 point

- Drum type

- The connection of NVC in DC. 3 point starters is across DC supply in series with

- armature

- starting resistor

- overload relay

- shunt field

- In a 3 point starter the No Volt coil (NVC) is connected in series with ..............

- armature

- supply

- starting resistor

- field

- A DC shunt motor connected through a 4 point starter starts normally when the starter handle is moved from OFF position to ON position. But the starter handle does not remain in ON position. One of the reason for the above is open in………

- shunt field

- protective resistor

- starting resistor

- overload coil

- For which one of the following applications, D.C. shunt motor is used?

- Electric trams or trains

- Electric hoist and cranes

- Drives with high starting torque

- Machine tool drives

- The full lead current of a 10 kW, 250 V, DC motor will be ...........

- 10 amperes

- 20 amperes

- 30 amperes

- 40 amperes

- The speed of a, Shunt motor formula is………

- Z = ϕEb

- N = Ebϕ

- N = K.Ebϕ

- N = ϕK.Eb

- Which type of fault is tested in the figure of D.C. motor?

- Testing the continuity of each winding

- Testing for open circuits

- Testing for grounds

- Testing the resistance value between each winding

- identify by reasoning the differentially compounded D. C. motor from the figure shown below

- Which type of D.C. motor is preferred for Elevators?

- D.C. Shunt motor

- D. C Series motor

- Cumulatively compounded D C. motor

- Differentially compounded D C motor

23. Transformers

- On removing the cores of a transformer, the production of magnetic flux:

- is reduced

- is increased

- remains the same

- none of these

- The no. or magnetic paths in a berry type transformer:

- is one

- are two

- are three

- are numerous.

- In a transformer, the linkage of magnetic flux takes places from:

- primary to secondary.

- secondary to primary

- primary to secondary, and from secondary to primary

- none of these,

- Since, the cores of a transformer are made of silicon-steel, therefore:

- copper-loss remains low

- eddy-current-loss remains low

- hysteresis-loss remains low .

- both Copper and eddy-current losses remain low.

- The efficiency of a transformer stands high because:

- it works on the principle of mutual induction

- it is a static instrument

- its windings remain immersed in the mineral oil

- there is no loss of electric power in it.

- For equal capacity, a shell type transformer as compared to a core type transformer will have

- low copper-loss

- low iron-loss

- low copper and iron losses

- high copper and iron losses.

- if the supply frequency of a transformer is reduced then its iron-loss will be :

- low

- high

- the same

- very high.

- If the transformers have different voltage ratios (Ep / Es )and their parallel operation is desired then:

- they will not work on load

- they will not work at all

- they will work at different power factors

- they will ‘work at an average voltage.

- The capacity of a star-connected transformer as compared to a delta connected transformer will be:

- 3 times

- 1/3 times

- 1/2 times

- equal

- In the breather of a transformer, Silica-jelly is used to stop the to reach up to the insulating oil:

- rain water

- dry air

- moisture

- steam

- The principle of working of a transformer is:

- static induction.

- self induction

- mutual induction

- dynamic induction.

- A transformer is used for changing the value of:

- voltage

- frequency

- power

- power factor

- A transformer operates :

- always at unity power factor

- at its own power factor

- at a power factor below a particular value

- at a power factor depending on the power factor of the load.

- The reactance of a transformer is determined by its :

- common core flux

- leakage flux

- size of the Core

- permeability of the core material

- The efficiency of a transformer is normally in the range of:

- 50% to 70%

- 60% to 75%

- 80% to 90%

- 90% to 98%

- Eddy current Ioss in a transformer can be reduced by.

- increasing the thickness of the laminations

- decreasing the thickness of the laminations

- Winding the dolls with a higher gauge wire

- decreasing the air-gap in the magnetic circuit.

- The main property of a transformer is this that it changes the voltage level of an a.c. signal:

- without changing its power

- without changing its frequency

- without changing its shape

- without changing its power, frequency and shape

- Transformers are rated in :

- kW

- W

- kWh

- kVA

- What type of core is used in a transformer designed to work at high frequencies (MHz range)?

- Aluminium core

- Iron core

- Air core

- Open type iron core.

- if the secondary turns of a transformer are doubled and at the same time the primary voltage is reduced to half men the secondary voltage will:

- be halved

- be doubled

- not change

- be four times high

- The no-load current of a transformer in terms of full-load current is usually :

- 1% to 2%

- 2% to 5%

- 5% to 12%

- 12% to 20%.

- Which is the common method of a cooling a power transformer used in distribution work?

- Air cooling

- Air-blast cooling

- On cooling

- Water cooling

- The transformer oil used in transformers provide:

  - cooling and lubrication

  - insulation and lubrication

  - insulation and cooling

  - insulation cooling and lubrication.

- What is the typical use of an auto transformer?

- To work as a control transformer

- To work as a variable transformer

- To work as a distribution transformer

- To work, as an isolating transformer.

- As compared to an amplifier a transformer cannot:

- increase output power

- increase Output voltage

- increase output current

- none of these.

- A conservator used in a transformer consists of:

- an air-tight metallic drum fixed at the top of the oil-tank

- an air-tight metallic drum fixed at the bottom of the oil-tank

- overload protection device

- none of these.

- In a power transformer, the winding nearer to the iron core :

- is a high voltage winding

- is a low voltage winding

- may be low or high voltage winding

- the two types of windings are placed in a sandwich manner.

- A good transformer oil should be free from:

- sulpher

- alkali

- moisture

- all the above.

- The two transformers operated in parallel will share the load depending upon their:

- rating in kVA

- efficiency

- leakage reactance

- per unit impedance

- In an  ideal transformer, the no-load primary current Io:

- is in phase with VP

- Lags behind VP by 90°

- Leads VP by 90°

- lags VP by an angle lying between 0° and 90°

- Which of the following reduces high voltage and current to safe values of measurement?

- Mega ohmmeter

- Auto transformer

- Variable transformer

- Instrument transformer.

- When a transformer is operating on no-load, the primary applied voltage is approximately balanced by

- Secondary induced e.m.f.

- Primary induced e.m.f.

- Terminal voltage across the secondary

- Voltage drop across resistance and reactance

- Which one of the following methods will be preferred for a transformer cooling, above 200 kVA?

- Insulating oil

- Air blast .

- Oil natural, Air forced

- Oil forced, Water forced.

- Scott connections are used to transform:

- single phase supply into three-phase supply

- single- phase supply into two phase supply

- three--phase supply into two phase supply

- star-connected primary into delta-connected secondary

- In a 3-phase delta-transformer, one of the phase windings has burnt. The transformer will then supply

- its full output rating

- zero output

- 86 6% of its output rating

- 58% of its output rating

- Which type of connection in 3 phase transformers is used for the sub-station end of the transmission lines?

- Star-star

- Star-delta

- Delta-Delta

- Delta-star

- in three phase. transformer, different methods of connections are possible. Which type of connection used in distribution transformer?

- Star-star

- Star-delta

- Delta-Delta

- Delta-star

- What is the formula used to find out the full load current in three phase transformer?

- √3.kVA×1000V

- 3 V.I. cos Ɵ

- kVA×1000√3×V

- kVA√3× cos Ɵ

- Which type of transformer winding has no ohmic resistance and no magnetic leakage resulting in no losses?

- Distribution transformer

- Power transformer

- ideal transformer

- instrument transformer

- The rated value of secondary voltage for single phase and three phase potential transformer as per IS. 3156 shall be:

- 240 V and 415 V

- 230 V and 400 V

- 220 V and 380 V

- 100 V and 110 V

- In a transformer, the number of turns per volt is 6. The primary and secondary. Find voltages are 240 V and 32 V respectively. the number of primary and secondary winding turns?

- 880 turns; 200 turns

- 1200 turns; 192 turns

- 1440 turns ;192 turns

- 1550 turns; 200 turns

- A 10KVA transformer is taking a power of 400 watts in open circuit test and 900 watts in short circuit test. What will be the copper loss at 50% of load?

- 1300 watts

- 500 watts

- 225 watts

- 450 watts

- What will be the copper loss at half load, of the fun load copper loss in 1600 watts?

- 1600 watts

- 800 watts

- 400 watts

- 200 watts

- The wattmeter reading of transformer on open circuit test is 900 W and on short circuit test at full load current is 1600 W. What will be the iron has at half full load?

- 600 watts

- 900 watts

- 1200 watts

- 1800 Watts

- The copper loss of the transformer during  the maximum efficiency is 1000 watts What will be the iron loss?

- 1200 W

- 1000 W

- 900 W

- 800 W

- The efficiency of a transformer is at a maximum value, when the..............

- copper loss is equal to iron loss

- popper less is more than iron loss

- copper loss is less than iron loss

- copper loss is zero

- Generally a transformer has the efficiency of….

- 40%

- 60%

- 80%

- 97%

- Results of a transformer with a rating of 15 kVA 1100V/220V, 50Hz are obtained, while testing Short circuit test power input = 510 Watts Open circuit test power input: 252 watts. What is the efficiency of the transformer at full load, 0.8 pf?

- 92%

- 93%

- 96%

- 94%

- What is the maximum load up to which an open delta-transformer can safely maintain supply to load?

- 33%

- 50%

- 58%

- 66%

- What is the caries for producing humming noise in the transformer core?

- Overload of the: transformer

- Lew rated voltage

- Insufficient core clamping of laminated

- Misalignment of stamping

- No load: loss. of a transformer has two components. They are eddy current and hysteresis loss. In which part of a transformer hysteresis loss will occur?

- Core

- Breather

- Transformer oil

- Conservator

- Which material is used for the construction of the power transformer core?

- Mild steel

- High carbon steel

- Silicon steel

- Stainless steel

- What is the formula to determine the transformation ratio ‘K’?

- K = N2N1 = E2E1 =I2I1

- K = N1E1 = N2E2 =I2I1

- K = N2I1 = E2N1 =I2E1

- K = N2N1 = E2E1 =I1I2

- The percentage of voltage regulation (VR ) of a transformer is determined by using the formula................

no load voltage

- VR = full load voltageno load voltage × 100

full load voltage

- VR = No load voltageno load voltage × 100

full load voltage

- VR = No load voltagefull  load voltage × 100

no load voltage

- VR = full load voltagefull load voltage × 100

- Which type of transformer does not isolate the secondary?

- Potential transformer

- Auto transformer

- Distribution transformer

- Current transformer

- Voltage stabilizers are having auto transformer with tappings or step less variation depending upon the type of stabilizer. Auto transformer in servo stabilizer is of...............

- core type

- shell type

- toroidal type

- distributed core type

- Single turn primary of a current transformer has a turns ratio 10:1. Which one of the following is the primary current it the current measured in secondary is 2 amperes?

  - 1 ampere

  - 10 amperes

  - 5 amperes

  - 20 amperes

- As compared to, the secondary of a step up transformer, the primary will have…………….

- lower voltage and higher current

- higher voltage and higher current

- higher voltage and lower current

- lower voltage and higher current

- in a transformer, if the number of turns in the secondary winding is twice that in the primary winding, the secondary voltage will be.............

- half that of primary voltage.

- same as the primary voltage.

- twice the primary voltage.

- four times the primary voltage.

- Which one of the following can be increased. by using a step-up transformer?

- Power

- Voltage

- Current

- Frequency

- In 1 kVA transformer the number of turns per volt is 8. What is the number of turns in high voltage side, if the voltage is 110V?

- 1920

- 880

- 192

- 125

- What is the full load current of 1 KVA, 240V/120V single phase transformer at 0.8 p.f.?

- 8.33 amps

- 6.66 amps

- 4.16 amps

- 3.33 amps

- Which among the following transformers, is used for starting the big induction motor?

- Constant current transformer

- Auto transformer

- Potential transformer

- Step-up transformer

- As compared to an amplifier a transformer cannot………..

- decrease the output power

- increase 'the output power

- increase the output current

- increase the output voltage

- Which among the following transformers is used for street lights?

- Constant current transformer

- Tapped transformer

- Auto transformer

- Potential transformer

- Small type transformer core for shell type generally used..................

- E & I type of core.

- F & F type of core.

- E & E type of core.

- T & U type of core.

- What is the - unit of output power consumed during the day taken for calculating the ALL DAY efficiency of a Transformer?

- kVA/H

- kVAH

- kW/H

- kWH

- When two transformers are operating in parallel, they will share the load depending upon their…………

- leakage reactance

- magnetising current

- per unit impedance

- kVA rating

- The Transformers are rated in..........

- kW

- kVA

- kWH

- kVAR

- The colour of silica gel in clay condition is………

- pink

- blue

- green

- red

- What is the material used inside the breather to prevent moisture entering the transformer?

- sodium chloride

- sodium silicate

- silica gel

- copper sulphate

- Efficiency of transformers is determined, by knowing its input and output power. Unit of power used for this purpose is kVA/kW. What is  the unit used for input and output when calculating the all day efficiency?

- kVAR

- kW

- kVA

- kWh

- The transformer used in conjunction with  voltmeter is called a potential transformer. The standard rating of secondary of potential transformer is...............

- 240 V

- 110 V

- 415 V

- 660 V

- Secondary of a current transformer will have...........

- more turn of thick gauge

- less turn of thin gauge

- more turn of thin gauge

- less turn of thick gauge

- If the current rating of the installation is high, a current transformer is used in the energy meter circuit,  if a current transformer 0f 100A/5A is connected with an energy meter, the multiplying factor for the recorded energy is............

- 5

- 20

- 95

- 105

- in constant voltage transformer at certain induced secondary voltage, the impedance of secondary winding will become equal to impedance of the capacitor across it. What is this condition of circuit known as?

- Saturation

- Equality

- Good

- Resonance

- What may be the cause for overheating of a transformer?

- High ambient temperature

- input voltage is too low

- Load is too high

- Are from the transformer primary to the secondary

- In which part of the distribution transformer, maximum heat occurs?

- Transformer core

- Transformer tank

- Transformer oil

- L.V. and H.V. winding

- What principle makes the instrument transformer working?

- Self induction

- Lenz’s law

- Mutual induction

- Fleming’s right hand rule

- What is the standard value of secondary current of current transformer?

- 10 amperes

- 5 amperes

- 2 amperes

- 3 amperes

- Current transformers are classified into five types. Which type of current transformer (C.T.) principle is used in clip on or clamp on Ammeter?

- Oil immersed C.T.

- Dry type C.T.

- Ring type C.T.

- Wound type C.T.

- What is the formula to be used to measure the actual power consumed, when the wattmeter is used along with C.T. and R.T?

- Wattmeter reading x C.T. ratio

- Wattmeter reading x P.T. ratio

- Wattmeter reading x C.T. ratio x P.T. ratio

- Wattmeter reading C.T.ratio x P.T. ratio

- When using a potential transformer, it should be connected to the..............

- primary winding

- secondary winding

- low voltage winding

- high voltage winding

- What is the standard rating of secondary of potential transformers?

- 110 V

- 220 V

- 330 V

- 440 V

- You find a C.T. of ,10 : 1 ratio and P.T. of    100 : 1 ratio are available in the stores. Ammeter of 0 to 5 amperes and voltmeter of 0 to 110 V are also available. The maximum high-value of voltage and current that could be measured by using the above is...........

- 1100 V, 50A

- 11 kV, 50A

- 11 W, 500 A

- 110 W, 500 A

- The function of a breather in a transformer is..........

- to provide cooling air to the winding ,

- to arrest the flow of moisture into the tank

- to control the level of bit in the tank

- to filter the transformer oil

- The function of a breather in a transformer is...........

- to provide protection against over current

- to suppress harmonics

- to arrest flow of moisture into the tank

- to control the level of oil in the tank

- The function of a breather in a transformer is to.........

- provide peeling air to the winding

- arrest the flow of moisture into the tank

- control the level of oil in the tank

- allows the moisture

- You have to measure the current and voltage of a 3 phase circuit of 6.6 kV and load of 500WA. in order to use a 5 A ammeter and 110 volts voltmeter, what is the ratio of CT and PT required?

- PT 60 : 1 CT : 15 :1

- PT 60 : 1,CT : 50 :1

- PT 1 : 60, CT  1 :10

- PT 25 : 1, CT 100 :1

- In a small workshop 5 A energy meter is connected through a CT. The meter reader recorded 1000 units for a particular month but, the meter shows only 25 units. What is the current in primary of CT when a current of 4 A is flowing through the meter?

- 40 amps

- 160 amps

- 200 amps

- 250 amps

24. Alternators

- A 4-pole, 50 Hz alternator will have a rotational speed of:

- 750 r.p m.

- 1500 r.p m.

- 3000 r.p m.

- I440 r.p m.

- An exciter used for field excitation of an alternator will be a:

- series generator

- shunt generator

- series-motor

- shunt motor

- The reason of using laminated cores in the stator of an alternator is to :

- reduce the weight of the machine .

- reduce the eddy current loss

- increase the magnetic flux

- increase the eddy current loss

- D.C. and A.C. generators are similar in one Important respect that is

- the generated e m.I. is D.C

- the generated e.m.f. is pulsating

- the generated e.m.f. is A.C.

- the generated e.m.f. is oscillating

- A 8-pole alternator rotates 750 rpm. its per cycle periodic time will be…………

- 10 ms

- 20 ms

- 30 ms

- 50 ms

- Alternators designed to be driven by steam turbines are built for:

- high speed operation

- low speed operation

- medium speed operation

- both low and medium speed operations

- Single phase portable generators are generally made in the range of:

- 5000 kVA

- 1000 kVA

- 100 kVA

- 5 kVA

- An alternator supplies electric power to various kinds of loads of a factory It the phase sequence of the alternator is changed. It will affect the

- 3-phase heating circuits

- 3-phase motor circuits

- Single phase motor circuits

- lighting circuits

- The values of pitch factor and distribution factor for an alternator are 0.9 and 0.8 respectively, This will develop an e.m.f. lesser than an alternator having full pitch coil and concentrated winding by:

- 0.7

- 0.72

- 0.88

- 0.92

- The winding of a 4-poie alternator is having 36 slots and a coil span of 1 to 8 ’it is short degrees. pitched, by............. electrical

- 80

- 40

- 20

- 10

- The number or slip-rings used in a 3-phase, star connected alternator having a rotating armature will be:

- 2

- 3

- 4

- 6

- The pitch factor is defined as the ratio of the e.m.f. is of the:

- full-pitch winding to short pitch winding

- short-pitch coil to full pitch coil

- full- pitch winding to concentrated winding

- distributed winding to full pitch winding

- The rotor of a salient pole alternator has 12 poles The number of cycles of e. m.f. in one revolution will be :

- 2

- 4

- 6

- 8

- in large sized alternators, dampers are used to:

- reduce current fluctuations

- reduce voltage fluctuations

- increase stability

- increase output e.m.f

- The number Of electrical degrees which passes through in one revolution of a 6-pole alternator is :

- 2160°

- 1080°

- 720°

- 360°

- The waveform of the e.m.f. generated by an alternator should be :

- pulse shaped wave

- square wave

- sine wave

- triangular wave

- The field circuit of a hydro alternator is supplied with:

- an a. c.

- a pulsating current

- an oscillating current

- a d.c.

- When a coil / conductor moves at right angle to the magnetic flux, the induced e. m..f will be :

- lagging

- leading

- maximum

- zero

- Salient pole type rotors as compared to the cylindrical pole type rotors are:

- smaller in diameter and larger on axial length

- larger in diameter and smaller in axial length

- larger in diameter and axial length

- smaller in diameter and axial length

- The frequency generated by a 8-pole alternator rotating at 600 r.p.m will be:

- 20 Hz

- 40 Hz

- 60 Hz

- 80 Hz

- The frequency of voltage generated by an alternator having 4 poles and rotating at 1800 R.P.‘M. is………..

- 60 Hz

- 7200 Hz

- 120 Hz

- 450 Hz

- A 50 H2 alternator will run at the greatest possible speed it it is wound for ........ poles

- 8

- 6

- 4

- 2

- When speed-of an alternator is changed from 3600 Ft PM to 1800 R. PM the generated e.m.f per phase will become

- one-half

- twice

- four times

- one-fourth

- The power factor of an alternator is determined by its

- speed

- load

- excitation

- prime mover

- For proper parallel operation, polyphase alternators must have the same

- kVA rating

- voltage rating

- speed

- excitation

- The number of poles in turbo alternator is…….

- 2

- 6

- 8

- 10

- The frequency of e.m.f. generated_ by an alternator depends upon the speed N and No. of poles is given by

- PN60

- 60 NP

- PN120

- 120 NP

- The current from the stator of an alternator is taken out to the external load circuit through

- slip rings

- commutator segments

- solid connections

- carbon brushes

- in asynchronous machine, the stator frame is made of

- Stainless steel

- Aluminium

- Cast Iron or welded steel plates

- Laminated silicon steel

- The stator of an alternator gets overheated due to……….

- open phase

- unbalanced currents in the phases

- improper alignment of the rotor

- any One or more of the above

- Which type of alternator is used in hydro electric power stations?

- Non salient pole alternator

- Turbo generator

- salient pole alternator

- steam turbine alternator

- The armature reaction effect in a synchronous machine depends on

- load current

- power factor of the load

- speed of the machine

- both A and B

- Skew of rotor bar eliminates

- the effect of space harmonics

- the entire effect of crawling

- magnetic noise

- variation due to unequal force developed on rotor

- The alternators are rated as............

- kw

- kVAR

- kVA

- kWh

- The generated frequency in lndia is

- F = 65Hz

- F = 60Hz

- F = 55 Hz

- F = 50 Hz

- The relation between frequency speed and number of poles in-Alternator is given as......

- f = PN60

- f = PN120

- f = P2 x N

- f = P×60N

- A 10 pole alternator rotates at a constant speed of 1200 rpm. The periodic time of voltage in seconds per cycle will be

- 0.01

- 0.02

- 0.04

- 0.08

- The field circuit of an alternator is supplied with.........

- an oscillating current

- a direct current

- an alternating current

- a pulsating current

- The DC. and A.C. generators are similar in one important function. That is the……….

- generated e.m.f. is a direct current

- generated e.m.f. is an alternating current

- generated e.m.f. is a pulsating current

- generated e.m.f. is an oscillating current

- Adjusting the speed of the prime mover of an alternator is primary/necessary. Which one of  the following is the reason for this?

- To get constant frequency

- To get constant voltage

- To get increase polarity

- None of the above

- Which one of the following is the reason for using dampers in large alternator work?

- To increase stability

- To increase output

- To decrease output

- To reduce voltage fluctuation

- The speed of a 6 pole 50 Hz of an alternator is......

- 1800 rpm

- 1200 rpm

- 1000 rpm

- 600   rpm

- The RMS value of emf induced per in alternator is given as phase

- Ep = 4.44 fϕT volt

- Ep = 4.44 ϕT volt

- Ep = 2.22 ϕT volt

- Ep = 2.22 ϕf volt

- A large diameter of alternator is running at slow speed will be having.......

- rotating alternator

- rotating field with a Smooth cylindrical rotor

- rotating field with a salient pole rotor

- rotating armature with field

- A 3-phase star connected alternator having a rotating armature will have……….

- 2 slip rings

- 3 slip rings

- 4 slip rings

- 6 slip rings

- A 3-phase, delta connected alternator having a rotating armature will have………

- 2 slip rings

- 3 slip rings

- 4 slip rings

- 6 slip rings

- A 3-Tphase, star connected alternator having a rotating armature will have 4 slip rings but as phase, delta connected alternator having armature will have

- 6 slip rings

- 4 slip rings

- 3 slip rings

- 2 slip rings

- What is the name of the part of an alternator shown in the figure

- salient pole type rotor

- semi enclosed slot- type stator

- smooth cylindrical pole type rotor

- totally enclosed slot type stator

- In star connected alternator. If the line

voltage is EL = √3 Eph

What Will be the phase voltage when it is

connected in delta?

- Eph = 13 EL

- Eph = √3 EL

- Eph  = EL√3

- Eph  = EL

- A 3-phase alternator is connected in delta Reactance of each phase is 100 Ὡ and is rated to produce 415 volts.

What is the current flow within delta

connection When the alternator is running

under no load condition?

- 41.5 amperes

- 4.5 amperes

- 0.415 amperes

- 0 amperes

- When all dark lamp-method is used for synchronizing two 8-phase alternators, the lamps are not becoming dark at the same time but occurs at different timings. Bulbs used are of same wattage and voltage rating, connection of the lamps were checked and found OK. This is due to the difference in one of the factors stated below identity the one?

- Frequency

- Speed

- Phase sequence

- Voltage

- What is the type of lamp method illustrated in. the figure for synchronizing 3 phase alternators?

- All dark lamp method ,

- All bright lamp method

- Two bright and one dark lamp method

- Two dark and one bright lamp method

- Polyphase Induction Motors

- The speed of rotating magnetic flux of a 3 phase induction-meter is given by the expression:

- 120.Pf

- P.f120

- 120.fP

- f120.P

- The synchronous speed of a 2, 4 and 8 poles motor at 50Hz cannot be :

- 3000 RPM.

- 1000 RPM.

- 1500 RPM.

- 750 RPM.

- The no of poles of a 3 phase induction motor running at 500 R. PM will be:

(supply frequency: 50 Hz)

- 12

- 76

- 4

- 2

- The relation of the torque developed in an induction-motor and the supply voltage is given by the expression.

- T ∞ V

- T ∞ V

- T ∞ V2

- T ∞ 1V2

- The starting torque of a 3-phase induction motor can be increased :

- by increasing the rotor resistance

- by Increasing the stator resistance

- by increasing the air-gap between in stator and the rotor

- none of these

- At the time of starting, the slip of an induction motor is equal to:

- infinite

- unit

- zero

- none of these

- The reason of drawing high current at the time of starting by an induction-motor is :

- high voltage

- low power factor

- zero back e.m.f

- none of these.

- For the speed control purpose, pole changing method can be adopted for a :

- slip-ring motor

- synchroronous motor

- squirrel-cage induction-meter

- single phase motor

- The principle of operation of an induction-motor is based on:

- Faraday’s law

- Rotating magnetic field

- Flemings right hand rule

- Ohms law

- The speed of rotor of an induction motor is:

- the same as the synchronous speed

- higher than the synchronous is speed

- lesser than the synchronous speed

- determined by the frequency and the number or poles

- To reverse the direction of rotation of a 3 phase induction-motor it is necessary to :

- interchange all the three line connections

- reverse the polarity of the rotor3 circuit

- increase the resistance of the rotor’ s circuit

- interchange any two out of the three line connections.

- Squirrel-cage induction- motor differs from a slip-ring induction- motor as it has no:

- slip rings

- rotor windings

- stator windings

- rotating part

- squirrel-cage induction motor has :

- zero starting torque

- very small Starting torque

- very high starting torque

- highest Starting torque amongst all types of induction meters.

- The frequency of the rotor current (fr) in an induction motor is given by the expression:

- f = S . fr

- fr = Sf

- fr = S . f

- S = f . fr

- The torque of a 3-phase induction motor :

- increases with an increase in supply voltage

- decreases with an increase in supply voltage

- has no relation with supply voltage

- T ∞ 1V2

- The purpose of connecting resistance in the rotor circuit of a slip-ring induction-motor is to :

- limit the starting current

- increase the; starting current

- limit the starting current, and to increase the starting torque

- decrease the starting' torque

- if the induction-motor of a pump is operated at 30%‘less supply voltage then what will happen?

- It will start after some time

- It will continue to run at a lower speed without any damage

- it will get heated up and consequently will get damaged

- Nothing adverse will happen.

- in an induction-motor, the fractional slip(s) is given by the expression :

- S = Nr-NsNs

- S = Ns-NrNs

- S = Nr-NsNr

- S = Ns-NrNr

- It is advisable to avoid the use of a (d)O.L. starter for starting an induction motor having 5HP. capacity because :

- the, starting torque will be very low

- the motor will take five to seven times more starting current than its full load current

- it will pick-up very high speed and may go out of step

- it will run in the reverse direction.

- A thermal overload relay is provided in a starter to protect the motor against :-

- short-circuit

- open-circuit

- excess current

- low voltage

- In a phase induction - motor started by a(d)O.L. starter, protection against short-circuit is given by the: ‘

- overload relay

- no-volt coil

- back-up fuses

- protective earth contractor

- When a squirrel cage induction-motor is started directly by a (d)O.L. starting current will be: starter, the

- equal to the full-load current

- twice the full-load current

- 3 times the full-load current

- 5 to 7 times the full-load current

- When we connect an induction-motor in star through a starter, the applied voltage across each winding becomes :

- VL volts

- VL x 3 volts

- VL3

- VL x 3

- Figure below illustrates the connections made for identification of terminals of 3-phase induction motor by voltmeter method.

if the linked terminals are connected as

per the figure, the voltmeter V2 reads:

- supply voltage (240 V)

- more than supply voltage

- less than supply, voltage

- zero volts

- Total electrical degrees for a 4 pole a.c. machine is equal to:

- 180°

- 360°

- 720°

- 1080°

- A 3-phase, 400 V, 50 Hz induction motor has a, rated speed of 715 rpm. How many poles are there in the machine?

- 2

- 4

- 6

- 8

- With a fractional slip of 0.07, the rotor speed of a 4 pole squirrel cage induction motor fed from a 50 Hz supply is :

- 25.65 rev/s

- 24.48 rev/s

- 23.25 rev/s

- 22.11 rev/s

- The starting torque of a squirrel cage-induction motor is low because of:

- low starting current, low starting P.F.

- low starting current, high starting P.F.

- high starting current, high starting P.F.

- high starting current, low starting P.F.

- It a 3-phase motor runs with two phases only. it is called single phasing. it will ultimately :

- carry no load

- Burn out

- run with triple speed

- not run efficiently.

- On switching ‘ON’ the starter, the magnetic contractor vibrates heavily. The possible cause is:

- low voltage and single phasing

- high voltage phasing

- dirt between the contacts

- low voltage and dirt between the contacts.

- A squirrel cage motor of above 3 HP. should not be started by a (d)O.L. starter because:

- the motor takes 5 to 7 times of its full load current which will spoil the starter

- the motor will pick up a tremendous speed

- the motor will-have a low starting torque

- the motor will run in the reverse direction.

- Thermal overload relays are ____than magnetic overload relays.

- faster

- slower

- much bigger

- very smaller

- Which of the following reduces vibration, chattering, noise, wear and heat in the armature of a starter?

- Electromagnet

- Shading coil

- Centrifugal Switch

- Holding lock

- Minimum requirement of contacts in a 3-phase (d)O.L. starter is………

- 4 nos

- 3nos and one NC

- 3 nos

- 2 nos and one NC

- NVC used in a starter is meant to protect the motor against

- short-circuit

- open-circuit

- excess current

- low voltage

- Full load line current of a 3-phase, 3 H.P. 415V, 50 Hz, squirrel cage induction motor is approximately:

- 3 A

- 4.5 A

- 6 A

- 9 A

- No- load current of an induction motor is approximately______percentage of full load current.

- 10

- 20

- 40

- 75

- In a (d)O.L. starter a NVC is connected across a phase and neutral of a 3-phase, 415 V. 50 Hz supply and it Works normally. What is the rating of NVC?

- 200 V

- 230 V

- 400 V

- 415 V

- A (d)O.L. starter can be used satisfactorily for a double cage squirrel cage induction motor up to:

- 1 kW

- 1.5 kW

- 3 kW

- 5 kW

- The purpose of a back-up fuse is to provide protection against:

- overload

- over voltage

- excess current

- short- circuit

- A 4pole, 3-phase 50 Hz motor’s speed is 95% of its synchronous speed then the actual speed of the motor is

- 500 r.p.m.

- 1300 r.p.m.

- 1400 r.p.m.

- 1425 r.p.m.

- The induction motor of a pump is run at a supply voltage 30% lesser than the normal voltage (415 V). The pump starts to work After some time what possibly can happen?

- The motor will become hot and will be damaged

- The motor will start after some time

- The meter will function normally

- None of the stated above will happen.

- The purpose of adding a rheostat to slip-ring induction motor is to:

- limit the starting increase

- increase the» starting current

- decrease the starting current so that the starting torque may increase

- none of the stated above

- The rotor current frequency Of a slip-ring induction motor depends on:

- rotor conductor

- rotor inductance

- amount of slip

- inductive reactance

- When the rotor of a slip ring induction motor Is standstill, the rotor frequency is:

- zero

- higher than the stator frequency

- lower than the stator frequency

- equal to the stator frequency

- If the load on a motor13 increased beyond its full- load point, the greatest effect of the Increase in slip is in:

- increasing the power factor

- decreasing the power factor

- increasing the current

- decreasing the current

- Regarding skewing of motor bars in a squirrel cage induction motor (SCIM), which statement is false?

- it prevents cogging

- it increases starting torque

- it produces more uniform torque

- it reduces motor “hum during  its operation

- The principle of operation of a 3-phase induction motor is similar to that of a

- synchronous motor

- repulsion start induction motor

- transformer with a shorted secondary

- capacitor start, induction run motor

- The effect of increasing the length of air gap in an induction increase the motor will be to

- power factor

- speed

- magnetizing current

- air gap flux

- In a 3 phase induction motor, the relative a speed of stator flux with respect to ______is zero.

- stator winding

- rotor

- rotor flux

- space

- A 3-phase, 4-pole, 50 Hz induction motor runs at a speed of 1440 RPM. The rotating field produced by the rotor rotates at a speed of ____R.P.M. with respect to the rotor.

- 1500

- 1440

- 60

- 0

- in a 3phaseinduction motor, the rotor field rotates at synchronous speed with respect to

- stator

- rotor

- stator flux

- none of the above

- Irrespective of the supply frequency, the torque when developed by a SClM is the same ever is the same.

- supply voltage

- external lead

- rotor resistance

- slip speed

- The number of stator poles produced in the rotating magnetic field- of a 3-phase induction motor having three slots per pole per phase is

- 3

- 6

- 2

- 12

- The power a factor of a squirrel cage induction motor is

- low at light loads only

- low at heavy loads only

- low at light and heavy loads both

- low at rated load only

- A 6-pole, 50 Hz 3-phase induction motor is running at 950 R.P.M. and has rotor copper loss ot5 K.W. Its rotor input is _____KW

- 100

- 10

- 95

- 5.3

- The efficiency of a 3-phase induction motor is approximately proportional to

- (1 – S)

- S

- N

- NS

- A 6 pole, 50 Hz, 3-phase induction motor has a full load speed of 950 R.P.M. At half load, its speed would be______ R.P.M.

- 475

- 500

- 975

- 1000

- If rotor input of a SCIM running with a slip of 10% is 100 KW,  gross power developed by its rotor is _____KW.

- 10

- 90

- 99

- 80

- When-applied rated voltage per phase is reduced by one-half, the starting torque of a. SCIM becomes _____ of the starting torque with full voltage

- 12

- 14

- 12

- 32

- The fractional slip of an induction motor is the ratio of

- rotor copper loss to rotor input

- stator copper loss to stator input

- rotor copper loss to rotor output

- rotor copper loss to stator copper loss

- The torque developed by a 3-phase induction motor depends on the following three factors

- speeds, frequency, number of poles

- voltage, current and stator impedance

- synchronous speed, rotor speed and frequency

- rotor e.m,f rotor current and rotor  power factor

- The efficiency and power factor of a SCIM increases in proportion to its

- speed

- mechanical load

- voltage

- rotor torque

- if the starter Voltage and frequency of an induction motor are reduced proportionalality, its

- locked rotor current is reduced

- torque developed is increased

- magnetising current is

- both A and B decreased

- An induction motor has a shaft circuit current 7 times the full load current and a full load slip of 4 percent. Its line starting is times the full load torque.

- 7

- 1.96

- 4

- 49

- if stator voltage of a SCIM is reduced to 50 percent of its rated value, torque developed Is reduced by _____ of its full load value percent

- 50

- 25

- 75

- 57.7

- A SCIM with 36-slot stator has two separate windings: one with 3‘coil groups per phase per pole and the other with 2 coil groups per phase per pole The obtainable two motor speeds would be in the ratio of

- 3:2

- 2:3

- 1:2

- 2:1

- A 6 pole, 3-phase induction motor taking 25kW from a 50 Hz supply is cumulatively cascaded to a 4-pole motor. Neglecting alt losses, speed of the 4-pole motor would be___________ RPM

- 1500

- 1000

- 600

- 3000

- Mark the WRONG statement.

charge motor is capable of behaving as:

- inverted induction motor

- slip ring induction motor

- shunt motor

- series motor

- When a stationary 3-phase induction motor is switched on with one phase disconnected –

- it is likely to burn out quickly unless immediately disconnected.

- it will start, but very slowly

- it will make jerky start with loud growing noise

- remaining intact fuses will be blown out due to heavy inrush of current

- It single phasing of a 3-phase induction motor occurs under running conditions, it

  - will stall immediately

  - will keep running though with slightly increased slip

  - may either stall or keep running depending on the load carried by it

  - will become noisy while it still keeps running’s

- lf single phasing occurs during running of a 3 phase motor, what is the effect on motor performance?

- Motor will run as 2 phase motor and performs normally

- Will continue to run on 2 phases but fails to take load.

- Continue to run on 2 phases, draws excess current, winding likely to burn.

- Run at reduced speed and can take only 2/3 of the load.

- You have to change the defective induction motor of a machine. The available motor is of a higher capacity than the existing one. What is the effect of using a higher capacity motor?

- The new motor will be overheated

- The new motor is better suitable for this work

- The efficiency and the power factor will become low

- The new motor has a longer useful working life and efficiency

- The synchronous speed of an induction motor refers to the speed of the.......

- rotor

- armature

- rotating magnetic field

- value marked on the name plate

- The synchronous speed formula is:

- Ns = 120fP

- Ns = 120fp

- Ns = 120Pf

- Ns = Pf120

- Calculate slip of an induction motor if it has 4 poles, 50Hz, rotor speed is 1400 rpm.

- 6.6%

- 6.0%

- 5.5%

- 5.0%

- In a 3-phase motor the rotating magnetic field is displaced at an angle of .......

- 30°

- 50°

- 90°

- 120°

- Which one of the following 3 phase motors is NOT self starting?

- Synchronous motor

- Squirrel cage induction motor

- Wound rotor motor

- Double cage motor

- The motor suitable for loads where constant speed is required is.......

- induction motor

- synchronous motor

- capacitor motor

- universal motor

- If No. of poles is ‘p’ and frequency ‘f' then the Synchronous speed ‘Ns  is given by………..

- Ns = 120Pf

- Ns = 120fP

- Ns = Pf120

- Ns = 120Pf

- The magnetic field produced in the stator of a three phase induction motor travels at........

- rotating speed.

- asynchronous speed

- synchronous speed

- slip speed

- The fractional slip of a 3 phase motor formula is.......

- S = Ns-NrNs

- S = Ns-NrNr

- S = Nr-NsNr

- S = Ns-Nr

- The amount of torque produced by 3-phase induction motor is......

- T = k.ϕ . IR cos.ϕ

- T = ϕ . IR cos.ϕ

- T = ϕ . IR sin.ϕ

- T = k.ϕ . IR sin.ϕ

- For a small capacity induction motor, the type of starter used is......

- Star-delta starter

- DOL starter

- Auto transformer starter

- Rotor resistance starter

- In a 3-phase induction motor started by a DOL starter the short circuit current is protected by.....

- overload relay

- no volt coil

- back-up fuses

- protective earth conductor

- It is advisable to avoid direct on line starting of induction, motor having, a capacity of more than 3 HP because......

- the starting torque is very low

- the motor takes five to seven times of its full load current.

- it will pick up very high speed

- it will run in the reverse direction

- On switching ON the starter the magnetic contactor vibrates heavily, the causes may be……..

- high voltage

- dirt in the armature contact surface.

- low voltage and single phasing

- low voltage and dirt on the armature contact surface.

- in the manual star delta starter, the stop button connection is in series with the……..

- no-volt coil

- overload relay contacts

- no-volt coil and load relay contacts

- no-volt coil and start button.

- You are working with a fully automatic star delta starter. You notice that when start switch is pressed the motor starts in star, but does not change to delta. in the above case the trouble will be in.......

- delta contactor

- timer and its circuit

- star contactor

- line contactor

- The handle of the Star/Delta starter shown in the figure does not hold at delta position. The reason for the above is that the.........

- hold on coil circuit is open at delta position

- hold on coil circuit will not get the potential difference in delta

- motor terminal connection at delta position is not correct

- connection is such that the hold on coil circuit will function at star but not in delta

- A 3-phase 15 HP, 415 V, 50 Hz squirrel cage motors star delta starter cables are found to be defective. Copper cables are replaced by same current capacity aluminium cables through the same metal conduit pipe. When the motor starter, connections were properly done and the motor started, the fuses have blown off. the reason for the above is.....

- aluminium cable insulation is weaker when compared to copper cable

- aluminium cables may not have the same voltage grading as copper babies.

- overall size of the aluminium cables are more than copper cables and hence insulation damaged white drawing

- over heat provided by aluminium cables is the cause of failure of insulations and short.

- Rotor resistance starters are used with wound rotor induction meter. When the motor is running the resistance in, starters are.........

- shorted

- open circuit

- connected in delta

- connected in star

26.Synchronous Motors

- An electric motor in which both the rotor and stator fields rotates with the same speed is called .................. motor

- D.C.

- charge

- synchronous

- universal

- When load on a normally excited synchronous motor is increased, its power factor tends to

- approach unity

- become increasingly lagging

- becomes increasingly leading

- remain unchanged

- If the field of a synchronous motor is under excited, the power factor will be

- lagging

- leading

- unity

- more than unity

- in a synchronous motor, the rotor copper losses are met by

- motor input

- armature input

- supply lines

- D.C. source

- Synchronous capacitor is

- an ordinary static capacitor bank

- an over-excited synchronous motor driving mechanical load

- an over excited synchronous motor running without mechanical load

- none of the above

- The maximum valve of torque angle in a synchronous motor is .......... degrees  electrical

- 45

- 90

- between 60 and 90

- below 60

- The effect ct increasing load on a synchronous motor running with normal excitation is to

- increase both its Ia and p.f.

- decrease Ia but increase p.f.

- increase Ia but decrease p.f.

- decrease both Ia and p.f.

- The direction of rotation of a synchronous motor can be reversed by reversing

- current to the field winding

- supply phase sequence

- polarity of rotor poles

- none of the above

- A synchronous machine is called a doubly excited machine, because

- it can be over excited

- it has two sets of rotor poles

- both its rotor and stator are excited

- it needs twice the normal exciting current

- in a synchronous motor, damper winding is provided in order to

- stabilize rotor motion

- suppress rotor oscillations

- develop necessary starting torque

- both B and C

- While running a synchronous motor is compelled to run at synchronous speed because of

- damper winding in its pole faces

- magnetic locking between stator and rotor pores

- induced e.m.f. in rotor field winding by stator flux

- compulsion due to Lenz’s law

- in a synchronous motor, the magnitude of stator back e.m.f. Eb depends on

- speed of the motor

- load on the motor

- both the speed and rotor flux

- D.C. excitation only

- The V-curves of asynchronous motor show relationship between

- excitation current and back e.m.f,

- field current and power factor

- D.C. field current and AC. armature current

- armature current and supply voltage

- When running under no load condition and with normal excitation armature current la drawn by a synchronous motor

- leads the back e.m.f. Eb by a small angle

- is large

- legs the applied voltage b a small angle

- tags the resultant voltage ER by 90°

- A synchronous motor running with normal excitation adjusts to load increases essentially by increase in its

- power factor

- torque angle

- back e.m.f.

- armature current

- The angle between the synchronously rotating stator flux and rotor poles of a synchronous motor is called ............. angle.

- synchronizing

- torque

- power factor

- slip

- A synchronous motor is started on:

- lagging power factor only

- leading power factor only

- both types of power factors i.e., lagging and leading

- none of the above.

- The motor whose rotor and rotating magnetic field of the stator rotate at the same speed is called:

- induction motor

- slip-ring induction motor

- synchronous motor

- universal motor

- A synchronous motor always works at :

- synchronous speed

- lesser than synchronous speed

- more than synchronous speed

- none of  the stated above

- Which one of the following characteristic is NOT applicable to synchronous motors?

  - runs at constant speed at all loads.

  - suitable to supply only mechanical load

  - can also be used for PF improvement

  - not self starting.

27. Single Phase Motors

- The type of the motor that does not have a commutator is :

- repulsion motor

- induction motor

- d.c. shunt motor

- a.c. series motor

- The rotor of a split-phase motor receives its power from :

- the line voltage

- slip-rings

- a centrifugal switch

- magnetic induction

- The induction motors are designed for :

- three phases

- two phases

- single phase

- all the above

- The direction of rotation of a capacitor start motor can be reversed by :

- interchanging the supply connections

- reversing the connections of the auxiliary winding (starting winding) only

- reversing the connections of either the running winding or the starting winding

- reversing the connections of the running winding only

- What is the synchronous speed of 6 poles motor running at 50 Hz frequency supply ?

- 1000 R.P.M.

- 1200 R.P.M.

- 6000 R.P.M.

- 7200 R.P.M.

- The-capacitance of a small (less than 1 HP) single-phase motor will be in the order of :

- hundreds of farads

- tens of farads

- tens and hundreds of pico farads

- tens and hundreds micro farads

- A repulsion-start, induction-run motor runs as a squirrel-cage motor only when :

- the slip-rings are short-circuited

- the commutator is short-circuited

- the compensating winding is short-circuited through the brushes

- a squirrel cage winding is provided

- The direction of rotation of a repulsion motor can be changed by changing :

- supply connections

- stator connections

- brush positions

- rotor winding connections

- Which one of the following motors will have the lowest starting torque :

- universal motor

- shaded pole motor

- capacitor-start capacitor-run motor

- repulsion-induction motor

- The purpose of the shaded coils in a shaded-pole motor is to :

- produce rotating magnetic field

- reduce friction loss

- reduce rough commutation

- reduce armature reaction

- Which one of the following motors can reverse its direction of rotation by simply reversing the supply connections ?

- Capacitor-start capacitor-run motor

- Split phase motor

- Shaded pole motor

- None of the above

- What is the function of a centrifugal switch?

- To disconnect the running winding

- To disconnect the starting winding when the motor attains 75% of its rated speed

- To disconnect the rotor resistors

- None of the above

- A D.O.L. starter is necessary for a single phase motor of more than one H.P. for:

- starting the motor

- stopping the motor

- providing overload protection to the motor

- all the above three reasons

- Capacitor-start and capacitor-start capacitor-run motors employ :

- ceramic capacitors

- mica capacitors

- paper or non-polarised electrolytic capacitors

- Styroflex capacitor

- The rotational speed of a universal motor can be controlled by :

- series voltage regulator

- tapped field method

- centrifugal switch method

- any one of the above stated methods

- In a capacitor-start induction-run motor, a capacitor is used for :

- suppressing the sparking produced across the centrifugal switch

- leading the current in main winding (running winding)

- improving the power factor

- phase splitting

- Which one of the following single-phase motor perform with better power factor ?

- Universal motor

- Repulsion motor

- Capacitor-start motor

- Capacitor-run motor

- Which type of motor is generally used in a portable electric blower ?

- Repulsion motor

- Universal motor

- Capacitor-run motor

- Hysteresis motor

- It a winding makes electrical contact with the stator core, the winding is said to be :

- short-circuited

- open-circuited

- closed

- grounded

- A capacitor-start induction-run, single-phase motor has its capacitance replaced by an inductor of equivalent reactance value. it the motor is connected to the supply, it will

- run in the reverse direction

- not start with load

- run with low torque taking a lower current

- start but will not take any load

- A capacitor-start, single-phase motor will usually have a P.F. of :

- 0.8 leading

- 0.8 lagging

- unity

- none of the stated above

- The most economical method of starting a single phase motor is :

- split-phase method

- inductance start method

- capacitance start method

- resistance start method

- Which of the following motors will give highest starting torque?

- Capacitor start motor

- Universal motor

- Shaded pole motor

- Reluctance motor

- A ceiling fan of 1400 mm sweep (56 inches) will have a motor of rating :

- 10 to 15 watts

- 50 to 70 watts

- 120 to 180 watts

- 250 to 500 watts

- One of the reasons for noise in a single phase capacitor motor is :

- short-circuited main winding

- short-circuited starting winding

- loose connections in overload relay circuit

- loose rotor bars

- A single-phase universal motor fails to start because:

- there is no supply voltage

- the supply voltage is more than the capacity of the motor

- the brush length is reduced by 10%

- there is no load on the motor at the time of starting

- The direction of rotation of a universal motor is changed by :

- interchanging the field connections only

- interchanging the connections only

- interchanging either the field or armature connections

- none of the above

- A universal motor gives electric shock to the operator. The possible reason is :

- earthed field winding

- short-circuited armature winding

- motor is run above its rated speed

- applied voltage is high

- Which of the following single phase motors will be cheapest?

- Capacitor-start motor

- Capacitor-run motor

- Repulsion-start induction-run motor

- Double capacitor motor

- What is/are the shortcomings of repulsion motors?

- Variation of speed with load

- Low power factor

- Tendency to produce sparking at the brushes

- All the above

- For the same rating the size of the single phase induction motor is about ............ that of the corresponding three phase induction motor

- three times

- the same as

- 1.5 times

- one-third

- For speeds higher than 3000 R.P.M., the machine used is

- induction motor

- synchronous motor

- universal motor

- none of the above

- Universal motor runs at

- higher speed with DC. supply and with less sparking

- higher speed with AC. supply and with less sparking

- same speed with both AC. and DC. supplies

- higher speed with AC. supply but with increased sparking at the brushes

- in a repulsion motor energy is transferred from stator field winding to the rotor

- conductivity

- inductively

- conductively as well as inductively

- any of the above

- A domestic mixer uses the following motor

- induction motor

- reluctance motor

- universal motor

- permanent magnet synchronous motor

- The electric motor generally used in house hold food mixers is

- universal motor

- shaded pole motor

- capacitor start motor

- none of the above

- For which of the following applications is the universal motor most suitable?

- Tape recorder

- Table fan

- Hand operated drilling machine

- Record player

- Synchronous speed of an inductor can be increased by

- reducing mechanical friction

- increasing supply voltage

- increasing number of poles

- increasing supply frequency

- Rotor resistance speed control is used in

- squirrel cage induction motor

- synchronous motor

- slip ring induction motor

- D.C. shunt motor

- During plugging of an induction motor

- phase sequence is reversed

- a D.C. source is connected to stator

- one phase is open circuited

- power is fed back to mains

- Capacitor in a single phase induction motor is used for

- improving the power factor

- improving the starting torque

- starting the motor

- reducing the harmonics

- Capacitor start and capacitor run motors are used in

- hoists

- rolling mills

- mining

- refrigerator

- A shaded pole single-phase induction motor can be used for

- wet grinders

- food mixies

- tape recorders

- personal computers as cooling fan

- if a single phase induction motor runs at a speed lower than the rated one, the  most likely defect is

- improper size fuses

- worn out bearings or low voltage or overload.

- open circuit in the winding

- short circuit in the winding

- The suitable motor for domestic fans is

- D.C. shunt motor

- D C. series meter

- single phase induction motor

- synchronous motor

- The washing machines generally employ

- single phase series motor

- resistance split phase motor

- shaded pole motor

- hysteresis meter

- For toys the preferred motor is

- shaded pole motor

- capacitor motor

- reluctance motor

- universal motor

- Reversal of speed in single phase motor is obtained by

- D.C. braking

- plugging

- phase reversal

- mechanical device

- Single phase induction motors are more noisy than 3-phase induction motors because of,

- inherently low power rating

- squirrel cage rotor design

- pulsating electrical power input

- constant mechanical power output

- The direction of rotation of a capacitor start single phase induction motor can be reversed by interchanging terminals of

- capacitor

- supply

- centrifugal switch

- starting winding

- The type of single phase induction motor having the highest power factor at full power load is

- shaded pole type

- split phase type

- capacitor start type,

- capacitor run type

- An electric motor gets overheated. This may be due to..................

- over loading,

- shorted stator winding

- worn out bearings

- low or high voltage

- If a single phase motor fails to start, but gives humming noise this may be due to.............

- low voltage

- high voltage

- shorted stator winding

- blown fuses

- If a single phase-motor fails to start, it may be due to ...............

- blown fuses

- open circuit in auxiliary winding

- open circuit in main winding

- any of the above causes

- it a single  phase induction motor runs at a speed lower than the rated one, the most likely defect is....................

- improper size fuses

- worn out bearings or low voltage or over load

- open circuit in the winding

- short circuit in the Winding

- When a ceiling tan using a capacitor run motor is switched on, it hums but does not run when it is driven by 'some external means, it runs in the direction in which it was made to run. The possible cause is.................

- open circuited capacitor

- blown fuses capacitor

- short circuited capacitor

- any of the above

- Single phase motor whose direction of rotation cannot be changed by changing the connection is a…….

- Capacitor start and run motor

- Double capacitor motor

- Repulsion induction motor

- Universal motor.

- The rating of fuse for the protection of single phase motors should be equal to three times the........

- running current

- starting current

- no load current

- full load current

- As per BIS 1709 1984 recommendation, what is the ohmic value of insulation test conducted between the shorted capacitor terminal and the metal can, when measured by a 500 V megger?

- 100 ohms

- less than 50 kilo ohms

- not less than 100 mega ohms

- more than to mega ohms

- The type of capacitor used in capacitor start motor is.....................

- electrolytic capacitor

- ceramic capacitor

- paper capacitor

- mica capacitor

- The position of shaded ring with respect to main pole of the shaded pole motor determines.........

- speed of motor

- direction of rotation

- efficiency

- torque

- In shaded pole single phase motor the revolving field is produced by ............

- capacitor

- regulator

- shading ring

- winding

- In repulsion start induction run motors, while the motor is running the brushes will..........

- be lifted from commutator and are shorted through a mechanism

- be lifted from commutator and the commutator is shorted by a mechanism

- remain on“ commutator with their terminals shorted

- remain on commutator. and AC supply is connected through a mechanism.

- Current drawn from the supply type by a permanent capacitor motor will be

- lagging the voltage by less than 90°

- lagging the voltage by 90°

- leading the voltage by 90°

- inphase with the voltage

- The direction of rotation of a permanent capacitor motor can be reversed by interchanging the ............

- centrifugal switch terminals

- auxiliary winding terminals

- capacitor terminals

- supply terminals

- In an universal motor, the armature reaction can be reduced by ...........

- salient pole

- compensating working armature

- shifting the brush position

- equalizer rings

- One of the operating characteristic of universal motor is .....

- constant speed at all loads

- high speed at full load

- constant speed at full load

- varying speed depending on load

- Single phase capacitor Start induction run motor is used in wet grinders. How  will you connect the capacitor in wet grinder motor?

- series with starting winding

- series with: running winding

- parallel with starting winding

- parallel with running winding

- The stator winding of any type of an induction motor burns immediately, when its rotor is not rotating due to the absence or back e.m.f. during an A.C. supply. Which motor stator winding will not burn, even the rotor of the motor does not rotate and kept ideal for a long time or required time?

- shaded pole motor

- slip ring motor

- capacitor motor

- stepper motor

- The function of capacitor in a capacitor start induction run motor is for ..............

- suppressing spark across centrifugal switch

- making current to lead in main winding

- improving, the power factor

- splitting the phase for producing the rotating magnetic field

28. Winding

- The maximum safe temperature’ for class ‘F’ insulating materials is :

- 105°C

- 120°C

- 130°C

- 155°C

- The type of slot used in small DC. armature is :

- open type with tapered slot

- open type with dovetail ,

- semi-enclosed type

- closed type

- The back pitch in wave and lap winding is equal to the :

- coil pitch

- average pitch

- commutator pitch

- resultant pitch

- in wave winding the resultant pitch should be approximately equal to :

- double the back pitch

- double the front pitch

- the sum of the back and front pitches

- the difference between the back and front pitches

- in a 8-pole duplex-lap winding, the total number of parallel paths will be

- 1

- 4

- 8

- 16

- To detect a short-circuit between the turns of the same coil in an armature winding, the method recommended to be used is the:

- ohm meter

- test lamp

- voltage drop

- Wheatstone bridge method

- The lap and wave winding differ basically from each other in the.

- shape of the two winding coils

- commutator connection

- method of placing the coils in the slots

- power output.

- in the armature winding the wedges are made of:

- fibre

- bamboo

- milinex

- all the three stated above

- Three types of slots are provided in armature cores which are :

- open, closed, semi-open

- open, closed, semi-enclosed

- round, square, triangular

- none of these

- varnish coating is done on winding in order to:

- increase the insulation of the windings

- make the windings vibration less

- provide an additional insulation layer on the windings

- all the three stated above

- Voltage drop test can be adopted:

- for short-circuit test only

- for open-circuit test only

- for earth fault test only

- for all the three ab0ve stated tests

- In the open coil Winding, the coil leads are kept.

- open-circuited

- short-circuited

- independent

- nothing of these

- the portion of a coil which remain out of the stator cores slot is called:

- active side

- inactive side

- coil lead

- unuseful side

- It is  not a type of A. C. winding.

- basket winding

- concentric winding

- skein winding

- Gramme winding.

- The three windings in a 3-phase induction-motor should be installed at a distance of :

- 90 electrical degrees

- 120 electrical degrees

- 360 electrical degrees

- 60 electrical degrees

- A distributed winding is one in which :

- the no. of coils per pole per phase is one

- the no. of coils per pole per phase is more than one

- the no. of coils per phase is one

- the no. of coils per phase is more than one

- For rewinding of high voltage 3-phase machines, the preferable type of winding is

- chain winding

- involute coil winding

- concentric winding

- drum winding

- A single layer winding is one in which :

- two coil sides are placed in each slot

- three coil sides are placed in each slot

- one coil side is placed in each slot

- none at these

- lithe slot angle is 30° electrical degrees, the main winding (running winding) is started in slot no.1 then the starting winding should start in :

- the same slot

- slot number 2

- slot number 3

- slot number 4

- What is the shape of the winding shown in the given figure?

- Mesh shaped coil winding

- Diamond mesh shaped coil winding

- Concentric winding

- Basket winding

- in metric system, the size of super enamelled copper wire is given in:

- gauge number

- diameter in mm

- resistance in ohms

- cross-sectional area in mm2

- The number of poles of a capacitor type fan motor is 8 and the number of coils is 16 (equal number for starting and running windings)

This type of winding is :

- half coil winding

- whole coil winding

- Single layer winding

- double layer winding

- if the number of poles is 4 and the number of slots is 24 then the pole pitch will be :

- 8 slots

- 6 slots

- 4 slots

- 2 slots

- Data pertaining to ‘end-connection group-connection’ and ‘lead-connection should be taken:

- before dismantling the meter

- before removing the ‘windings

- after removing the windings

- after taking coil dimensions

- While skein winding of a motor having concentric coils, the-safest procedure is :

- to wind all the coils and then start inserting them in, the slots.

- to complete the and connections by proper soldering within a concentric coil group.

- to complete the coil group connections for one winding by soldering and insulating with sleeves.

- to insert first set of concentric coils to see the correctness of coil’s sizes

- In a single layer distributed winding if the number of coils is 24 then the number of slots will be :

- 8

- 16

- 24

- 48

- No. of coils per phase is equal to :

- total no.  of coilsno of phases

- total no.  of coilsno of phases ×no of pole pairs

- no.  of slotsno of poles

- total no.  of coils/phaseno of poles

- To calculate the slot-distance in degrees for a stator

- total electrical degreesno of slots

- no of slotstotal electrical degrees

- no of slots90° ×no.of slots

- 90° ×no.of slotsno of slots

- The type of slots used in armature of a high voltage d.c. motor is :

- closed type

- semi-enclosed type

- open type with dovetail

- open type with tapered slot

- in distributed winding, all the coils will have the same:

- pitch

- width

- length

- thickness

- The pitch factor Kp in alternator is

expressed as Kp = cosα2 where angle

α represents...........

- α = No of stator slotsno of poles

- α = different in pitchfull pitch × 180°

- α = 180°number of slot per pole

- α = 180°number of pole

- Pole pitch is calculated by...............

- No of polesTotal No of slots

- Total No of slotsNo of poles

- Total No ofcoilNo of poles

- Total No of slotsTotal No of coil

- Total No. of coils per phase per pole is………

- Total No ofcoilNo of phases

- Total No ofcoilNo of phases×No of poles

- No of phasesTotal No ofcoil

- No of phases×No of poleTotal No ofcoils

- Number of coils per phase is equal to………

  - Total No ofcoilNo of phases

  - Total No ofcoil/phasesNo of poles

  - Total Number ofcoilNo of phases×Number

  - Total No of slotsNo of poles

- Laying the coils - clock wise, the connections to, the commutator, segments will proceed anti-Clockwise, then this type of winding is called as……..

- lap winding.

- wave winding

- progressive lap or wave winding.

- retrogressive lap or wave winding

- Laying the coils clockwise, the connections to the commutator segments will also proceed clock wise, then this type of winding is called as..........

- retrogressive lap or wave winding

- progressive lap or wave winding

- lap winding

- wave winding

- Phase displacement in terms of slots in a stator of 3-phase is................

- 120°slot distance in degrees

- 360°slot distance in degrees

- slot distance in degrees360°

- slot distance in degrees120°

- The distance measured in terms of armature slots or armature conductors between two sides of a coil is...............

- back pitch

- coil pitch

- front pitch

- pole pitch

- The distance measured in terms of armature conductors which a coil advances on back of the armature is called as.............

- front pitch

- coil pitch.

- back pitch

- pole pitch.

- The end lead of the coil 1 is connected to the beginning of a coil placed at a distance equal to one pole pitch is called.............

- simple lap winding.

- simple wave winding

- duplex lap Winding

- duplex wave winding.

- The end lead of coil 1 is connected to the beginning lead of adjacent coil (coil 2) through the commutator segments is………….

- simple lap winding.

- duplex lap winding.

- simple wave winding.

- duplex wave winding

- What are the two methods by which polarity of field (poles) of a winding can be checked?

- By internal glower and external glower

- Magnetic compass and search coil.

- Magnetic compass and internal glower

- Search coil and external glower

- When a rewound armature is rolled on the surface of a properly leveled surface plate, the armature stops at the same position of its periphery for each rolling, the portion of armature touching the surface plate is regarded to have……….

- Higher weight than the diametrically opposite portion.

- Lesser weight than the diametrically opposite portion.

- Equal weight to that of the opposite portion

- Higher weight to that of the opposite portion.

30. Illumination

- A 250 V lamp has a total flux of 3000 lumens and takes a current of 0.8 ampere from 250 V mains. Its luminous efficiency is

- 12 lumens per watt

- 9.6 lumens per watt

- 15 lumens per watt

- 240 lumens per watt

- A lamp has a total luminous flux of 1570 lumens. its mean spherical candle power is

- 1570

- 500

- 250

- 125

- Colour of light depends upon

- velocity of light

- frequency

- wave length

- both (b) and (c)

- Glare is caused due to

- excessive luminance

- excessive lighting contrast in the field of vision

- either (a) or (b)

- none of the above

- A perfect diffuser surface is one that

- transmits all the incident light

- absorbs all the incident light

- diffuses all the incident light

- all of the above

- The solid angle subtended at the centre of a hemisphere of radius “r” will be

- 2πr

- 2π

- 4π

- 2πr

- The illumination at a surface due to a source of light placed at a distance “d’ from the surface varies as

- 1d2

- 1d

- d

- d2

- The illumination at various points on a horizontal surface illuminated by the same source varies as

- cos Ɵ

- cos2Ɵ

- cos3 Ɵ

- 1cos Ɵ

- Which of the-following colour has wave length between blue and yellow?

- violet

- orange

- red

- green

- The study of illumination involves the :

- study of lighting and wiring systems

- designing-of a lighting system

- calculating of wiring materials required in wiring

- study of photometry

- Energy radiated continuously in the from of light-waves is called.

- luminous flux

- luminous-intensity

- lumen

- illumination

- The luminous-flux emitted in unit solid angle by a light source having a luminous-intensity of one candela is called:

- illumination

- luminance

- one lumen

- one flux

- The illumination at a point of a surface is the luminous-flux per unit area of the surface and its unit is :

- flux

- lumen

- candela

- watt

- The illumination of a surface is :

- directly proportional to its distance from the source

- inversely proportional to its distance from the source

- directly proportional to the square of its distance from the source '

- inversely proportional to the square of the distance from the source

- The illumination required for a workplace. where fine work is performed will be in the order of ......

- 500 lumens/sq. metre

- 1000 lumens/sq. metre

- 1500 lumens/sq. metre

- 2000 lumens/sq. metre

- The illumination required for a workplace where fine work is performed will be in

the order of ...............

- 1000 lumens/ metre

- 1000 lumens/sq. metre

- 1500 lumens/sq metre

- 2000 lumens/sq metre

- The illumination required for a workplace where rough or unskilled work is performed will be in the order of...............

- 100 lumens/sq. metre

- 150 lumens/sq. metre

- 200 lumens/sq. metre

- 250 lumens/sq. metre

31. Sources of Light

- The lumen output of a 230 volts, 100 watts gas-filled lamp is approximately:

- 700

- 1000

- 1200

- 1500

- The principle of working of a H.P.M.V. lamps is that

- the radiation from the discharge excites a fluorescent material

- the radiation is produced directly by the discharge

- the radiation is produced by the argon gas

- very high voltage is needed to start the discharge

- Low pressure in a fluorescent tube is obtained by :

- high starting voltage

- high current density

- low current density

- low starting voltage

- When fluorescent temps are used to replace incandescent bulbs usually:

- greater illumination occurs

- less illumination occurs

- only one colour can be obtained

- more shadow occur, but there is less eye strain

- If the starter of a glowing tube-light is withdrawn then the tube-light.

- will become OFF

- will continue to glow

- filaments will be damaged

- will glow intermittently

- The inner tube of a H.P.M.V. lamp is filled with:

- Argon gas

- Halogen gas

- Helium gas

- Nitrogen gas

- H. R.M.V. lamps are available in a rating of:

- 40, 60 watts

- 100, 200 watts

- 80,125 watts

- 150, 300 watts

- Voltage required to operate a neon-sign tube depends upon:

- diameter of the tube

- gases in the tube

- length of the tube

- thickness of the tube

- What is the advantage of a parallel twin-tube, fluorescent lamp circuit?

- The true power consumption of the circuit is almost zero

- A very small current flows in both the branches

- The power factor of the circuit becomes 1 and the tube-lights do not flicker

- The total current drawn by the circuit is almost zero

- Sodium vapour lamps are particularly suitable for:

- street, lighting

- storage yards

- foggy areas

- all the three stated above

- Gas-discharge lamps take some time in illuminating fully. Which of the following lamps takes comparatively more time in illuminating fully?

- Tube lamp

- Mercury vapour lamp

- Sodium vapour lamp

- Néon lamp

- What is the temperature at which a gas filled tungsten filament lamp operates?

- 1035°C

- 1200°C

- 1500°C

- 2300°C

- When a fluorescent lamp shows a dense blackening at each end, it could possibly mean that the:

- gas filled in the lamp was wrong

- lamp was started infrequently

- starter is working improperly

- lamp is  new

- The advantage of a preheat fluorescent lamp as compared to incandescent lamp is that it:

- has a faster illumination

- provides less illumination per watt

- generally has a longer life

- can be used for heating as well as lighting

- How many lamps of 3 volts rating are required to be connected in series to a supply of 200 V for satisfactory operation?

- 67

- 85

- 135

- 33

- The most popular type of fluorescent lamp for new installation is the :

- preheat lamp

- glow tube

- ballast lamp

- instant start lamp

- The type of fluorescent lamp that can be used in flashing circuits is the :

- ballast lamp

- preheat lamp

- instant start lamp

- rapid start lamp

- in a H P.M.V. lamp, the mercury vapour discharges at:

- a higher pressure

- a medium pressure

- a low pressure

- none of the above

- Most recent development in fluorescent lighting is the:

- rapid start lamp

- pre-heat lamp

- glow lamp

- instant start lamp

- In a high pressure mercury lamp, to vaporize all the mercury and for the lamp to reach full brilliance, it requires :

- 2 to 4 minutes

- 4 to 6 minutes

- 6 to 8 minutes

- 8 to 10 minutes

- The rating of I.C.D.P. switch used in a neon-sign tube should be:

- 15A, 250 V

- 15 mA, 230 V

- 15mA, 1000 V

- 15 A, 1000 V

- H.P.M.V. lamp gives :

- 30 lumens / watt

- 50 lumens / watt

- 60 lumens / watt

- 80 lumens / watt

- If on removing a defective starter, the fluorescent tube lights up, the fault lies in the :

- choke

- tube

- stater

- line switch

- A separate starter and socket is required for a fluorescent lamp. What is it called as?

- instant start

- Ballast

- Rapid start

- Preheat

- What is the purpose of the choke after the fluorescent tube is just, lighted to its full brightness?

- For supplying a higher operating current

- Smoothing the sinusoidal operating current

- Limiting the current flowing through the fluorescent tube filaments

- Heating the filaments continuously during operation

- As shown in the figure given below one part of starter is labelled as ‘A’. Which among the following is this part

- Base with pins

- Radio frequency suppressor

- Bimetallic switching contact

- sealed glass tube fitted with gas (Neon)

- There is some vibrating sound from the choke when a tube lamp is switched on. Which One of the following is the reason for this.

- Loose connection in the choke

- Loose core

- Loose winding turns

- Loose cover screw

- Name the part indicated as ‘X’ of fluorescents lamp shown in the figure

- Cathode filament

- Pins

- Argon gas

- Phospher coating

- The starter of a fluorescent tube lamp has been removed from its holder after the tube is lighted what Will happen to the function of the tube lamp?

- Tube lamp will begin to flicker

- Tube lamp will go out immediately

- Tube will operate normally

- Tube will-begin to-hum very much

- The function of choke in tube light is to induce

- low voltage and limit current

- low voltage and increase current

- high voltage and limit current

- high voltage and increase current

- The friction of choke in tube light is to induce...........

- low voltage and limit current

- low voltage and increase current

- surge voltage and limit current

- surge voltage and increase current

- It a choke is not connected to an A.C. tube’s circuit than the tube :

- will continue to glow

- will get fused

- will not glow

- will glow intermittently

- if the starter is withdraw from the circuit after glowing the tube then :

- the tube will be fused

- the tube will not emitt light

- the tube will continue to glow

- the tube will start ﬂickering

- As per lE regulations when calculating the incandescent lamp power for load purpose, every lamp holder is taken as………..

- 40 watts

- 60 Watts

- 75 watts

- 100 watts

- When two 250 V, 100 W bulbs are connected is series to 250 V supply, they will give.........

- bright light

- dim light

- one dim and the other bright light

- no light

- Gas discharge lamps lights up to full brightness after a time delay.

Which one of the lamps mentioned below has more time delay when compared to other lamps?

- Tube lamp

- Mercury vapour lamp

- Sodium vapour lamp

- Neon lamp,

- Which one of the lamps mentioned below has more time delay when compared to other lamps?

- Tube lamp

- Mercury vapour lamp

- Sodium vapour lamp

- Néon lamp

- Name the part indicated as ‘X’ in sodium vapour lamp in the figure shown below?

  - Base

  - Sodium vapour

  - Arc tube

  - Electrode

- What is the colour of the light-emitted by sodium vapour lamp?

- bluish light

- yellowish light

- reddish light

- greenish light

- The ignition voltage of sodium lamp varies from..........

- 400 to 600 V

- 400 to 550 V

- 400 to 500 V

- 400 to 450 V

- Name the part indicated as ‘X’ of gas discharge lamp shown in the figure?

- glass shell

- lead in wire

- electrodes

- gas and metal

- In a high pressure mercury vapour lamp the mercury lamp discharges ..........

- at every high pressure

- at a high pressure.

- at a medium pressure

- at a low pressure

- In mercury vapour lamp, a resistor is connected in series with auxiliary electrode. What is the purpose of resistor connected to it.

- To limit the power factor

- To limit the starting current

- To limit the voltage

- To limit the energy

- The life of MA (mercury argon) type lamps is..........

- 9000 hrs

- 8500 hrs

- 8000 hrs

- 7500 hrs

- Upon which one of the following, the current flow through the Neon-sign tubes depends?

- The diameter of the tube

- The gases in the tube

- The length of the tube

- The colour of the tube

- Colour of the light from the Neon- sign lamp depends on which one of the following..........

- Colour of the building

- Current ﬂowing through the tube

- Temperature of the tube

- Gases in the tube

32. Decoration Lighting

- The direct lighting scheme is most efficient, but is liable to cause

- glare

- hard shadows

- monotony

- both A and B

- Total flux required in any lighting scheme depends inversely on

- surface area

- space height ratio

- illumination

- coefficient of utilization

- To avoid glare

- object should be viewed from a distance

- object should be move constantly

- object should be viewed from a close vicinity

- viewer should move constantly

- The depreciation factor depends on

- ageing of the lamps

- type of work carried out at the premises

- lamp cleaning schedule

- all of the above

- Utilisation factor depends on

- size of the room

- space height ratio of the lamps

- colour of walls/Ceiling

- all of the above

- Glare is reduced by lamp

- increasing the mounting height of the

- using diffusers

- using reflectors to cut-off the light at certain angle

- all of the above

- The lamp is provided with a reflector in order to

- avoid glare

- provide better illumination

- protect the lamp

- all of the above

- Depreciation factor (maintenance factor) takes account of :

- depreciation in the capital cost of the installation

- effects of dust, dirt and typeset lamps used?

- effects of dust, dirt and fall in light output of the lamp during its life

- condition of the room and the type of lamp used

- To compensate the lower value of cold resistance of the filament, which one of the following is connected in series with the serial set?

- NTC thermistor

- PTC thermistor

- Voltage dependent resistor

- Flasher

- In a serial set of bulb, the flasher is used. What is the function of the flasher?

- To break and make the circuit but it gives no light

- To break and make the circuit and it gives light

- To break the circuit only but gives no light

- To make the circuit only and also gives, light

- To operate 6 volt bulb in a 240 volt supply, number of bulbs required is ......

- 30

- 32

- 40

- 42

- In a decorative series lamp connection of 6 V lamp is connected to a 240 V, AC. supply. The number of lamps required is……….

- 40

- 35

- 30

- 25

33. Converter and Rectifier

- In which of the following applications DC. is essential?

- Illumination

- Machine shop motors

- Electric traction motors

- Electric home appliances.

- Which type of motor is used in large sized motor-generator sets?

- Squirrel-cage induction motor

- Wound-rotor induction motor

- Synchronous motor

- A.C. commutator motor

- The equipment used to convert D.C. into A.C. is called :

- a converter

- an inverter

- a rectifier

- a regulator

- A single machine which can generate A.C. or D.C. as well as A.C. and DC. both is called :

- M.G. set

- synchronous motor

- inverter

- rotary converter

- Modern types of equipments employ :

- metal rectifier

- mercury arc rectifier

- solid state rectifier

- tunger rectifier

- The main advantage of a M. G. set is this that:

- it does not require any maintenance

- the generated D.C. remains completely isolated from the A.C

- its efficiency is very high

- none of these

- A rotary converter may be of :

- single-phase type only

- three-phase type only

- single or three phase type

- both single and three phase type

- A rotary converter can be started by :

- d.c. supply

- a.c. supply

- mechanical power

- either of the above three

- D. C. supply is essential for the operation of :

- mercury vapour lamp

- arc lamp

- sodium vapour lamp

- fluorescent lamp

- Which of the following devices can convert A.C. power to D.C. power?

- Generator

- Alternator

- Transformer

- Rectifier

- Deterioration of a selenium rectifier is indicated by :

- sluggish operation

- a reduction in output voltage

- an increase in input current

- all the above factors

- Which of the following is known as a metal rectifier?

- Gas tube diode

- Silicon diode

- Copper oxide rectifier

- All the above stated.

- The size of transformer used in a battery charger/inverter depends upon :

- Charging voltage of the battery

- Charging current of the battery

- Charging power of the battery

- Charging energy of the battery.

- In which of the following application, DC is essential?

- Electric traction motor

- Electric home appliances

- Machine shop motors

- illumination

- in the motor generator set the type of AC, motor used is

- squirrel cage induction meter

- wound rotor induction motor

- AC commutator motor

- synchronous motor

- In which type of AC to DC converter losses are less and efficiency is higher?

- Motor generator set

- Rotary converter

- Mercury arc rectifier

- Metal rectifier

- Before installing a meter a proper foundation has to be provided. Where foundation bolts are embedded in concrete, their location must .be determined with great accuracy. Location of foundation bolts correctly in position an be done by...........

- Steel rule

- Template

- Try square

- Plumb bob

- One of the diodes of a bridge rectifier connected to 100 V 50 Hz AC input is found to be damaged. The DC output of the circuit is 90V. The minimum value of PIV of the diode required for replacement is.........

- 45 V

- 50 V

- 100 V

- 150 V

- In a full wave rectifier one of the damaged diodes is replaced by a diode of PIV equal to input AC voltage. The newly replaced diode in the circuit..........

- will fail early

- functions normally

- supplies low DC output

- supplies less current

- A 500 mA, 12 V eliminator power has bridge rectifier. The P.W. rating of diode is

- 9 V

- 12 V

- 15 V

- 18 V

- A three phase half wave rectifier has an AC input of 415 V 50 Hz. The ripple. frequency of DC output in pulses/sec is………..

- 50

- 100

- 150

- 300

- A 3-phase full wave rectifier has an AC input of 415 V, 50 Hz; The ripple frequency of DC output in pulses per second is.........

- 50

- 100

- 150

- 300

- You have to measure the ripple voltage of a DC power supply to confirm with that of the manufacturer’s specification. What is the instrument you need for his measurement?

- DC voltmeter

- AC voltmeter

- Oscilloscope

- Vacuum tube voltmeter (VTVM)

34. Semi-conductor Devices

- In amplifiers, transistors are used for

- enlarging small signals

- reducing a signal

- converting an ac. signal into d.c. signal

- all the above stated purposes

- An amplifier in which the shape of applied input signal does not change is called

- pulse amplifier

- linear amplifier

- non-linear amplifier

- none of these

- The emitter bypass capacitor used in CE type amplifier affects

- the input impedance

- the gain of the circuit

- the frequency of the amplified signal

- all the above stated factors

- In a CE amplifier which one of the following is an advantage of using an emitter bypass capacitor (CE)?

- increased gain

- Reduced input impedance

- Voltage gain changes throughout the input cycle

- All the above stated facts

- Small signal amplifiers are also known as………

- large power amplifiers

- medium power amplifiers

- low power amplifiers

- both medium and low power amplifiers

- What is the possible reason of excessive heating of the emitter bypass capacitor (CE)?

- Very high capacitance value of CE

- Low voltage rating of CE

- Reverse connection of CE

- As mentioned in points (b) and (c)

- Which of the following components fails most frequently in amplifiers?

- Resistor

- Transistor

- Electrolytic capacitor

- Ceramic capacitor

- What is the possible cause of zero collector voltage in the circuit shown in the given figure?

- Emitter lead is broken

- Base-emitter bias is zero

- + VCC is zero

- Transistor is defective

- The minimum value of input coupling capacitor for 20 Hz to 20 kHz amplifier should be at least

- 10 pF

- 1 uF

- 4.7 uF

- 10 uF

- In a CE amplifier, the voltage gain of the circuit is reduced to…. times of its maximum value (mid frequency) value at the cut-off frequency.

- 0.9

- 0.707

- 0.637

- 0.5

- The gain (voltage or current) of a CE amplifier is highest at

- high frequency

- low frequency

- mid frequency

- all frequency

- In a three stage cascade amplifier, if the gain of first stage is 50, of second stage is 10 and of third stage is 5, then what is the overall gain…...

- 27.689

- 55.378

- 65.0

- 2500

- Which one of the following points is true in case of direct coupled amplifiers?

- Poor low frequency response

- Higher cost

- Needs a ripple free power supply

- insensitive to temperature changes

- in RC coupled amplifier, the reduction in gain at low frequencies is, caused due to -

- bias resistors

- capacitive reactance

- inductive reactance

- transistor

- The advantage of impedance coupling (using a choke as collector load) is -

- low d.c. resistance and high impedance

- low d.c. resistance and low impedance

- high d.c. resistance and high impedance

- high d.c. resistance and low impedance

- Transformer coupling is not preferred in audio frequency amplifiers because —

- its cost is high

- it is bulky

- it does not give a uniform frequency response over the entire at range

- all the above stated drawbacks

- In a.f. amplifiers, the most commonly used method of coupling is –

- RC Coupling

- impedance coupling

- Transformer coupling

- Direct coupling

- The disadvantage of impedance coupling (LC coupling) is -

- impedance of choke varies with frequency

- choke is quite bulky and occupies large space

- expensive in comparison to RC coupling

- all the above stated disadvantages

- The first stage of an audio amplifier is designed to work as class ‘A’ due to its

- high efficiency

- high power gain

- low distortion

- all the above stated reasons

- Class ‘A’ amplifier conducts during -

- positive half cycle of the input only

- negative half cycle of the input only

- entire cycle of the input

- less than positive half cycle of the input

- if a portion of output voltage is fed back to the input of an amplifier at a phase difference of 180°, the phenomenon is called

- negative feedback

- positive feedback

- voltage feedback

- current feedback

- Which one of the following is not an advantage of negative feedback?

- Reduced noise

- Reduced distortion

- increased gain

- increased sound quality

- if the emitter resistor is not bypassed by a capacitor, the feedback circuit is called

- current feedback

- voltage feedback

- inverting voltage feedback

- fold back

- For stabilising the gain of an amplifier —

- positive feedback is used

- no feedback is used

- input voltage is varied

- negative feedback is used

- The automatic volume control (A.V.C.) system of a radio receiver incorporates…….

- positive feedback

- negative feedback,

- inverting feedback

- no feedback

- The technique of feeding the output of one amplifier stage to the input of the second amplifier stage is called……..

- joining

- coupling

- isolating

- bypassing

- A power amplifier is essentially -

- an impedance matching device

- a noise amplifier

- a current amplifier

- a voltage amplifier

- if the two half signals applied to a push- pull amplifier stage are not symmetrical, the result will be –

- increased gain

- distorted output

- reduced gain with high efficiency

- increased efficiency

- "the efficiency of a class ‘B’ amplifier is around

- 10% - 30%

- 30% - 50%

- 50% - 60%

- 100%

- A complementary symmetry amplifier is preferred over simple push-pull circuit because of –

  - low crossover distortion

  - no need of output transformer

  - its increased gain

  - its increased efficiency

- in class ‘B’ amplifier, the output current flows for –

- less than half input cycle

- more than half input cycle

- half input cycle

- entire input cycle

- A complementary symmetry class ‘B’ amplifier requires —

- two NPN transistors

- two PNP transistors

- one NPN and one PNP transistors

- two NPN and two PNP transistors

- Which one can be the possible reason for distorted output in an A.F. amplifier?

- Open coupling capacitor

- Open ON/OFF switch

- Very high input signal level

- Open bypass capacitor

- The suitable method of fault-finding in A.F. amplifiers is —

- random checking

- checking step by step

- trial and error checking

- logical checking

- A suitable equipment for fault-finding in A.F. amplifier is -

- signal generator

- signal tracer

- signal injector

- any one of the above stated equipments

- An A.F. amplifier is dead, what is the possible defect?

- Open speaker coupling capacitor

- Open output transistor

- Open emitter bypass capacitor

- All the above stated defects

- An Op-Amp. is designed to amplify -

- a.c. signals only

- d.c. signals OWY

- both as. and dc. signals

- pulse signals only

- A summing amplifier is basically a/an -

- voltage follower amplifier

- impedance matching amplifier

- Inverting amplifier with more than one input

- none 0f the above stated

- In an Op-Amp. the use of negative feedback increases

- the frequency response

- stability of amplification

- gain

- l none of the above stated

- The input and output resistances of an Op-Amp. are

- 2kὩ, 200kὩ

- 2 MὩ, 20Ὡ

- 200kὩ, 2kὩ

- 2Ὡ, 2MὩ

- Passive tone control circuits employ

- diodes and transistors

- capacitors and resistors

- diodes and resistors

- transformers

- Active tone control circuits employ

- diodes, transistors and I.C.

- capacitors and resistors

- transformers

- transformers and resistors

- lC CA 810 can be used as output amplifier in -

- radio receivers

- television receivers

- tape recorders

- all the above stated systems

- A stereo amplifier requires at least -

- one speaker

- two speakers

- four speakers

- eight speakers

- A stereo amplifier contains a special control named as -

- bass control

- treble control

- balance control

- volume control

- A P.A. system consists of —

- a power amplifier and loudspeakers

- an audio mixer and a tone control

- microphones and pre-amplifiers

- all the above stated systems ,

- In P.A. system cables should be used to minimize noise-

- insulated

- shielded

- short lengthed

- fibre

- Speaker used in Hi-Fi audio system for reproducing audio frequencies below 1 kHz is called —

- woofer

- tweeter

- squawker

- booster

- Voltage follower amplifier is used —

- for large voltage amplification

- for current amplification

- to avoid loading effect of the signal source

- all the purposes stated above

- An intercom is a —

- wireless internal communication system

- wired internal communication system

- mobile communication system

- video communication system

- Stereophonic system requires -

- two separate microphones

- two separate amplifiers

- two separate speakers

- all the above stated items

- A pre-amplifier is a m

- current amplifier

- voltage amplifier

- power amplifier

- video amplifier

- The process of limiting high frequencies is called -

- pre-emphasis

- de-emphasis

- tuning

- none of the above

- Speaker used for the reproduction of high audio frequencies above 3 kHz in Hi-Fi audio system is called……….

- tweeter

- woofer

- squaker

- booster

- if the colour bands of carbon resistors are Brown, Red, Red and Golden then its ohmic value is :

- 122 ohms ± 5%

- 1200 k ohms ± 5%

- 1200 ohms ± 5%

- 12.2 ohms ± 5%

- The resistor marked with ......... has an actual value of 15,00,000 ohms.

- 1k 5

- 1M5

- M15

- 1M 5k

- The cathode lead of 0A79 diode is identified by

- a colour dot at one end

- a colour band at one end

- a short lead length out of the two leads

- one lead is marked as ‘K’

- in the symbol of a PNP or NPN transistor, the lead marked with an arrow is :

- the collector lead

- the base lead

- the emitter lead

- the  earthing lead

- The three leads of a FET are called :

- emitter, base and collector

- source, gate, and drain

- B1, E, B2 ,

- cathode, gate and anode

- Which element is used as a semiconductor material?

- Copper

- Plastic

- Silicon

- Tungsten

- Valence electrons are :

- those which occupy the outermost shell

- those which occupy the innermost shell

- free electrons that move between atoms in the free space

- the total number of electrons in an atom less the no. of free electrons.

- Which is the element added to intrinsic semi-conductor material as an impurity to produce P-type semiconductor ?

- Antimony

- Phosphorus

- Bismuth

- Indium

- Name the element used as an impurity to produce N-type semiconductor.

- Antimony

- Platinum

- Gallium

- indium

- When forward bias is applied to a PN junction diode, it :

- increases the potential barrier

- decreases the potential barrier

- reduces the majority carrier current to zero

- reduces the minority carrier current to zero

- A silicon junction diode starts to conduct at a p.d. of :

- 0.7 V

- 0.3 V

- 1 V

- 0.1 V

- Which electrode of a CRT mainly controls the brightness of the image on the screen?

- Focusing electrodes

- Control grid

- Anode

- Cathode

- To which terminal of a CRO, the signal to be viewed is applied?

- V terminal

- H terminal

- Z-axis terminal

- External sync. terminal

- The screen or a CRO is set for 5 V/cm. if the height of a signal displayed is 6 cm then it represents :

- 5 volts P-P

- 15 volts P-P

- 30 volts r.m.s.

- 30 volts P-P

- The PIV (peak inverse voltage) rating of diodes in a full-wave bridge rectifier should be at least:

- 2 x peak AC. voltage

- peak AC. voltage

- 4 x peak AC. voltage

- 6 x peak AC. voltage

- The frequency of rectified AC. of a full-wave rectifier is :

- the same as the supply frequency

- twice the supply frequency

- zero

- half of the supply frequency

- A rectifier is used to change :

- A.C. to D.C.

- D.C. to A.C.

- tubes to transistors

- transistors to diodes

- A bridge rectifier employs :

- two diodes

- four diodes

- a centre tapped transformer and two diodes

- only half of the input voltage

- The filter action in a DC. power supply can be improved by :

- adding capacitance

- subtracting capacitance

- removing capacitance

- using very low value of capacitance

- The ripple present in the output of a rectifier is defined as :

- varying direct current

- another name of alternating current

- the varying positive and negative components

- filtered direct current

- The connection of a zener diode in a circuit is always:

- forward-biased

- in series

- reverse-biased

- in parallel,

- The value of current in a zener diode is :

- limited by the external circuit resistance

- determined by the zener voltage

- totally independent of temperature

- always in microampere range

- The current amplification (alpha) is calculated by:

- IC/IE

- IC/IB

- IB/I/C

- IB/IE

- The CE amplifier is most commonly used because of:

- less power gain

- less output voltage

- more power gain

- low cost

- When a transistor is used as a switch, the terminals that correspond to the ON/OFF switch terminals are:

- collector and emitter

- collector and base

- emitter and base

- base and collector

- Semiconductors are not conductor nor insulators. One method of increasing the conductance of a semiconductor is to :

- remove impurity atoms

- add impurity atoms

- add pure atoms

- use voltage

- What is the method of biasing used in the amplifier circuit shown in the given figure?

- Fixed bias

- Collector to base bias

- Emitter bias

- Self-bias

- Voltage stabilizers are having auto transformer with tappings or step less variation depending upon the type of stabilizer. Auto transformer in a servo stabilizer is of :

- core type

- shell type

- toroidal type

- distributed core type

- The given figure shows the digital multimeter display.

Which segments are to be connected to the power supply to show the number 5 at the display?

- ABCDEF

- ABCDG

- AFGGD

- ABDEG

- In an electronic circuit, transistor is used for switching ON and OFF a relay. When the transistor switches OFF the relay, a higher voltage appears across the transistor. How can a transistor be protected from this voltage?

- A capacitor in series to the relay

- A resistor in series to the relay

- An inductor parallel to the relay

- A diode parallel to the relay

- A resistor has a colour code band sequence of Brown, Black, Green and Golden then its value is:

- 1 kὩ ± 10%

- 10 kὩ ± 5% '

- 1000 kὩ ± 5%

- 1 MὩ ± 5%

- Which one of the following is a passive device?

- FET

- SCR

- Air core inductor

- Zener code

- Which one of the symbols given below represents a FET?

- What will be the value of VD in the given figure?

- 0 V

- 5 V

- 4.3 V

- 0.7 V

- Semiconductor diodes are called bipolar devices because :

- they contain both P and N type materials

- conduction across them is due to both ‘holes’ and ‘free electron’ V

- they work on both positive and negative supplies

- all the above stated reasons

- The time base of a CRO is adjusted to 20 ms/div. What is the frequency of the waveform shown in the figure.

- 25 Hz

- 10 Hz

- 50 Hz

- 25 kHz

- The main difference between a function generator and A.F. generator is this that a function generator can :

- produce sine wave but at. generator can produce square wave only

- produce sine wave, square wave, triangular waves but at generator can produce sine wave only

- produce triangular wave but at generator can produce sine wave only

- none of the stated above

- The function of a zener diode is to maintain a constant:

- input voltage

- input current

- output voltage

- output current irrespective of load resistance

- The load current flowing in the given circuit is :

- 40 mA

- 100 mA

- 60 mA

- 30 ‘mA

- The forward voltage for a general purpose LED ranges:

  - 1.0 V to 3.0 V

  - 1.1V to 3.0 V

  - 1.5 V to 3.0 V

  - 1.5 V to 3.5 V

- The side of a PCB on which the components are mounted is called :

- copper side

- track side

- solder side

- component side

- The advantage of transistor over thermionic valves is/are :

- very small size

- low operating voltage

- minimum power loss in the form of heat

- all the three mentioned above

- The purpose of mounting transistors on a metal plate is to :

- improve the conductivity of the transistor

- provide additional support to the transistor

- radiate the excessive heat of the transistor

- none of the above

- The gain of a CE amplifier. is highest at :

- mid frequencies

- low frequencies

- high frequencies

- all frequencies

- if negative feedback is used in an amplifier circuit, it results in :

- increased distortion

- increased gain

- reduced distortion

- no effect on distortion

- The gains of three stages of a cascade amplifier are 50, 10 and 5 db respectively. its overall gain will be :

- 500

- 5000

- 65

- 2500

- A RC. Oscillator uses :

- positive feedback

- negative feedback

- no feedback

- both positive and negative feedbacks

- The frequency of an oscillator circuit is calculated by the formula :

- f = 14π2.  f2.  C

- f = 12π.L. C

- f = 12π.L. C

- f = 1L. C

- The most common method of coupling used in a.f. amplifiers is :

- L. C. coupling

- direct coupling

- R. C. coupling

- transformer coupling

- How are SCRs normally turned to ON ?

- By applying a reverse break over voltage

- By applying a forward break over voltage

- By a separate commutation circuit

- By applying gate voltage

- A UJT has

- anode, cathode and gate

- two anodes and one gate

- two bases and one emitter

- one emitter and two gates

- A UJT is a

- current controlled device

- voltage controlled device

- a relaxation oscillator

- none of the above

- A UJT has

- stable negative resistance characteristics

- low firing current

- use as a wave form generator

- all of these characteristics

- Which semiconductor device acts like a

- SCR

- Triac

- UJT

- Diac

- For constructing a relaxation oscillator

- Zener diode is used

- UJT is used

- CRO is used

- none of the above

- A switched mode power supply operating at 20 kHz to 100 kHz range uses as the main switching element

- thyristor

- MOSFET

- triac

- UJT

- A triac is a……..

- unidirectional diode

- unidirectional thyristor

- a combination of two RN. diodes

- none of these

- A diac is equivalent to

- pair of SCRs

- triac with two gates .

- pair of four layer SCRS

- diode with two transistors

- The LED’s for their display require

- a voltage of 1.2V and a current of 2 mA

- a voltage of 25V and a current of 20 mA

- a voltage of 25V and a current of 100 mA

- a voltage of 1.2 V and a current of 100mA

- Thyristor is

- P-N-P-N device

- P-P-P-P device

- N-N-N-N device

- none of these

- A thyristor (SCR) is a

- current controlled device

- bilateral device

- voltage controlled device

- none of these

- Thyristor is nothing but a

- controlled switch

- controlled transistor

- amplifier with large current rating

- amplifier with higher gain

- A thyristor equivalent of a thyratron tube is V

- SCR

- UJT

- Diac

- Triac

- In thyristor, anode current is made of

- electrons

- holes

- electrons and holes

- positive ions

- Thyristors are not made using germanium because

- it is a stable semiconductor

- group V elements cannot be doped with it

- its current handling capacity is less  than silicon

- it has high leakage current

- In comparison with the turn-on time, the turn-off time of a thyristor is

- lesser

- equal

- larger

- uncomparable

- When a thyristor is in ON state, its gate drive be

- removed to save power

- removed or may not be removed

- not removed as it will turn off the thyristor

- removed to avoid increased losses and higher junction temperature.

- Turn off time of a thyristor

- depends upon junction temperature and forward current

- is a constant

- depends on load

- is not a constant

- During forward blocking state, thyristor is associated with

- low current and large voltage

- large current and low voltage

- low current and low voltage

- large current and large voltage

- Thyristor as a solid state relay is useful

- only for AC. control

- only for D.C. control

- both for AC. and D.C. control

- either D.C. or AC.

- Demodulation

- is just opposite to modulation

- removes side bands

- is accomplished at transmission end

- rectifies modulated signal

- Which semiconductor power device, but of the following, is not a current triggered device?

- Thyristor

- GTO

- Triac

- MOSFET

- An SCR is

- three layer three terminal device.

- three layer tour terminal device.

- four layer three terminal device.

- four layer four terminal device.

- An SCR conducts appreciable current, when

- anode and gate both are negative.

- anode is negative and gate is positive.

- anode and gate both are positive.

- anode is positive and gate is negative.

- ln an SCR. the anode current

- is independent of gate current.

- decreases with the increase in gate current.

- increases with the increase in gate current.

- none of the above.

- Which of the following does not cause permanent damage to an SCR?

- High current

- High rate of rise of current

- High temperature rise

- High rate of rise of voltage.

- After triggering an SCR, the gate pulse is removed, the current in the SCR will

- remain the same

- immediately fall to zero

- rise up

- rise a little and fall to zero

- Latching current of an SCH is .............. of rated current.

- below 10%

- 10% to 20%

- 20% to 25%

- 25% to 30%

- Suppose the anode current of a conducting SCR is 50 A. it its gate current is reduced to

15 th, its anode current will become

- 10 A

- 50 A

- 25 A

- zero

- In a class A amplifier, there will be a worst condition

- when signal input is zero

- when signal input is maximum

- when load resistance is high

- when there is a transformer coupling

- In a class-A amplifier, conduction extends over 360°, because Q-point is

- located on Ioad line

- located at or near cut-off point

- located near saturation point

- centered on load line

- The circuit efficiency of a class A amplifier can be increased with

- direct coupled load

- low D.C. power input

- transformer coupled load

- low rating resistor

- Transformer c0upiing is used in a class-A amplifier so as to make it

- more efficient

- less bulky

- less costly

- distortion tree

- The output of a class-B amplifier

- is distortion free

- comprises short duration current pulses.

- consists of positive half cycles only.

- is similar to that of a full wave rectifier

- A class-B push pull amplifier has the main advantage of being free from

- unwanted noise

- even order harmonic distortion

- amplitude distortion'

- circuit imbalance

- The maximum theoretical efficiency of a class-B push pull transistor amplifier is approximately

- 25%

- 50%

- 70.7%

- 78.5%

- The type of power amplifier which exhibits cross over distortion in its output is .

- class A

- class B

- class AB

- class C

- Cross over distortion occurs in ...................... amplifiers

- class A

- class B

- class C

- push pull

- Which' of the following classes of amplifiers has maximum distortion?

- class A

- class B

- class AB

- class C

- Negative feedback in amplifier increases

- its gain

- its stability

- its distortion

- its gain or distortion

- A transformer audio amplifier is found to have an overall efficiency of the order of 70%. It is

- class A push pull amplifier

- class 8 push pull amplifier

- single stage class C amplifier

- direct coupled amplifier

- Which one is not the advantage of push pull amplifier?

- More output.

- improved linearity

- All harmonics (ever or odd) removed.

- Balanced out effect of ripple in power supply.

- One of the effects of negative feedback in amplifiers is the

- increase in noise

- increase in harmonic distortion.

- decrease in bandwidth.

- decrease in harmonic distortion

- Heat sink is 'used in the power amplifier circuit

- to increase the output power.

- to reduce the heat loss in a transistor.

- to increase the voltage gain of the power amplifier.

- to increase the collector dissipation ratings of the transistor

- The output power of a power amplifier is several times its input power. This is possible due to the fact that

- step up transformer is used in the circuit.

- there is a positive feedback in the circuit.

- a negative resistance is introduced.

- power amplifier converts a part of the input D.C. power into A.C. power.

- Class A8 operation is often used in power (large signal) amplifiers in order to

- get maximum efficiency.

- remove even harmonics.

- overcome cross over distortion.

- reduce collector dissipation.

- Negative feedback in amplifiers

- increases the band width

- raises its upper cut-off frequency.

- lowers its lower cut-off frequency.

- all of above

- Which of the following statements is not true, regarding negative feedback in amplifiers?

- Gain stability is improved.

- Distortion is reduced.

- Frequency response is improved.

- Gain bandwidth product is increased.

- A tuned amplifier amplifies

- a wide band of frequencies.

- audio frequencies only.

- video frequencies only.

- a narrow band of frequencies

- For tuned radio frequency we use ................. amplifier.

- class A

- class B

- class AB

- class C

- Cascaded amplifiers are used on

- video amplifiers

- voltage amplifiers:

- power amplifiers.

- tuned amplifier design

- The feedback in emitter follower is

- 50%

- 100%

- 25%

- 75%

- The negative feedback in amplifier

- reduces the voltage gain.

- increases the voltage gain.

- increases the gain bandwidth products.

- reduces, the input impedance

- Negative feedback in amplifiers

- increases the band width.

- raises its upper cutoff frequency.

- lowers its lower cut-off frequency.

- all of above

- The voltage gain of an amplifier is 100. On applying negative feedback with ß = 0.04, its gain will reduce to

- 4

- 20

- 60

- 99.96

- An amplifier has gain without feedback as 10. To make it oscillate ß must be

- 0.2

- 0.1

- 0.5

- 0.8

- Feedback in an amplifier increases the input impedance from 1 kiloohm to 40 kiloohms because of

- positive feedback.

- shunt Current negative feedback.

- series current negative feedback.

- shunt voltage negative feedback.

- Feedback in amplifier always helps in

- controlling its output.

- increasing its gain.

- reducing its input impedance.

- stabilizes its gain.

- An emitter follower is mainly used as

- an impedance matching device.

- a power amplifier.

- a follower of base signal.

- none of the above

- Emitter follower is an example of................ feedback

- negative current

- negative voltage

- positive current

- positive voltage

- An emitter follower is widely used in electronic instruments as

- its voltage gain is very high.

- its voltage gain never exceeds unity.

- its output impedance is high and input impedance is low.

- its input impedance is high and output impedance is low

- Which one of the following parameters is used for distinguishing between a small signal and a large signal amplifier?

- Voltage gain

- Frequency response

- Harmonic distortion .

- input / output impedances

- Once the 808 starts conducting its gate current loses control over

- anode current

- anode voltage

- anode resistance

- all of the above

- With the increase in gate current of an SCR, the forward breakdown voltage will

- increase

- reduce

- remain unchanged

- none of these

- Which of the following will not decrease as a result of introduction of negative feedback?

- instability

- band width

- overall gain

- distortion

- in high frequency region, amplifier will behave like

- band pass filter

- high pass filter

- low pass filter

- none of the above

- A thyristor is a semiconductor switch which is

- unilateral and astable

- bilateral and astable

- unilateral and bistable

- bilateral and bistable

- Holding current of a thyristor is always

- more than its latching current

- less than its latching current

- equal to latching current

- none of the above

- The ratio of latching current to holding current is

- 1.5

- 1.0

- 1.5

- 2.5

- The device commonly used for triggering a triac is

- diode

- transistor

- zener diode

- diac

- Modulation is primarily accomplished to

- produce side bands.

- mix two waves of  different frequencies

- transmit audio frequency signals over long distances

- improve transmission efficiency

- What are the ON / OFF terminals of a transistor when it is operated as a switch?

- Collector to base

- Collector to emitter

- Base to collector

- Emitter to base

- What is the characteristic of an ideal voltage amplifier with respect to the input and output impedance?

- Low input and high output impedance

- High input and low output impedance

- Low input and low output impedance

- High input and high output impedance

- In battery powered digital multimeter, LCD display is most commonly used. The reason for this is......

- mains power is not available everywhere

- It requires only a small amount of power

- LCD can be read even in dark

- LCD has a bright display

- In a three stage amplifier the first stage gives a gain of 50, the second stage gives 10 and the third stage gives a gain of 5.  Which one of the following is the overall gain of the amplifier?

- 65

- 250

- 500

- 2500

- Optcouplers are used for electrical isolation between two circuits. An optocoupler is a signal package of.......

- LED and single diode

- LED and photodiode

- two photodiodes

- two LEDs

- The code 80177 on electronic means that it is a .........

- Ge. power transistor for switching applications

- Ge transistor for switching applications

- Si transistor for a.f. applications

- Si. transistor for i.f. applications

- Which one is NOT a passive component ?

- Transistor

- Inductor

- Capacitor

- Resistor

35. Digital Electronics

- The binary equivalent of decimal number 29 is………

- 1001 0001

- 0001 1101

- 0011 0001

- 0100 1101

- Which of the following lC contains NOT gates

- 7400

- 7402

- 7404

- 7408

- The digits used in binary number system are -

- 0,0

- 0, 1

- 1, 2

- 2,2

- Which of the following gates gives a high output with all its inputs high —

- NOT

- OR

- NOR

- AND

- The given figure shows electrical equivalent of……. gate.

- OR

- NOR

- NOT

- AND

- Which of the following logic gate gives high output with all its inputs low -

- OR

- NOR

- NAND

- AND

- if in a RS FF, R=S=0 then its mode of operation is -

- Set

- Reset

- No change

- Invalid

- The logic gate which has high output when its inputs are different (low high or, high-low) is

- Ex-OR

- Ex-NOR

- NAND

- NOR

- In JK FF, the condition J = K = 1 represents —

- Set

- Reset

- No change

- Complement of last state (toggle)

- a digital clock, constant frequency pulse is generated by -

- blocking oscillator

- astable multivibrator

- bistable multivibrator

- monostable multivibrator

36. Electrical Appliances

- The operating temperature range of an electrical stove with a pure element is :

- 300°C to 400°C

- 500°C to 600°C

- 750°C to 900°C

- 1100°C to 1300°C

- A 1000 W, 230 V kettle is connected to a 15 A plug-socket using a power cord rated to carry 15 A. What should be the fuse rating of the appliance?

- 4.0 A

- 5.0 A

- 15.0 A

- 16.0 A

- The heat proof insulating material used in heaters etc. is :

- mica

- asbestos

- glass wool

- porcelain

- In an automatic electric Iron a thermostat iz used to:

- reduce the current for controlling the temperature

- change the direction of current flow for controlling the temperature

- regulate the heat and thus the temperature by switching the element to ON-OFF

- switch ON-OFF the indicator lamp

- The type of motor used in ac. ceiling fans now produced in India is :

- repulsion motor

- permanent capacitor motor

- capacitor-start induction-motor

- shaded-pole motor

- A regulator is used in a ceiling fan circuit to:

- decrease the current

- drop the voltage applied to the fan

- increase the speed

- increase the torque

- The purpose of a capacitor in a fan is to :

- increase the speed

- protect-the fan in case a fault occurs

- produce a phase-shift

- control the speed

- An immersion heater is a non-repairable equipment because :

- it has no element ,

- its element is closed in a sand tilled spiral, narrow pipe

- it can’t be opened

- it gets burst on becoming defective

- In an electric iron, the function of pressure plate is:

- to cover the heating element

- to exert pressure on the garments while pressing them

- to obtain a close fitting of the heating element with the sole plate

- to act as a base of the equipment

- What is the function of a fusible safety plug in a geyser?

- it protects the geyser against blasting due to excessive steam pressure

- it protects the heating element against burning

- It releases the extra water out of the geyser

- It protects the thermostat of the geyser

- You have to test an electric heater which does not heat up You have detected that there is no defect in the electric supply and the switch

What further step will you take?

- To check all the components of the heater by dismantling the same

- To contact the supplier / manufacturer for faultfinding chart.

- To use a megger for checking the insulation resistance.

- To check the voltage on the terminal box of the heater

- You have to replace a fused lamp or a detective fan. What is the safest way in such case?

- To stand on a wooden stool or work bench.

- To use rubber blocks

- To switch-off S. P. switch

- To switch-off main supply.

- What should be the resistance between a water heater’s body and the connecting cable as per lSI?

- 0.01 mega ohms

- Not less than 0.5 mega ohms

- 1.0 kilo ohms

- 0.01 kilo ohms

- Why it is not possible to do the troubleshooting in electrical equipments having short-circuit, with the operating voltage?

- The high short-circuit current could destroy the measuring instrument

- The over current protection connected on the supply side will operate and disconnect the supply.

- Dangerous electric arcs may be produced because of high short- circuit voltage

- The high short-circuit current may overload the supply source.

- Sweep of ceiling fan refers to :

- radius of circle formed by its rotation

- double the length of the blade

- the distance from centre of motor to tip of the blade

- double the distance from centre of motor to tip of the blade

- if a table fan does not respond to the operation of the speed selector switch, the problem may be that the :

- capacitor is burnt out

- regulator is open-circuit

- regulator is short-circuit

- oscillating mechanism is defective

- if a table fan running smoothly starts giving up smoke, the reason is :

- short-circuited winding

- failure of the centrifugal switch to open the starting winding circuit

- bearing trouble

- overload

- The type of motor used in A.C. table fans now produced in India is :

- repulsion motor

- shaded pole motor

- capacitor run induction motor

- universal motor

- A ceiling tan is rotating at low speed and becomes hot. Pick up the item which is, not a cause :

- capacitor burnt out,

- connections of supply lead interchanged

- winding earthed or partially short- circuited

- worn out bearings and a bent blade

- What should be the resistance of a new thermostat?

- Zero or few ohms

- In kilo ohms

- Infinity

- In mega ohms

- A heater with name plate markings of 250 V 1000 W, heater element 500°C is connected to 230 V supply. What will happen to the heater?

- The output heat remains the same

- The glow of the heater element will be dim

- The glow will be brighter because of more current

- The efficiency will increase

- As a precaution the switch which controls the geyser should be provided :

- outside the bathroom

- inside the bathroom

- near the main switch

- opposite wall of the geyser

- What is the range of operating speed of mixers?

- 1400-2000 r.p.m.

- 1400-2600 r.p.m

- 1400-3000 r.p.m.

- 3000-12000 r.p.m

- The, electric motor generally used in house hold food mixers is...........

- universal motor

- shaded pole motor

- capacitor start motor

- none of the above

- Which appliance works on heating effect of electric current?

- Washing machine

- Food mixer

- Toaster

- Table fan

- When a housewife accidently touched the metal part of the handle of a kettle, which was kept ‘ON’ she received shock. The reason is.........

- the fuse wire in the circuit is of higher capacity

- the kettle is not earthed

- there is no water in the kettle

- the Safety, plug has become defective

- As a precautionary measure the heating element used in heaters, electric iron and water heaters should not attain a temperature more than......

- 335°C

- 635°C

- 735°C

- 1035°C

- Operating temperature range of heating elements in hot plate is..........

- 550°C to 700°C

- 700°C to 850°C

- 750°C to 900° C

- 800°C to 850°C

- What should be the insulation resistance between the body of the electrical water heater and the connecting cable as per l.S.l.?

- 0.01 mega ohm

- 1.00 mega ohm

- 1.00 kilo ohm

- 0.01 kilo ohm

- You have to repair storage type water heater which has steam in the hot water. The possible remedy for this trouble can be found by checking.........

- all points for possible leakage

- the thermostat setting

- all wirings for loose connection

- elements for its condition

- Every appliance should be tested for insulation resistance before commissioning. For a water heater the insulation resistance should not be less than.........

- 1 mega ohm

- 2 mega ohm

- 0.5 mega ohm

- 0.25 mega ohm

- The insulating material used in connecting leads of the electric heater is……….

- Bakelite material

- Porcelain material

- Rubber material

- Asbestos material

- Calculate the current and the resistance of the heating element for an Electric heater of 250 V, 1500 W, working at 500°C.

- 4 amps and 62.5 ohms

- 5 amps and 55.5 ohms

- 6 amps and 41.66 ohms

- 7.5 amps and 31.25 ohms

- Which among the following is the operating temperature range of an electrical stove with a bare heating element?

- 200°C to 300°C

- 500°C to 600°C

- 750°C to 900°C

- 1000°C to 1100°C

- Which among the following is the temperature regulating component of an automatic electric iron?

- Heating element

- Sole-plate

- Thermostat

- Pressure plate

- Which one of the following is the required size of Nichrome wire for a heating element of 1000watts, 230 V heater at 500° C?

- 18 SWG

- 19 SWG

- 20 SWG

- 24 SWG

- What is the material used in between the outer casing and the inner tank of a geyser?

- Air

- Mica

- Glass wool

- Asbestos

- Why is it necessary to fill the space between the outer casing and the inner tank of water heater with glass wool?

- To prevent damage of inner tank during transit

- Corrosion protection for the inner tank

- To prevent heat loss of the hot water

- The protect the heating element, against mechanical damage

- The space between the outer casing and the inner tank of storage water heater is fitted with the heat insulation materials is……….

- asbestos

- glass

- mica

- rubber

- Glass wool is provided in between the outer casing and inner container in storage water heater is to..........

- reduce the water leakage

- reduce the heat loss

- increase the life of water heater

- limit the load current

- What should be the value of resistance  between the contacts of a new thermostat at the lowest temperature setting of the control knob in heater ?

- In kilo ohms

- Zero ohm or few ohms

- infinity

- High resistance in mega ohms

- You have to repair a geyser, which does not provide hot water. What can be the possible cause?

- Thermostat contacts are welded together.

- Thermostat does not disconnect the heater from the supply.

- Open in the thermostat contacts.

- Thermostat set too high or out of calibration.

- In the geyser, the following fault is occurred i.e. the fuse is blowing repeatedly. What will be the main reason for the above fault?

- Leakage around the thermostat

- Thermostat setting is too low

- Leakage around heating unit flange

- Grounded heating element

- There is a fault in the geyser ie. it does not provide hot water when it is switched “ON". What may be the cause for this fault?

- Thermostat contacts are in open condition

- Thermostat contacts are welded together

- Thermostat is set in too high range

- Thermostat does not disconnect the heater from the supply.

- You have to repair a hot water geyser. The geyser does not provide hot water. The defect can be found by checking………

- inlet valve for its full open condition.

- correct water level in the tank.

- wiring connection for open circuit and loose connections.

- thermostat, for its welded contacts.

- You have to repair a hot water geyser with a leaking tank. What can be the possible cause?

- Leaking around the thermostat.

- Inlet fully open.

- Corroded outer cover.

- Excess inflow of water.

- In Electric iron, Blister on the sole plate fault is occurred during its operation, What is the cause for this fault?

- Defective heater element

- Excessive heat

- Low line voltage

- Dry sole plate

- What is the main cause for occurring Blisters on the sole plate in automatic electric iron?

- Detective thermostat

- Excessive heat

- Defective heater element

- Loose connection

- Electric irons are available in standard power rating

- 1000 Watts

- 750 Watts

- 250 Watts

- 100 Watts

- The function of the thermostat in an Automatic electric iron is to..........

- reduce the current to control temperature

- limit the current flow through the element

- regulate the heat by switching ON and OFF

- control the ON and OFF switch function of indicating lamp

- The purpose of the fusible plug fitted on the top of the water heater casing is to………

- disconnect the supply of electricity in case of short circuit

- release the pressure inside the tank in case or thermostat failure

- release the steam inside the heater in case of over heat

- eject the three pin plug so as to disconnect the supply in case of earth fault

- What is the kW rating of geyser, for 50 litres capacity.

- 1 kW

- 2 kW

- 3 kW

- 4 kW

- What is the purpose of the pressure plate in the Kettle?

- it supports the heating element in one position

- it reduces the heat loss

- it holds the sole plate in position

- it keeps the heat uniformly

- In which type of  washing machine, the drum itself rotates to wash the loaded clothes?

- Pulsator (disc) type

- Agitator type

- Tumbler type

- Turbulence jet type

- Which type washing machine contains two tubs?

- Ordinary without timer type

- Ordinary with timer type

- Semi-automatic type

- Automatic type

- A ceiling fan uses..........

- capacitor motor

- capacitor start capacitor run motor

- capacitor start motor

- universal motor

- Ceiling tan is..........

- three phase induction motor

- single phase induction motor

- single phase synchronous motor

- none of these

- When a ceiling fan using a capacitor run motor is switched “ON”, it hums but does not run when it is driven by some external. means, in the direction in which it was made to run,

- open circuited capacitor

- flown fuses

- short circuited capacitor

- any of the above

- The fan regulator Is provided With ceiling fan circuit is to..........

- limit the current

- increase the torque

- drop the voltage applied to the fan

- increase the life of fan

- Which type of motor is used in ceiling fan?

- shaded pole motor

- universal motor

- permanent capacitor start motor

- capacitor start of run motor

- The capacitor used in fan motor is connected in.........

- parallel with winding

- parallel with starting running winding

- series with starting winding

- series with starting and running winding

- Which among the following is the reason for a fan running hot and slowly?

- Rotor winding open

- Field winding open

- Capacitor open

- Regulator short

- Which one of the following is the reason for a fan rotating slowly in opposite direction?

- Capacitor open

- Capacitor short

- Field winding open

- Rotor winding open

- If the capacitor in a fan is shorted, then the fan will.........

- not run

- run slowly

- run fast

- run with noise

- The blade of the ceiling fan is fitted to the..........

- Top cover of the fan

- Body of the fan

- Shaft of the tan

- Canopy of the fan

- The sweep of the ceiling fan Is determined by..........

- length of the blade from fan centre

- the distance between adjacent blade tips

- the diameter of the circle formed by the blade tips of the tan

- the breadth and length of the blade

- The batch of ceiling fans are re-fitted after overhauling. Out of this two fans are found to be wobbling. Which one of the following is the cause of wobbling?

- The rotor of the fan is changed

- The bearing of the fan is changed

- The rotor position is changed

- The blades of the fan is changed

- The table fan is not oscillating, the cause may be..........

- the blades are out of balance

- the spur gear with broken teeth

- the defective speed

- the loose guards control switch

- Pumps can be classified mainly into a .......... categories.

- 2

- 3

- 4

- 5

- Reciprocating pumps is also known as :

- jet pump

- piston pump

- centrifugal pump

- submersible pump

- A submersible pump can lift water from any depth as the:

- length of the unit is more

- efficiency is more

- suction pipe is not there

- diameter of the unit is smaller

- When a pump is newly installed or when it has been dismantled for any repair etc, for proper functioning two important conditions must be fulfilled. The direction of rotation and priming before starting. Which pump does not need priming?

- Reciprocating pump

- Jet pump

- Submersible pump

- Centrifugal pump

- A pump motor has an incorrect direction of rotation because of wrong connections. How do you notice this fault?

- The pump supplies too much water

- The pump does not supply water

- The pump motor runs too hot

- The fuse in the motor circuit blows out

- You are repairing a pump that lifts too less water than earlier. What could be probable reason for this fault?

- Level of water in sumps is too high

- Impeller damaged

- Pump is running in reverse direction

- Motor running fast due to higher frequency

- A centrifugal pump of efficiency 80% has to deliver 5000 litres of water in 30 minutes to a overhead tank on the top of a house 8 m from ground level where the pump is installed. The pump is driven by a single phase 240 V 50 Hz capacitor start motor. The horse power of the motor required is...........

- 0.32 HP

- 0.37 HP

- 0.42 HP

- 0.45 HP

37.  Relays and Voltage Regulators

- What will happen when 12 V, 50 Hz A.C. relay is connected to a 12 V, D.C. supply?

- The switching speed will be increased because of more current drawn by relay

- The switching capacity of the relay will be decreased very strongly

- The relay draws more current in the absence of inductive reactance which bruns out the coil

- D.C. operation causes the core and armature to become partly permanent magnet and fails to function after a few operations

- Starter contacts are indicated either for their position or for their function. in such a case, normally closed contact means that they are………

- in closed condition in all the status of relay

- open when the relay coil is de- energized

- open when the relay coil is energised

- closed when the relay coil is energised

- The relay coil is connected to an A.C. voltage of 50 Hz it the frequency of the applied voltage raises to 60 Hz. Which one of the following four changes will occur?

- The active resistance R increases

- The reactance XL decreases

- The reactance XL increases

- The quality of the coil decreases

- Which type of relay is used to protect the damage of equipment during under and over voltage fault?

- current sensing relay

- under current relay

- voltage sensing relay

- latching relay

38. Generation of Electrical Energy

- A hydroelectric power station is commonly found in

- desert areas

- hilly areas

- swamps

- grasslands

- The largest size hydroelectric unit in India Is

- 350 MW

- 500 MW

- 165 MW

- 90 MW

- In hydro power stations what is an enlarged body of water just above the intake and used as a regulating reservoir, called?

- Spillways

- Forebay

- Reservoir

- Penstock

- Location of a surge tank, in an hydroelectric power station, is near

- turbine

- tail race

- reservoir

- dam

- in high head hydroelectric power plant  the velocity of water flow in penstock is around

- 2 metres per second

- 4 metres per second

- 7 metres per second

- 10 metres per second

- For variable heads of near about, but less than 30 meters, which type of turbines is used in hydro power stations?

- Pelton

- Kaplan

- Francis

- None of the above

- For high head and low discharge, the water turbine used is

- Pelton wheel

- Kaplan turbine

- Francis turbine

- Propeller turbine

- Turbines installed at Bhakra Nangal are

- Pelton wheels

- Francis turbines

- Kaplan turbines

- Propeller turbines

- An impulse turbine

- is most suited for low head and high discharge power plants

- operates by initial complete conversion to kinetic energy

- makes use of a draft tube,

- always operates submerged

- The power output from an hydroelectric power plant depends on

- type of dam, type of catchment area and discharge

- type of dam, head and system efficiency

- discharge, head and system efficiency

- type of turbine, type of dam and type of catchment area

- The advantages of hydroplants are:

- low operating cost

- they can be started and loaded very quickly

- they can be used as base load and peak load plants as well

- all of the above

- In hydroelectric power plants

- operating cost is low and initial cost is high

- operating cost is high and initial cost is low

- both operating and initial costs are high

- both operating and initial costs are low

- Gross head of an hydroelectric power station is

- the difference of water level between the level in the storage and tail race

- the height of water level in the river where the tail race is provided

- the height of water level in the river where the storage is provided

- all of the above

- A penstock is used as a conduit between

- the steam chest and the turbine in a thermal station

- the dam and the turbine in a hydrostation

- the turbine and the discharge drain

- the heat exchanger and the turbine in a nuclear power plant

- the function of a surge tank is to

- supply water at constant pressure

- relieve water hammer pressure in the penstock pipe

- produce surge in the pipeline

- none of the above

- Turbo-alternators run at

- a variable speed around 2000 R.P.M.

- a constant speed of 1000 R.P.M.

- a constant speed of 1000 R.P.M.

- a variable speed above 1000 R.P.M.

- the first nuclear power plant installed in India is

- Kota (Rajasthan)

- Tarapur (Maharashtra)

- Kalpakkam (TN)

- Narora (U.P)

- One atomic mass unit is equivalent to

- 1.66 x 10-24 kg

- 1.66 x 10-27 kg

- 1.66 x 10-19 kg

- 1.66 x 10-13 kg

- One kilogram of natural Uranium gives energy equivalent to about

- 100 kg of coal

- 1000 kg of coal

- 5000 kg of coal

- 10000 kg of coal

- One atomic mass unit (amu) is equivalent to

- 931.4 MeV

- 251.2 MeV

- 120.4 MeV

- 315.5 MeV

- Particles having the same atomic numbers but different mass numbers are called the

- beta particles

- decayed particles

- isotopes

- neutrons

- Nuclear reactors usually employ

- ﬁssion

- fusion

- both fission and fusion

- none of the above

- One fission of O235 releases energy to

- 335 MeV

- 200 MeV

- 430 MeV

- 525 MeV

- Graphite is used in nuclear power plant as a

- fuel

- coolant

- moderator

- electrode

- Control rods used in nuclear reactors are made of

- zirconium

- boron

- beryllium

- lead

- Which material is used in controlling chain reaction in a. nuclear reactor?

- Thorium

- Heavy water

- Boron

- Beryllium

- in a nuclear reactor, chain reaction is controlled by introducing

- iron rods

- cadmium rods,

- graphite rods

- brass rods

- Reflectors of a nuclear reactor are made of

- cast iron

- beryllium

- steel

- boron

- Which one of the following fuels is used by the slow thermal nuclear reactors for power generation?

- U235

- U238

- Th232

- Pu239

- In a nuclear reactor thermal energy is obtained from

- fission of radioactive materials

- fusion of radioactive materials

- burning of the fuel rods in oxygen

- none of these

- The function of moderator in a nuclear reactor is

- to absorb the excess neutrons

- to increases the energy of the neutrons

- to slow down the neutrons

- none of these

- Tarapur atomic power station has

- CANDU type reactors

- boiling water reactors

- pressurised water reactors

- gas cooled reactors

- A CANDU (Canadian Deuterium Uranium) reactor uses

- highly enriched uranium as fuel and light water as moderator and coolant

- natural uranium as fuel and heavy water as moderator and coolant

- enriched uranium as fuel and ordinary water as moderator and cooler

- only fertile material

- The power output of a nuclear power station is proportional to

- the rate at which fission reaction occurs

- square root of the rate at which fission reaction occurs

- square of the rate at which fission occurs

- none of the above

- Fast breed reactors are best suited for India owing to

- large thorium deposits

- large plutonium deposits

- large uranium deposits

- mine of the above

- The function of reflector in a nuclear reactor is to

- bounce back most of the neutrons that escape from the fuel core

- reduce the speed of the neutrons

- stop the chain reaction

- all of the above

- Thermal shielding is provided to

- absorb the fast neutrons

- protect the operating personnel from exposure to radiations

- prevent the reactor wall from getting heated

- all of the above

- The main sources of production of biogas are:

- wet cow dung

- human waste

- wet livestock waste

- all of the above

- Biogas consists

- only methane

- methane and carbon dioxide with some impurities

- only ethane

- a special organic gas

- Biogas plants are suitable for

- metallurgical industries

- commercial complexes

- rural areas

- coal mines

- The main by-product of the biogas plant is

- biomass

- biogas

- organic manure

- none of the above

- The disadvantage of renewable sources of energy is

- intermittency

- lack of dependability

- availability in low energy densities

- all of the above

- Wind as a source of power

- in non-steady and unreliable

- does not possess the basic requirements of any energy source

- is plentiful, inexhaustible, renewable and non-polluting

- all of the above

- Maximum wind energy available is proportional to….

- air density

- cube of the wind velocity

- square of the rotor diameter

- all of the above

- A tidal power plant is installed in India near

- Bay of Bengal

- Visakhapatnam

- Goa

- Gulf of Cambay

- The turbine normally employed in tidal power plant is

- simple impulse type

- propeller type

- reaction type

- reversible type

- Tidal energy utilizes

- kinetic energy of water

- potential energy of water

- both potential as well as kinetic energy of water

- none of the above

- Tidal power schemes, could not be found economically justified because of V‘

- high cost of civil engineering works

- non-availability of tidal energy in India

- both A and B

- none of the above

- A geothermal field may yield

- dry steam

- wet steam

- hot water

- all of the above

- Geothermal energy is

- inexhaustible energy source

- a renewable energy source

- alternative energy source

- all of the above

- In comparison to conventional steam power plants, geothermal power plants have

- lower load factor

- the same load factor

- higher load factor

- unpredictable

- In geothermal power plants, waste water is

- discharged back to earth

- discharged into sea

- recirculated after cooling in cooling towers.

- evaporated in ponds

- In a fuel cell, electrical energy is produced by

- reaction of hydrogen with oxygen

- thermionic action

- combustion of fuel in absence of oxygen

- any of the above

- In a fuel cell, cathode is of

- oxygen

- ammonia

- hydrogen

- carbon monoxide

- The energy radiated by Sun on a bright sunny day is  about

- 2.5 kW/m2

- 1.0 kW/m2

- 500 W/m2

- 200 W/m2

- in solar power plants, the solar heat is transferred to

- molten salts

- liquid metals

- water steam

- any of the above

- Which of the following area is preffered for solar plants?

- Coastal areas

- Hot arid zones

- Mountain taps

- High rainfall zones

- The function of a solar collector is of converting solar energy into

- radiations

- electrical energy directly

- thermal energy

- any of the above

- Reflector mirrors employed for exploiting solar energy are called the

- mantle

- heliostats

- diffusers

- ponds

- A pyrometer can be used for measurement 0f

- diffuse radiations only,

- direct radiation only

- both direct and diffuse radiations

- none of  the above

- Temperature attained by cylindrical parabolic collector is of the order of

- 50-100°C

- 100-150°C

- 150-200°C

- 200-300°C and above

- The flat plate collector gives a

- temperature of about 90°C with an efficiency of 30-35%,

- temperature of about 120°C with an efficiency of 45%

- temperature of about 150°C with an efficiency of 52%

- none of the above

- in a solar collector, the transparent cover is provided to

- protect the collector from dust

- reduce the heat losses from collector beneath to atmosphere

- transmit solar radiation only

- all of the above

- Solar thermal power generation can be had by using

- flat plate collectors

- focusing or concentrating collectors

- solar ponds

- any of the above

- Main applications of solar energy may be considered as

- direct thermal applications

- fuel from biomass

- solar electric applications

- all of the above

- Sun tracking is required in case of

- cylindrical parabolic and paraboloid

- flate plate collector

- both A and B

- none of the above

- The efficiency of solar cell is about

- 25%

- 15%

- 40%

- 60%

- Photovoltaic solar energy conversion system makes use of

- solar pond

- fuel cell

- Edison cell

- none of the above

- Solar cells are made of

- aluminium

- germanium

- silicon

- cadmium

- The output of a solar cell is of the order of

- 0.5-1 V

- 1-2V

- 2-3 V

- 4-5 V

- For satellites the source

- solar cell of energy

- fuel cell

- Edison cell

- cryogenic storage

- the order of solar cell is of

- 0.5 W

- 1.0 W

- 5.0 W

- 10.0 W

- A module is a

- series arrangement of solar cells

- parallel arrangement of solar cells

- series-parallel arrangement of solar cells

- none of the above

- Solar cells, for power generation have drawbacks of

- low efficiency

- lack of availability

- high cost and maintenance problems

- all of the above

- The solar or photovoltaic cell converts

- chemical energy into electrical energy

- solar radiations into electrical energy

- solar radiations into thermal energy

- thermal energy into electrical energy

- T he nature of the current developed in MHD generator is

- A.C.

- D.C.

- AC. or D.C.

- none of these

- in an MHD generator, the conductor is made of

- copper or aluminium

- liquid metal

- gas

- liquid metal or gas

- Which of the following liquid metal is not used as an MHD working fluid?

- Potassium

- Sodium

- Lithium

- all of the above

- Seeding material commonly used in MHD generator is

- Sodium

- aluminium

- potassium

- cesium

- The concept of MHD generation depends much more on the..... of the gas in the duct

- Permeability

- permittivity

- conductivity

- none of these

- The advantages of an MHD system are

- low pollution and good efficiency

- conservation of fuel

- absence of moving parts

- all of the above

- Which of the following power plants is the least reliable?

- Wind

- Tidal

- Geothermal

- Solar

- What is the approximate total power installed capacity in India?

- 12000 MW

- 175000 MW

- 200000 MW

- 50000 MW

- Which of the following power plants can generate power at unpredictable or uncontrollable time?

- Tidal power plant

- Wind power plant

- Solar power plant

- Any of the above

- The power station that does not require any moving part is

- Thermionic converter

- Wind power generation

- Tidal power plant

- Solar power generation

- Which of the following power plants is free from environmental problems?

- Diesel engine

- Nuclear

- Hydroelectric

- Steam

- Which of the following plants will take least time in starting from cold conditions to full load operation?

- Nuclear power plant

- Steam power plant

- Hydroelectric plant

- Gas turbine plant

- Which of the following plants has the maximum capital cost?

- Steam plants

- Hydro plants

- Diesel plants

- Nuclear plants

- Which of the following generating plants has the minimum operating cost?

- Nuclear plant

- Hydroelectric plant

- Steam plant

- Diesel plant

- Direct conversion of heat energy into electrical energy is possible through

- MHD generators

- Thermionic converters

- Fuel or Solar cell

- both A and B

- Which of the following methods, generating electric power from the sea water is more advantageous?

- Ocean currents

- Wave power

- Tidal power

- None of the above

- The power development programmes that are possible in Tamil Nadu are largely based on

- steam power plants

- hydroelectric power plants

- nuclear power plants

- diesel power plants

- A generating station which has a high investment cost and low operating cost is usually operated as a

- peak load station

- base load station

- medium load station

- none of the above

- The commercial sources of energy are

- fossile fuels, water and radioactive substances

- solar, wind, biomass

- wood, animal wastes and agricultural wastes

- none of the above

- Transmission and Distribution

- Underground cable is protected by :

- M.S. and G.l. pipe

- granite stone

- a layer of sand and bricks

- a layer of stone slabs.

- A 11 kV underground cable is layed at a depth of :

- 0.1 m

- 0.5 m

- 0.9 m

- 1.5 m.

- Epoxy compound used with underground cable does-not need :

- heating

- cooling

- stirring

- mixing

- A cable used to supply ‘power’ and ‘light and fan’ electric supply should be of :

- two core type

- three core type

- 3 ½ core type

- four core type

- The purpose 0t providing armouring to an underground cable is to prevent :

- white-ant attack

- entry of moisture

- mechanical injury

- bursting

- If the underground cable has a lead tube as metallic sheath, the cable joint made once does not need :

- testing with megger

- separate earthing

- metallic junction box

- any plumbing work

- The depth of a pole below the ground level in case of normal soil should be :

- 1/4 th of its full height

- 1/5 th of its full height

- 1/6 th of its full height

- 1/8 th of its full height

- While Using wooden poles in a service line, the span should not exceed :

- 100 to 300 m

- 60 to 100 m

- 50 to 80 m

- 40 to 50 m

- While connecting a copper wire with an aluminium wire, use :

- straight sleeve and nut connector

- bimetallic universal P.G. clamp

- standard P.G. clamp

- Compression connector

- In a service line of a residential colony which type of poles are preferred ?

- Wooden

- Steel tubular

- R.C.C.

- Steel tower

- Which type of insulator is used in stay wire?

- Pin type

- Egg type

- Shackle type

- Suspension type

- if a distribution line and street lighting fixtures are installed on the same pole then their span should not exceed :

- 30 m

- 45 m

- 60 m

- 100 m

- As per l. E. rules, no service line should be tapped :

- at the mid span

- at the beginning

- at the termination

- near to an insulator

- As per I. E. rules, no conductor of overhead line including service lines be erected across a street shall at any part thereof be, at a height less than .................. metres for low and medium voltage lines.

- 4 (13 feet)

- 4.5 (14.7 feet)

- 5 (16.4 feet)

- 5.486 (18 feet)

- As per l.E. regulation, when service lines are insulated elsewhere other than along or across any street with insulated conductor, the conductor of the line at any part small have a height not less than a specified value.

What is this specified height up to 11000 V line?

- 3.963 m

- 4.572 m

- 5.486. m

- 5.791 m

- In industries, number of machines are installed in rows. In such places‘ overhead bus system is used for distribution of electrical supply to each machine. The recommended maximum current density for aluminium bus bar is ......

- 118 Amps/cm2

- 156 Amps/cm2

- 165 Amps/cm2

- 181 Amps/cm2

- Service connection to consumer premises is provided either through overhead or through underground cable by tapping from OH line.

Service connection line should be tapped from OH line at ............

- mid span only

- any point along the span

- a point of support

- a point one meter away from support

- What is the type of insulator used for holding the H.T. overhead conductor on  straight running of poles?

- Post insulators

- Pin type insulators

- Stay insulators

- Shackle insulators

- XLPE cable is one type of H.T. cable. The full form of XLPE is………

- Cross lead paper ethylene cable

- Cross line poly ethylene cable

- Cross linked poly ethylene cable

- Cross linked paper ethylene cable

40. Corona, Lightning and Skin Effect

- Corona is accompanied by:

- Violet visible discharge in darkness

- Hissing sound and power loss

- Vibration and Radio interference

- All of the above

- Which of-the following statements is true regarding corona

- corona takes place at a voltage lower than breakdown voltage

- corona takes place at a voltage higher than breakdown voltage

- corona is a current phenomenon

- corona increases the transmission line efficiency

- The current drawn by the line owing to corona loss is

- Non sinusoidal

- Sinusoidal

- D.C.

- Square

- To reduce corona effect, usually

- the distance between the conductors is reduced

- the conductor diameter is reduced

- bundled conductors are used

- stranded conductors are used

- Corona loss can be reduced by the use of hollow conductors because

- the current density is reduced

- the eddy current in the conductor is eliminated

- for a given cross section, the radius of the conductor is increased

- of better ventilation in the conductor

- Corona loss in a transmission line is dependent on

- diameter of the conductor

- material of the conductor

- height of the conductor

- none of above

- In context of coma, with smooth and polished conductors

- there will be no corona glow

- corona glow will be uniform along the length of the conductor

- there will be minimum power loss

- hissing sound will be more intense

- In context of corona, which statement is not true?

- corona is voltage effect

- corona takes place on short transmission lines

- corona is accompanied with power loss

- corona attenuates lightning surges

- Corona occurs between two transmission conductors when

- they have high potential difference

- they are closely spaced

- they carry DC power

- both A and B

- Corona can be reduced by,

- increasing the operating voltage

- reducing the spacing between conductors

- increasing the effective conductor diameter

- any of the above

- Which of the following affects corona the least?

- Mean free length

- Atmospheric temperature

- Number of ions

- Size and charge per ion

- The effect of corona is

- increased inductance

- increased reactance

- increased power loss

- all of the above

- The chances of occurrence of corona are maximum during

- humid weather

- dry weather

- winter

- hot summer

- In humid weather, the corona occurs at a voltage

- much less than that needed in fair weather

- much higher than that needed in fair weather

- equal to that needed in fair weather

- none of the above

- The only advantage of corona is that it

- produces a pleasing luminous glow

- makes line current sinusoidal

- works as a safety valve for surges

- ozone gas is produced

- Corona is likely to occur maximum in case of

- distribution lines

- transmission lines

- domestic wiring

- service mains

- Presence of ozone owing to corona

- improves the power factor

- reduces the power factor

- corrodes the material

- improves regulation

- Corona is affected by

- size of conductor

- shape and surface condition of the conductor

- operating voltage

- all of the above

- Corona usually occurs when the electrostatic stress in the air around the conductor exceeds maximum of

- 40 W per cm

- 30 W per cm

- 22 W per cm

- 6.6 W per cm

- Power. loss due to corona is directly proportional to

- spacing between conductors

- radius of conductor

- supply frequency

- none of the above

- Corona has the disadvantage of

- interference with neighbouring communication circuits

- introducing the harmonics, predominately third harmonics, into the transmission lines

- power loss

- all of the above

- The critical valve of surge impedance of a long transmission line is

- 1000 Ὡ

- 400 Ὡ

- 250 Ὡ

- 50 Ὡ

- The serge-impedance of cables is around

- 20 Ὡ

- 50 Ὡ

- 100 Ὡ

- 250 Ὡ

- A lightning arrester connected between the line and earth in a power system

- protects the terminal equipment against travelling surges

- protects the terminal equipment against direct lightning stroke

- suppresses high frequency oscillations in the line

- reflex back the travelling waves approaching it

- A thyrite type lightning arrester

- blocks the surge voltage appearing in a line

- absorbs the surge voltage appearing in a line

- offers a low resistance path to the surge

- returns the surge back to the source

- Skin effect in transmission line is due to

- supply frequency

- self inductance of conductor

- high sensitivity of material in the centre

- both A and B

- Skin effect depends upon

- cross-section of conductor

- supply frequency

- permeability of conductor material

- all of the above

- The conductor carries more current on the surface in comparison to its core. This phenomenon is called the

- Skin effect

- Corona

- Ferranti effect

- Lenz’s effect

- Skin effect exists in

- cable carrying; DC. current

- D.C transmission line only

- A.C. transmission line only

- D.C. as well as A.C. transmission lines

- Skin effect of a conductor reduces with the increase in

- supply frequency

- resistivity of the conductor material

- cross section of conductor

- permeability of the conductor material

- To calculate the diversity factor is.....

- Diversity factor = Minimum Actual loadInstalled load

- Diversity factor = Installed loadMinimum Actual load

- Diversity factor = Installed load + Minimum Actual load

- Diversity factor = Installed load x  Minimum Actual load